3GPP TSG-RAN WG2 Meeting #121                                    	R2-2301986

# Athens, Greece: 27th February – 3rd March, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1561 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on figure clarifying HARQ RTT timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently HARQ RTT timer is calculated taking into account UE-eNB RTT time. However, the way HARQ RTT timer length and deltaPDCCH are described is not very clear. The following definition adds confusion whether the RTToffset is added twice in the HARQ RTT timer, one together with 3 (i.e., 3+RTToffset) and the other for deltaPDCCH definition. For NB-IoT, when single TB is scheduled by PDCCH or when multiple TBs are scheduled for the interleaved case when HARQ-ACK bundling is configured the HARQ RTT Timer is set to k+3+N subframes plus RTToffset + deltaPDCCH, where k is the interval between the last subframe of the downlink transmission and the first subframe of the associated HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback, and deltaPDCCH is the interval starting from the subframe following the last subframe of the associated HARQ feedback transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion. To avoid this confusion, illustrative figure was added in Annex C, however, it is missing RTToffset adding the same confusion of whether RTToffset is added twice. To avoid any possible confusion in future, it is better to clarify the figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The figure in Annex is updated to take into account the RTToffset. Impact analysis: Impacted functionality: HARQ RTT timer calculation Inter-operability: No inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be confusion whether the RTToffset is added twice in the calculation of HARQ RTT timer length. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

Annex C (informative):
Intended UE behaviour for DRX Timers

When a DRX timer is set to a value of X, and n denotes the subframe in which the related event is triggered according to the clause 5.7, the intended behaviours of each DRX timer are presented in the Table C-1 below:

Table C-1: Intended UE behaviour for DRX timers

| DRX Timers | Intended UE behaviour ([x, y] means including subframe x and y) |
| --- | --- |
| drx-InactivityTimer | The MAC entity monitors PDCCH in PDCCH-subframes during the subframes [n+1, n+m]. The MAC entity starts or restarts drxShortCycleTimer, and uses Short DRX Cycle in the subframe n+m+1, if configured. |
| drx-InactivityTimerSCPTM | The MAC entity monitors PDCCH in PDCCH-subframes during the subframes [n+1, n+m]. |
| mac-ContentionResolutionTimer or mac-ContentionResolutionTimer for the corresponding enhanced coverage level, if it exists | The MAC entity monitors PDCCH in PDCCH-subframes during the subframes [n+1, n+X]. |
| drx-RetransmissionTimer or drx-ULRetransmissionTimer | The MAC entity monitors PDCCH in PDCCH-subframes during the subframes [n, n+m-1]. |
| onDurationTimer or onDurationTimerSCPTM | The MAC entity monitors PDCCH in PDCCH-subframes during the subframes [n, n+m-1]. |
| drxShortCycleTimer | The MAC entity uses the Short DRX Cycle during the subframes [n, n+X-1]. The MAC entity starts to use the Long DRX Cycle in the subframe n+X. |
| HARQ RTT Timer | The MAC entity starts drx-RetransmissionTimer in the subframe n+X, if needed. |
| UL HARQ RTT Timer | The MAC entity starts drx-ULRetransmissionTimer in the subframe n+X, if needed. |
| NOTE 1: For FDD, m is equal to X; for TDD, m is equal to the minimum number of subframes so that X PDCCH-subframes are included during the subframes [x, y]. NOTE 2: A MAC entity configured with eIMTA monitors PDCCH in some subframe(s) in addition to PDCCH-subframes, as specified in clause 5.7. NOTE 3: For BL UE or UE in enhanced coverage, m is equal to the minimum number of subframes so that X PDCCH-subframes are included during the subframes [x, y]. |  |

For drx-InactivityTimerSCPTM, drx-InactivityTimer, drx-RetransmissionTimer and drx-ULRetransmissionTimer, if X=0, the timer does not make the MAC entity to monitor the PDCCH.

The intended UE behaviours in Table C-1 are not applicable for NB-IoT.

For NB-IoT, the intended UE behaviour regarding setting the HARQ RTT Timer is shown in Figure C-1 and for the UL HARQ RTT Timer is shown in Figure C-2.

Figure C-1: Setting the HARQ RTT Timer for NB-IoT

Figure C-2: Setting the UL HARQ RTT Timer for NB-IoT

END OF CHANGE