3GPP TSG RAN WG2#121	R2-2302048

Athens, Greece, 27th February - 3rd March, 2023

3GPP TSG RAN WG1 Meeting #112	R1- 2302146

Athens, Feb. 27th – March 3rd, 2023

Title:	LS Reply on PRU Procedures

Response to:	R1-2301915 (S2-2303861)

Release:	Rel-18

Work Item:	NR_pos_enh2-Core, 5G_eLCS_Ph3

Source:	RAN1

To:	SA2

Cc:	RAN2, RAN3

Contact person:

Name:	Gabi Sarkis

E-mail Address:	gsarkis@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks SA2 for their LS on PRU procedures and would like to provide the replies below.

SA2 request:

Regarding the procedure for location of a target UE using PRUs (clause 6.x.4 of the attached CR), SA2 noticed that RAN had agreements to support some positioning methods that require simultaneous measurements of the target UE and PRU. In the SA2 agreed procedures, it is assumed that the LMF can make the decision and realize such simultaneous measurements by using a common scheduled location time. SA2 would like to seek RAN1 and RAN2’s feedback on such assumption and whether additional enhancement would be needed to support the simultaneous measurements for the target UE and PRU. SA2 would update the specification according to the RAN1 and RAN2 feedbacks if necessary.

RAN1 reply:

Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU.

RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU.

Note: The enhancements might or might not have RAN1 specification impact.

SA2 request:

Additionally, SA2 also would like to know what location information can be provided by PRU, and whether RAN1 and RAN2 see benefit to allow PRU to provide its location information together with the location measurements reports.

RAN1 reply:

A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report.

# Actions

To SA2:

Action: RAN1 kindly asks SA2 to take the above reply into account for future work.

# Dates of Next TSG-RAN1 Meetings:

RAN1 #112-bis	17 April – 26 April 2023	Electronic

RAN1 #113	22 May – 26 May 2023	Incheon