3GPP TSG-RAN2 Meeting #121	R2-2302219

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1869 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capability parameter cellIndividualOffsetForNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Reliance Jio, CEWiT, Indian Institute of Technology Madras, Indian Institute of Technology Hyderabad, Saankhya Labs, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 02.03.2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To ensure backward compatibility of UEs supporting Cell specific offset support in NR measurement object for LTE with existing UEs a new UE capapbility parameter has been introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are introduced: 1. Added new UE capability parameter cellIndividualOffsetForNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In absence of new UE capability parameter cellIndividualOffsetForNR LTE BS would not be able to distinguish the UEs supporting Cell specific offset in NR measurement object for LTE from other UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 36.331 CR 4912 |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------Start of change----------------------

### 4.3.34	Inter-RAT Parameters NR

#### 4.3.34.1	en-DC-r15

This field indicates whether UE supports E-UTRA NR Dual Connectivity as specified in TS 37.340 [38].

#### 4.3.34.2	supportedBandListEN-DC-r15

Only applicable if the UE supports E-UTRA NR Dual Connectivity or NG-RAN E-UTRA-NR Dual Connectivity. This field includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

#### 4.3.34.3	supportedBandListNR-SA-r15

This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

#### 4.3.34.4	eutra-5GC-HO-ToNR-FDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.5	eutra-5GC-HO-ToNR-TDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.6	eutra-5GC-HO-ToNR-FDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR2. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.7	eutra-5GC-HO-ToNR-TDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-1 as specified in TS 38.101-x [xx]. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.8	eutra-EPC-HO-ToNR-FDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.9	eutra-EPC-HO-ToNR-TDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.10	eutra-EPC-HO-ToNR-FDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR2. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.11	eutra-EPC-HO-ToNR-TDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-1 as specified in TS 38.101-x [xx]. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.12	sa-NR-r15

This field indicates whether the UE supports standalone NR as specified in TS 38.331 [35].

#### 4.3.34.13	ims-VoiceOverNR-FR1-r15

This field indicates whether the UE supports IMS voice over NR FR1.

#### 4.3.34.14	ims-VoiceOverNR-FR2-r15

This field indicates whether the UE supports IMS voice over NR FR2-1 as specified in TS 38.101-x [xx].

#### 4.3.34.15	eventB2-r15

This field defines whether the UE supports event B2. In this release of specification, it is mandatory for a UE supporting NR SA operation to support eventB2-r15.

#### 4.3.34.16	ss-SINR-Meas-NR-FR1-r15

This field indicates whether the UE can perform NR FR1 SS-SINR measurement as specified in TS 38.215 [36].

#### 4.3.34.17	ss-SINR-Meas-NR-FR2-r15

This field indicates whether the UE can perform NR FR2 SS-SINR measurement as specified in TS 38.215 [36].

#### 4.3.34.18	ng-EN-DC-r15

This field indicates whether UE supports NG-RAN E-UTRA-NR Dual Connectivity as specified in TS 37.340 [38].

#### 4.3.34.19	nr-HO-ToEN-DC-r16

This field indicates whether the UE supports inter-RAT handover from NR to EN-DC while NR-DC or NE-DC is not configured as defined in TS 37.340 [38]. It is mandatory to support inter-RAT handover from NR to EN-DC if the UE supports E-UTRA NR Dual Connectivity.

#### 4.3.34.20	ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR FDD FR1. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.21	ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR TDD FR1. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.22	ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR FDD FR2-1 as specified in TS 38.101-x [xx]. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.23	ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR TDD FR2. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.24	extendedBand-n77-r16

This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 in the USA as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [33]. If absent, the UE supports only restriction to the 3700 - 3980 MHz range of band n77 in the USA. A UE that indicates this field shall also support NS value 55 as specified in TS 38.101-1 [33]. A UE supporting NS value 55 shall indicate this field.

#### 4.3.34.25	cellIndividualOffsetForNR

This parameter defines whether the UE supports cell specific offset for inter-RAT measurement in LTE for NR neighbors as specified in TS 36.331 [5].

#### 4.3.34.26	eutra-5GC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-2 as specified in TS 38.101-x[xx]. A UE that indicates this field also supports eutra-5GC-r15. A UE supporting handover from E-UTRA/5GC to NR TDD FR2-2 shall also support the RRM measurements for FR2-2 as specified in TS 36.331 [5].

#### 4.3.34.27	eutra-EPC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-2 as specified in TS 38.101-x[xx]. A UE supporting handover from E-UTRA/EPC to NR TDD FR2-2 shall also support the RRM measurements for FR2-2 as specified in TS 36.331 [5].

#### 4.3.34.28	ims-VoiceOverNR-FR2-2-r17

This field indicates whether the UE supports IMS voice over NR FR2-2 as specified in TS 38.101-x[xx].

#### 4.3.34.29	ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR TDD FR2-2 as specified in TS 38.101-x[xx]. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.30	extendedBand-n77-2-r17

This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 in Canada as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [33]. If absent, the UE supports only restriction to the 3450 - 3650 MHz range of band n77 in Canada. A UE that indicates this field shall also support NS value 57 as specified in TS 38.101-1 [33]. A UE supporting NS value 57 shall indicate this field.

----------------------------End of change----------------------