3GPP TSG RAN WG2#121	R2-2300082

Athens, Greece, 27th February - 3rd March, 2023

# 3GPP TSG-SA WG2 Meeting #154-AH-e	S2-2301858

Electronic Meeting, 16 - 20 January, 2023	(Revision of S2-2300932)

Title:	Reply LS on long eDRX support for RRC_INACTIVE

Response to:	Reply LS on long eDRX support for RRC_INACTIVE

Release:	Rel-18

Work Item:	NR_REDCAP_Ph2

Source:	SA2

To:	RAN3, RAN2

cc:

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 4081, 23.502 CR 3685

1. Overall Description:

SA2 thanks RAN3 for the answers provided in S2-2300055/R3-226776 and waits for further responses for issues that RAN3 indicated with no agreement yet.

SA2 would also like to inform RAN3 and RAN2 that SA2 agreed CRs related to the triggering of the RAN request for CN to handle the MT communication and related to the eDRX parameter handling for RRC_INACTIVE.

SA2 would like to check the understanding of the first bullet in the reply from RAN3:

- On the triggering criteria for gNB sending a CN based MT communication handling request to CN, it is decided by RAN3 to leave it to network implementation.

SA2 would like to clarify further that the scope of CN based MT communication handling request to CN is for eDRX cycle lengths larger than 10.24s, as for values equal or smaller than 10.24s there is no needed action from 5GC  as per Rel-17 solution. However, the RAN3 answer is unclear on whether the scope of the answer is (1) for eDRX cycle lengths larger than 10.24s or (2) for any eDRX cycle length (included <=10.24s). SA2 assumes it is only for values larger than 10.24s (1), but would like to ask RAN3 for clarification, especially if the intention was (2).

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN3 and RAN2 to take above information into consideration and provide feedback as requested.

3. Date of Next SA2 Meeting:

SA2#155	February 20 - 24, 2023	Athens, GR

SA2#156	April 17 - 21, 2023 	TBD