3GPP TSG RAN WG2#121	R2-2301932

Athens, Greece, 27th February - 3rd March, 2023

3GPP TSG-SA WG2 Meeting #155	S2-2303306

20 - 24 February, 2023, Athens, Greece	rev of S2-2303292

# Title:	Response to “LS from NRG to 3GPP SA2 on UEs behaviour on detecting an emergency call whilst in Limited Service State”

# Response to:	S2-2302198 = NRG016_207

# Release:	Rel-18

# Work Item:

Source:	SA2

To:	GSMA NRG, CT1, RAN2

Cc:

Contact Person:

Name:	Chris Pudney

Tel. Number:

E-mail Address:	chris dot Pudney at Vodafone dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: CR 0370 to TS 23.167

1. Overall Description:

SA 2 thanks GSMA NRG for their LS and its embedded change to the VoLTE Implementation Guide in NRG 016_006r5.

SA 2 has discussed the issue and agreed the attached CR to TS 23.167.

The CR does not exactly align to GSMA NRG’s changes in NRG 016_006r5, but SA 2 hopes that it meets GSMA NRG’s requirements. To explain the differences, SA 2 provides the following comments:

- We believe that existing 3GPP specifications cover the case of the UE being PS attached but not IMS registered in the VPLMN.

- As specified (see below) in e.g., clause 5.2.9 of TS 36.304, a UE in limited service state will attempt to find an ‘acceptable cell’, and an ‘acceptable cell’ is one that supports emergency calls. Hence it will not camp on an E-UTRAN cell that is broadcasting “emergency call not supporting” unless the UE is out of coverage of all acceptable cells. RAN 2 and CT 1 may wish to comment on this.

-	if the UE supports voice services and the current cell does not support emergency call as indicated in System information specified in TS 36.331 [3], the UE should perform cell selection/ reselection to an acceptable cell of any supported RAT regardless of priorities provided in system information from current cell, …

- We assume that 2G and 3G cells support CS domain emergency calls.

- We anticipate that it is necessary to clarify the handling when the VPLMN has no 2G/3G radio coverage but still allows the UE to use “SMS over NAS” (either “SMS over SGs” or “SMS in MME/over SGd”).

- We believe that, in the following paragraph from NRG 016_006r5, “without” could be clarified, e.g.:

UEs in limited service state can only perform an IMS emergency call by performing emergency registration with the MME/AMF but without IMS emergency registration. This is also applicable to UEs with reduced/limited voice capabilities, e.g. inbound roamers where there is no VoLTE Roaming agreement in place, also do not perform IMS emergency registration.

2. Actions:

To GSMA NRG

ACTION:	SA 2 kindly asks NRG to take the above into account and provide further feedback if needed.

To RAN 2 and CT 1

ACTION:	SA 2 kindly asks RAN 2 and CT1 to take the above into account and provide feedback if needed.

3. Dates of Next TSG SA WG2 Meetings:

TSG-SA 2 Meeting #156-e	 Apr 17 - 21, 2023				Elbonia

TSG-SA 2 Meeting #157	May 22 - 26, 2023				Berlin