3GPP TSG RAN WG2#121	R2-2302016

Athens, Greece, 27th February - 3rd March, 2023

3GPP TSG-RAN WG4 Meeting # 106	R4-2303684

# Athens, Greece, February 27 – March 3, 2022

Title:	LS on applicability of SIB19 for NR ATG

Release:	Rel-18

Work Item:	NR_ATG-Core

Source:	RAN4

To:	RAN2

Contact person:

Name:	Bin Han

E-mail Address:	binhan@qti.qualcomm.com

# 1   Overall Description:

RAN4 agreed ATG UE needs to perform time and/or frequency compensation. ATG BS is expected to provide location information to assist UE-based time and/or frequency pre-compensation. The signaling of BS location could use the IE EphemerisInfo in TS38.331 (as defined for NTN SIB19). Other solutions to provide ATG BS location information are also not precluded in RAN4.

From RAN4 perspective, with the assumption of reusing SIB19 content for ATG, BS shall broadcast EphemerisInfo to ensure ATG BS location can be received for the UEs which rely on SIB EphemerisInfo to do the frequency/time pre-compensation. Note that ATG network will operate on the TN bands and the example bands discussed in RAN4 are including n1, n77 and n79.

RAN4 would like to respectively request RAN2 to check whether SIB19 could be applied for ATG and if not, whether an alternative could be applied to provide the ATG BS location to ATG UE from RAN2 perspective.

# 2   Actions

To RAN2:

RAN4 would like to respectfully request RAN2 to take the above information into account and check whether SIB19 could be applied for ATG. If SIB19 cannot be applied, RAN2 is kindly requested to create an alternative solution.

# 3   Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #106-bis-e	April 2023	Online

TSG-RAN WG4 Meeting #107	May 2023	Korea