3GPP TSG-RAN WG2 Meeting #121 	R2-2302295

# Athens, Feb-March 2023

Title:	LS on further questions on feMIMO RRC parameters

Response to: 	-

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	3GPP TSG-RAN WG2

To:	3GPP TSG-RAN WG1

Contact Person:

#### Name:	Helka-Liina Määttänen

E-mail Address:	Helka-liina.maattanen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

UL power control configuration in IE TCI-State

RAN2 discussed the field description of pathlossReferenceRS-Id and ul-powerControl in TCI-state (for Rel-17 joint TCI states) and cell in QCL-Info in TCI-State.

We present the question with TCI-State IE:

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                             OPTIONAL,   -- Need R

...,

[[

additionalPCI-r17                   AdditionalPCIIndex-r17               OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17          PUSCH-PathlossReferenceRS-Id         OPTIONAL,   -- Cond JointTCI

ul-powerControl-r17                 Uplink-powerControlId-r17            OPTIONAL    -- Cond JointTCI

]]

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                        OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                               OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

The current field descriptions states:

ul-PowerControl

Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell.

pathlossReferenceRS-Id

The ID of the reference signal (e.g. a CSI-RS  or a SS block) used for PUSCH, PUCCH and SRS path loss estimation.

cell

The UE's serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-State is configured. The RS can be located on a serving cell other than the serving cell in which the TCI-State is configured only if the qcl-Type is configured as typeC or typeD. See TS 38.214 [19] clause 5.1.5.

Same issue and questions apply to the field description of pathlossReferenceRS-Id and ul-powerControl in TCI-UL-state and servingCellId in TCI-UL-State.

#### –	TCI-UL-State

The IE TCI-UL-State indicates the TCI state information for UL transmission.

TCI-UL-State information element

-- ASN1START

-- TAG-TCI-UL-STATE-START

TCI-UL-State-r17 ::=             SEQUENCE {

tci-UL-State-Id-r17              TCI-UL-State-Id-r17,

servingCellId-r17                ServCellIndex                                         OPTIONAL,   -- Need R

bwp-Id-r17                       BWP-Id                                                OPTIONAL,   -- Cond CSI-RSorSRS-Indicated

referenceSignal-r17              CHOICE {

ssb-Index-r17                    SSB-Index,

csi-RS-Index-r17                 NZP-CSI-RS-ResourceId,

srs-r17                          SRS-ResourceId

},

additionalPCI-r17                AdditionalPCIIndex-r17                                OPTIONAL,   -- Need R

ul-powerControl-r17              Uplink-powerControlId-r17                             OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17       PathlossReferenceRS-Id-r17                      OPTIONAL,   -- Cond Mandatory

...

}

-- TAG-TCI-UL-STATE-STOP

-- ASN1STOP

servingCellId The UE's serving cell in which the referenceSignal is configured. If the field is absent, it applies to the serving cell in which the TCI-UL-State is applied by the UE.

pathlossReferenceRS-Id The ID of the reference Signal (e.g. a CSI-RS or a SS block) used for PUSCH, PUCCH and SRS path loss estimation.

ul-powerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell.

RAN2 would like to check the understanding of the pathlossReferenceRS-Id and ul-powerControl fields.

For pathlossReferenceRS-Id, RAN2 considered two possibilities:

1) pathlossReferenceRS-Id is an index referring to a list element in a list configured in the UL BWP and serving cell where the TCI state is applied, regardless if the field cell is configured in IE TCI-State or not, or respectively the field servingCellId in IE TCI-UL-State or not.

2) When the field cell is configured in IE TCI-State(or the field servingCellId in IE TCI-UL-State), pathlossReferenceRS-Id is an index referring to a list element in a list configured in an UL BWP of the serving cell indicated by cell (or servingCellId ). When cell (or servingCellId ) is absent, pathlossReferenceRS-Id is an index referring to a list element in a list configured in the UL BWP and serving cell where the TCI state is applied.

In 2), when cell (or servingCellId) ). is absent, it may be necessary to add a new field to indicate in which UL BWP the list is to be found.

Question 1

Is 1) or 2) or yet another alternative the correct understanding for pathlossReferenceRS-Id? If it is 2), please indicate how to know the UL BWP when the field cell (or servingCellId ) is present .

For ul-powerControl, RAN2 considered two possibilities:

1) ul-powerControl is an index referring to a list element in a list configured in the serving cell where the TCI state is applied, regardless if the field cell (or servingCellId )  is configured or not.

2) When the field cell (or servingCellId ) is configured, ul-powerControl is an index referring to a list element in a list configured in the serving cell indicated by cell (or servingCellId ). When cell (or servingCellId ) is absent, ul-powerControl is an index referring to a list element in a list configured in the serving cell where the TCI state is applied.

Question 2

Is 1) or 2) or yet another alternative the correct understanding for ul-powerControl?

2. Actions:

To RAN1 group:

ACTION: 	RAN2 respectfully asks RAN1 to provide responses to above questions.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #121b-e 	April 2023    Electronic

TSG-RAN WG2 Meeting #122 	May 2023    Korea