3GPP TSG RAN WG2 Meeting #121bis-e	R2-2302610

Electronic meeting, April 17th – April 26th, 2023

Agenda item:	7.11.3

Source:	TD Tech, Chengdu TD Tech

Title:	Simultaneous unicast reception and broadcast reception

Document for:	Discussion and Decision

- Introduction

In the RAN2#121 meeting, the following agreement was made on the simultaneous unicast reception and MBS broadcast reception in UE for the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services”.

Indicate the capability of receiving MBS broadcast from a non-serving cell. FFS whether the granularity is at FeatureSetDownlink or FeatureSetDownlinkPerCC level.

FFS Whether to include additional information in MII can be controlled by the network. Should consider whether this would be two-step procedure or one-step procedure (e.g. having more info in SIB1)

In the contribution, the simultaneous unicast reception and MBS broadcast reception in UE is further discussed with the related proposals suggested.

- Simultaneous unicast reception and MBS broadcast reception in UE

For the simultaneous unicast reception and MBS broadcast reception in UE, the following questions need to be solved.

Question 1: When to trigger the MII reporting procedure by UE?

Question 2: Whether or not other information needs to be added to the MII message?

For Question 1, UE can trigger the MII procedure in the following scenarios:

- When UE finds it can’t support the simultaneous unicast reception and broadcast reception

- When UE finds a broadcast session can’t be received with an acceptable performance

For example, if UE finds the BLER of a broadcast session is higher than the target BLER, it can trigger the MII reporting procedure.

For Question 2, the content of the MII message can be extended to include the following information:

- SCS for broadcast reception in the non-serving cell

- Bandwidth for broadcast reception in the non-serving cell

Based on the discussion above, the following proposals are suggested.

Proposal 1: UE triggers the MII reporting procedure for the simultaneous unicast reception and broadcast reception under the following scenarios:

- When UE finds it can’t support the simultaneous unicast reception and broadcast reception

- When UE finds a broadcast session can’t be received with an acceptable performance

Proposal 2: The following extra information is carried by MII message.

- SCS for broadcast reception in the non-serving cell

- Bandwidth for broadcast reception in the non-serving cell

- Conclusions

The simultaneous unicast reception and MBS broadcast reception is discussed in the contribution with the following proposals suggested:

Proposal 1: UE triggers the MII reporting procedure for the simultaneous unicast reception and broadcast reception under the following scenarios:

- When UE finds it can’t support the simultaneous unicast reception and broadcast reception

- When UE finds a broadcast session can’t be received with an acceptable performance

Proposal 2: The following extra information is carried by MII message.

- SCS for broadcast reception in the non-serving cell

- Bandwidth for broadcast reception in the non-serving cell

- References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e