3GPP TSG-RAN2 Meeting #121-bis	R2-2302639

# Electronic, April 17-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0431 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on applicability of timing error margin of RxTEG in NR-Multi-RTT-SignalMeasurementInformation field descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the LS from RAN4: R2-2301999 Reply LS on applicability of timing error margin of Rx TEG (R4-2303244; contact: CATT), the applicability of timing error margin for Rx TEG applies to RSTD measurement only, not for the Rx TEG in multiRTT. RAN4 feedback: The applicability of timing error margin for Rx TEG applies to RSTD measurement only. So the description of applicability of timing error margin for Rx TEG in NR-Multi-RTT-SignalMeasurementInformation which refers to TS38.133 should be deleted because applicability of timing error margin for Rx TEG does not apply to UE Rx-Tx timing difference in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LMF should consider the UE Rx TEG timing error margin value in NR-Multi-RTT-SignalMeasurementInformation to be the maximum value available in IE TEG-TimingErrorMargin, if the IE NR-UE-RxTx-TEG-Info is present with choice case3 and nr-UE-RxTEG-TimingErrorMargin is absent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of applicability of timing error margin for Rx TEG in NR-Multi-RTT-SignalMeasurementInformation is not correct and not aligned with TS 38.133. Impact analysis Impacted functionality: Multi-RTT Inter-operability: If the UE is implemented according to the CR while the network is not, the network won’t set the correct timing error margin value of RxTEG. If the network is implemented according to the CR while the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.5.12.4	NR Multi-RTT Location Information Elements

–	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Case2-3 | The field is mandatory present if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice case3. Otherwise it is not present. |
| TEGCase2-3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase1-2 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case1 and case2. Otherwise it is not present. |

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-NTA-Offset This field provides the NTAoffset used by the target device as specified in TS 38.133 [46], Table 7.1.2-2. Enumerated values nTA1, nTA2, nTA3, and nTA4 correspond to NTAoffset of 25600 Tc, 0 Tc, 39936 Tc, and 13792 Tc, respectively.

nr-SRS-TxTEG-Set This field provides the SRS for Positioning Resources associated with a particular UE Tx TEG and comprises the following subfields: - nr-TimeStamp specifies the start time for which the NR-SRS-TxTEG-Element is valid. If this field is absent, the nr-TimeStamp of this instance of the NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set is the same as the nr-TimeStamp of the previous instance of the NR-SRS-TxTEG-Element. If this field is also absent in the first NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set, all NR-SRS-TxTEG-Element's provided are valid for the measurement period of the NR-Multi-RTT-SignalMeasurementInformation. - nr-UE-Tx-TEG-ID specifies the ID of this UE Tx TEG. - carrierFreq specifies the frequency of the SRS for positioning resources. - srs-PosResourceList specifies the SRS for Positioning Resources belonging to this UE Tx TEG. For each UE Tx TEG, there may be up to 8 changes (different nr-TimeStamp) of the TEG-SRS association information provided in nr-SRS-TxTEG-Set, i.e., the maximum value for maxTxTEG-Sets is 64.

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case3 and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-TxTEG-TimingErrorMargin This field specifies the UE Tx TEG timing error margin value for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case2 or case3 and this field is absent, the receiver should consider the UE Tx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-RxTxTEG-TimingErrorMargin This field specifies the UE RxTx TEG timing error margin value for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case1 or case2 and this field is absent, the receiver should consider the UE RxTx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 38.215 [36].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimeStamp This field specifies the time instance for which the measurement is performed.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the measurement.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-UE-RxTx-TEG-Info This field provides the ID(s) of the UE TEG associated with the nr-UE-RxTxTimeDiff or nr-UE-RxTxTimeDiffAdditional measurement. One of the following combinations of TEG IDs can be provided: - case1 provides the UE RxTx TEG ID; - case2 provides the UE RxTx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on; - case3 provides the UE Rx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the TRP or resource. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-Multi-RTT-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-Multi-RTT-MeasElement, provides UE Rx-Tx time difference measurements of up to 4 DL-PRS Resources of a TRP with different UE RxTx or UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 measurement results with respect to different UE RxTx or UE Rx TEGs. If this field is present, the field nr-Multi-RTT-AdditionalMeasurements should not be present.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result. The mapping of this field is defined as in TS 38.133 [46].

nr-UE-RxTxTimeDiffAdditional This field provides the additional UE Rx-Tx Difference measurement result relative to nr-UE-RxTxTimeDiff. The UE Rx-Tx Difference value of this measurement is obtained by adding the value of this field to the value of the nr-UE-RxTxTimeDiff field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the resource. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

END OF CHANGE