3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2302738

# e-Meeting, 17th April – 26th April 2023

Agenda item:			7.2.1

Source:			Intel Corporation

Title:		Further Considerations on new SLPP specification

Document for:	 	Discussion and decision

# Introduction

At RAN2#121, based on [5] [6],  RAN2 discussed the structure of the SLPP specification and concluded:

Agreement:

Use “SLPP” (without hyphen) as the name of the new protocol.

Regarding the structure of SLPP, e.g. general part, procedure part , Information Element Abstract Syntax Definition,  the structure of LPP (TS 37.355) can be used as baseline for further discussion. The content of each section will be added in accordance with future agreements, not based on LPP legacy directly. FFS on procedure description in the field description as LPP.

Regarding the ASN.1 part of SLPP, follow NR RRC approach, e.g.

-	FFS on Need code (e.g. how to support no UL/DL), support of delta signalling

-	Define ASN.1 elements for common UE capabilities in a dedicated section (i.e. “UE capability information elements”); FFS whether any positioning method specific capability IEs should be grouped by positioning method.

-	Common section for constraints

-	“nonCriticalExtension” at message level

-	Fields in the field description are sorted based on alphabetical order

-	FFS on whether setup release structure should be introduced in SLPP

At RAN#99, RAN plenary approved the updated WID on expanded and improved NR positioning [7] and allocated the specification number TS38.355 for new specification on Sidelink Positioning Protocol as:

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TS | 38.355 | NR; Sidelink Positioning Protocol (SLPP); Protocol Specification | 101 | 102 | Core part, RAN2-led Editor: Yi Guo (yi.guo@intel.com) |

In this contribution, we continue the discussion on the structure of TS38.355.

# Discussion

### 2.1 TS Skeleton

Based on agreements made in RAN2#121, the TS Skeleton is provided in [8].

In summary, we captured following agreements in [8].

Regarding the structure of SLPP, e.g. general part, procedure part , Information Element Abstract Syntax Definition,  the structure of LPP (TS 37.355) can be used as baseline for further discussion.

Regarding the ASN.1 part of SLPP, follow NR RRC approach, e.g.

Define ASN.1 elements for common UE capabilities in a dedicated section (i.e. “UE capability information elements”);

-	Common section for constraints

Proposal 1: Endorse the TS Skeleton in R2-230xxxx as baseline for further updates.

### 2.2 Open issues

For details and FFS, further updates are needed when RAN2 has further concrete agreements on procedures.

-	“nonCriticalExtension” at message level

-	Fields in the field description are sorted based on alphabetical order

-	The content of each section will be added in accordance with future agreements, not based on LPP legacy directly.

-	FFS on procedure description in the field description as LPP.

-	FFS on Need code (e.g. how to support no UL/DL), support of delta signalling

-	FFS whether any positioning method specific capability IEs should be grouped by positioning method.

-	FFS on whether setup release structure should be introduced in SLPP

Regarding the open issue “FFS on Need code (e.g. how to support no UL/DL)”, it was raised during offline discussion [6] , i.e. how to define Need Code for SLPP message since it is different from LPP or RRC message, SLPP has no UL/DL differences. “While for PC5, there is no UL/DL difference. So the current PC5-RRC is quite unclear in this aspect. Hope the issue can be well addressed in SLPP”.

Currently the Need codes defined in NR RRC are used in the downlink direction for RRC message and also in the sidelink for PC5 RRC message as agreed in [9].

| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| --- | --- |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

To our understanding, the principle used for PC5 RRC is to follow legacy RRC, i.e. Need code is applied if the PC5 RRC message is defined as downlink in legacy RRC, e.g. Need code is applied for RRCReconfigurationSidelink  message, but not applied for RRCReconfigurationCompleteSidelink  message. We can follow the same principle for SLPP message, i.e. Need code is applied for the messages which are provided from anchor/server to a target UE.

Proposal 2: Need code is applied for SLPP messages transmitted from the anchor/server node/UE.

Similar to PC5 RRC, if some IE definitions from LPP can be reused for SLPP, we may simply import them from LPP specification, as

IMPORTS

Xxx

FROM LPP-PDU-Definitions;

Proposal 3: We may import some IE definitions from LPP specification if needed.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: Endorse the TS Skeleton in R2-230xxxx as baseline for further updates.

Proposal 2: Need code is applied for SLPP messages transmitted from the anchor/server node/UE.

Proposal 3: We may import some IE definitions from LPP specification if needed.

# Reference

[1] RP-223549 New WID on Expanded and Improved NR Positioning

[2] TS 36.331

[3] TS 37.355

[4] TS 38.331

[5] R2-2300409	Considerations on new SLPP specification	Intel Corporation

[6] R2-2302147	[AT121][411][POS] SLPP specification (Intel)	Intel Corporation

[7] RP-230328	Revised WID: Expanded and Improved NR Positioning	Intel Corporation

[8] R2-230xxxx	TS 38.355 Skeleton	Intel Corporation

[9] R2-2204321 [H634] Correction for the need code and conditions for optional fields in PC5 RRC message	Huawei, Hisilicon