3GPP TSG RAN WG2 Meeting #121bis-e       	      	                	               R2- 2302748

Electronic, 17th – 26th Apr, 2023

Agenda Item:	7.16.2.3

Source:	Intel Corporation

Title:	Architecture impact on model transfer method

Document for:	Discussion and Decision

# Introduction

During RAN2 #121 meeting, following solutions were agreed and their analyzed pros/cons are summarized in the table.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

Considering the selection of solutions are highly related to the architecture assumption and LCM entity mapping within the network. In this contribution, we further analysis the architecture assumption impact to model transfer method.

# Discussion

## General Assumptions

#### Model Format

Proprietary model format and open format used for model transfer are discussed in RAN1.

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

It is understanded that the proprietary model may not support interoperability, while the open format allows model formats (e.g. ONNX, etc) that can be shared across different vendors. For proprietary model, during model transfer, the network can only transfer the model as a whole to the UE. However, for open format, it allows a possibility to transfer the model associated with its model parameters and model structure to the UE. The model parameter and model structure may not transfer together with the model itself.

Hence, from RAN2 point of view, the model itself being transferred as a container is equivalent between open format and proprietary model.

Observation 1: RAN2 can discuss model transfer solution independent from what model format is used assuming the model itself is transferred as a container.

#### Model transfer within network

To understand better about the limitation and supported scenario of different solutions, it is necessary to combine the architecture assumption (e.g. LCM functionality mapping) with model transfer methods.

In our understanding, the locations listed in above solutions are not necessarily mean they are the locations for model generation. The solutions only show the link between UE and the actual node to  transfer/delivery the model at the network side. It does not preclude the AI/ML model to be transferred within network from the upper layer (e.g. OAM, 3rd party server). In our companion contribution [1], by jointly consider RAN1 agreements in RAN1 #112 meeting, following model transfer options can be considered:

| Model storage | Model transfer solution in RAN2 | Model transfer within network entities |
| --- | --- | --- |
| CN | Solution 2a, 3a, 2b, 3b | CN -> UE |
|  | Solution 1b | CN -> RAN -> UE |
| OAM | Solution 1a | OAM -> RAN -> UE |
|  | Solution 4 | OAM -> UE |
| NG-RAN | Solution 1a, 1b | NG-RAN -> UE |

It is possible that the model is first transferred within the network, then use RAN2 agreed solutions for model delivery over the air interface.

Proposal 1: RAN2 confirms that the network entity used for model storage can be different from the entity listed in RAN2 agreed solutions, that is, model delivery within network entities needs to be supported if model storage location is not the same as the actual node to delivery/transfer the model.

#### Mobility Support

As captured in the model transfer solution table, whether service continuity on model transfer can be supported or not is one of the key evaluation points. However, based on our understanding, for AI/ML models, whether mobility needs to be supported is questionable.

First of all, as discussed in RAN1, the AI/ML model for one sub-use case can be site/scenario/configuration-specific. When one UE moves from one cell to another, due to the change of configuration or environment, it is highly possible that the UE needs to require a new model. Therefore, there’s limited benefit to support AI/ML model transfer service continuity, since anyway a new model needs to be transmitted to the UE after its handover.

Even though a unified model can be trained for general purpose, the performance of the unified model may be worse than the site/scenario/configuration specific models due to lack of uniqueness. It would be doubtful that if a UE prefers to prioritize using a generic model over specific models for service continuity purpose.

Furthermore, to support service continuity, especially when model is stored at gNB, sharing/transferring AI/ML between NG-RAN nodes needs to be supported. However, during RAN3 AI/ML NG-RAN WI discussion, such scenario is not supported.

Observation 2: There’s little benefit to support mobility, considering the model can be site/scenario/configuration specific which provides a better performance than a generic model. Model transfer between NG-RAN nodes is also not supported by RAN3.

Therefore, when analyzing the pros/cons of different solutions, we propose to remove mobility aspects from the analysed table.

Proposal 2: Service continuity of AI/ML model transfer is not supported.

## Further analysis on Solution 1a

During RAN2 #121 meeting discussion, all other solutions except solution 1a has dependency with other WGs. Therefore, in this contribution, we mainly focus on solution 1a, where RAN2 could further progress and consider how to support model transfer with limited optimization.

It was captured in R2-2302268 [2], solution 1a has following limitations:

1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes)

2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery

3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the gNB to send new/updated AI/ML to the UE

For the 1st and 2nd limitation, as discussed earlier in above table, the model can be either stored at OAM or gNB. though the existing RRC signaling cannot support large size of AI/ML model’s transfer due to limited segmentation number, when the model is stored at OAM, this limitation could be solved by allowing model segmentation takes place at OAM, hence avoid exceeding maximum size of RRC message. Furthermore, storing AI/ML model at OAM can also reduce workload and storage challenge at gNB side.

Even the model is stored at gNB, increasing the support the number of RRC segmentation may not be a challenging issue.

For the 3rd limitation, as discussed earlier, there’s limited benefit to support mobility for AI/ML model transfer.

Based on above analysis, Solution 1a is feasible.

Furthermore, whether to use existing SRB or a new SRB was also discussed. As mentioned earlier, the AI/ML model is not an urgent task and require real-time transmission. Similar as QoE, a new low priority SRB is used to carry AI/ML model. The priority of this new SRB can be the same as SRB4 for Rel-17 QoE.

Proposal 3: RAN2 confirms that Solution 1a is feasible and following observations are proposed to be captured in the TR:

- In Solution 1a, model is stored at OAM, and transmitted to NG-RAN for model transfer

- In Solution 1a, the large size AI/ML model can be segmented at upper layer (e.g. OAM) within the maximum size of RRC message. No need to extend existing segmentation number supported by RRC.

- In Solution 1a, A new low priority SRB is introduced to carry AI/ML model.

As we discussed earlier, there’s limited benefit to support service continuity. However, even one may want to support continuity during handover, it can be solved by simply extending PDCP reestablishment to SRB without impacting NG-RAN architecture or general principle of establishing DRB in 5G era.

As captured in TS38.323 [3], current PDCP entity reestablishment does not support for SRB. To achieve continuity during handover, this could be simply achieved by extending current PDCP entity reestablishment to this new SRB used for AI/ML model.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall: - for SRBs (except SRBx for AI/ML model) and UM DRBs, set TX_NEXT to the initial value; - for SRBs (except SRBx for AI/ML model), discard all stored PDCP SDUs and PDCP PDUs; - for SRBx (for AI/ML model) and AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below: - … - submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1. When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall: - process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1; - for SRBs (except SRBx for AI/ML model), discard all stored PDCP SDUs and PDCP PDUs; - for SRBs (except SRBx for AI/ML model), UM DRBs and UM MRBs, if t-Reordering is running: - stop and reset t-Reordering; - for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression; - for SRBs (except SRBx for AI/ML model), UM DRBs and UM MRBs, set RX_NEXT and RX_DELIV to the initial value;

Proposal 4: If RAN2 agrees to require service continuity for model transfer, to support Solution 1a with service continuity, following specification changes are expected:

- PDCP reestablishment is applicable to this new SRB .

# Conclusion

In this contribution, we analyze the general architecture impact to model transfer method:

Observation 1: RAN2 can discuss model transfer solution independent from what model format is used assuming the model itself is transferred as a container.

Proposal 1: RAN2 confirms that the network entity used for model storage can be different from the entity listed in RAN2 agreed solutions, that is, model delivery within network entities needs to be supported if model storage location is not the same as the actual node to delivery/transfer the model.

Observation 2: There’s little benefit to support mobility, considering the model can be site/scenario/configuration specific which provides a better performance than a generic model. Model transfer between NG-RAN nodes is also not supported by RAN3.

Proposal 2: Service continuity of AI/ML model transfer is not supported.

Proposal 3: RAN2 confirms that Solution 1a is feasible and following observations are proposed to be captured in the TR:

- In Solution 1a, model is stored at OAM, and transmitted to NG-RAN for model transfer

- In Solution 1a, the large size AI/ML model can be segmented at upper layer (e.g. OAM) within the maximum size of RRC message. No need to extend existing segmentation number supported by RRC.

- In Solution 1a, A new low priority SRB is introduced to carry AI/ML model.

Proposal 4: If RAN2 agrees to require service continuity for model transfer, to support Solution 1a with service continuity, following specification changes are expected:

- PDCP reestablishment is applicable to this new SRB .

# References

- R2-2302746, General architecture assumptions, model ID and entity mapping, Intel Corporation

- R2-2302268, Report of Offline 027 model transfer delivery

- TS38.323, NR; Packet Data Convergence Protocol (PDCP) specification