3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303265

E-meeting, 17th – 26th Apr. 2023

Agenda Item:	7.12.2

Source: 	vivo

Title:         	Mobile IAB remaining issues

Document for: 	Discussion and Decision

- Introduction

At RAN2#120 meeting, RAN2 achieved the following agreements:

- R2 assumes that It is up to RAN3 or SA2 to decide whether to support early mobile IAB indication in Msg5 because it depends whether donor CU needs to select an AMF supporting mobile IAB. - R2 assumes that Donor CU can determine mobile IAB node's moving status via legacy reporting (e.g. mobility state and UE location / velocity specified in SON/MDT), i.e. R2 assumes enhanced / new reporting is not needed. - A mobile IAB node may camp on and connect to legacy Rel-16/Rel-17 IAB capable cell. - R2 assumes "supporting mobile-IAB" indication is provided by Rel-18 Mobile IAB capable parent cell. - Regarding the assumed mobile-IAB cell type indication, RAN2 assumes is may be specified if some related UE behaviour is specified.

At RAN2#121 meeting, RAN2 achieved the following agreements:

Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection. FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

In this contribution, we further discuss some mIAB mobility enhancement issues:

- Discussion

  - Broadcasts indication of “supporting mobile-IAB”

Currently, the iab-support indication is used to indicate whether a cell is IAB-capable cell. From our understanding Mobile IAB are basically IAB as in R16/R17. the main different is that mobile IAB are nomadic in nature. From camping perspective, mobile IAB seems to present anything different from static IAB. Thus, the current iab-support is sufficient to allow a mobile IAB to camp on a cell or not.

- No need to Introduce that stationary network broadcasts indication of supporting mobile-IAB camping. The iab-support is sufficient to allow mobile IAB camping on an IAB-capable cell

Even, if a cell A is static IAB only capable, a mobile IAB can still be able to connect to such cell as normal IAB. If needed, the cell can still redirect to a neighboring mobile IAB capable cell or the mobile IAB perform reselection to neighboring mobile IAB capable cell.

Therefore,

- Rel-16/Rel-17 IAB procedure with additionally mobile IAB cell reselection or redirection is sufficient to guarantee mobile IAB connection to network, without network broadcasting indication of supporting mobile-IAB

  - Mobile-IAB cell type indication

RAN 2 agreed that 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs. As mobile IAB are also supposed to serve legacy, by specifying the usage of such bit indication may impact legacy accessing to mobile IAB. Thus,

- Specifying how the 1 bit mobile-IAB cell type indication is used by UE, may have negative impact on legacy UE accessing mobile IAB.

Therefore,

- The usage of the 1 bit mobile-IAB cell type indication by Rel-18 UE is left to implementation

  - Conditional handover execution

For conditional handover execution of group HO, at last meeting the following options were discussed:

Option 1: The RRC Reconfiguration messages are sent to the logical source IAB-DU, where they are withheld until a condition has been met, e.g., the IAB-MT has received its own handover command.

Option 2: The RRC Reconfiguration messages are sent to the UEs, where they are withheld until a condition has been met, which may be based on a broadcast by the logical source IAB-DU.

Option 3: Legacy CHO is configured on the UEs and the handover is triggered by powering down the source logical IAB-DU cell.

RAN2 has assumed that Option 1 and Option 3 could work, but no clear assumption for Option 2.

From Option 2 description, it may require source IAB-DU to broadcast some additional indication to UE in other to properly execute HO. This new information broadcast can be used by Rel-18 UEs to execute the HO when the necessary conditions are fulfilled. But, as legacy UEs does not recognize such new indication, Option 2 may not work for legacy UEs.

- Option 2 may not be applicable to legacy UEs as it requires IAB-DU to broadcast some additional indication.

Therefore,

- Solution Option 2, i.e., “The RRC Reconfiguration messages are sent to the UEs, where they are withheld until a condition has been met, which may be based on a broadcast by the logical source IAB-DU.” is deprioritized as it may not work for legacy UEs.

Based on RAN2#120, it can be observed that Options 1 and Option 3 (as they are Rel17 and earlier with no change) seem more preferable. Additionally, as Option 1 can support legacy UEs without further optimization, it can be considered as baseline.

Therefore,

- For simplicity Solution Option 1 is considered as baseline.

  - Cell search for mobile IAB

Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection. FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

For cell search for mobile IAB, mobile IAB is not normal cell and UE do not get to encounter them frequently, but rather UE access to them in some specific area. This seems similar to CAG cell. Network or mobile IAB may broadcast some assistance information for UE to search for mobile IAB. But, UE may use the information maybe once or twice in year. So, the information is not very useful in practical network.

- Assistance information for mobile IAB cell search is not very useful in practical network, as UE may rarely use such information.

When on-board of mobile IAB, if UE (re)select to non-mobile IAB cell, UE would keep performing cell reselection along mobile IAB trajectory and as mobile IAB cell quality is constantly good for UE, UE could definitively select the mobile IAB based on UE cell reselection configuration setting.

- Existing UE cell reselection configuration would allow UE to detect and UE can autonomously reselect to mobile IAB.

Therefore,

- There is no need for any assistance information for UE search and measure for mIAB cells on different frequencies.

- Conclusion

In this paper, the following observations and proposal are given:

- Specifying how the 1 bit mobile-IAB cell type indication is used by UE, may have negative impact on legacy UE accessing mobile IAB.

- Option 2 may not be applicable to legacy UEs as it requires IAB-DU to broadcast some additional indication.

- Assistance information for mobile IAB cell search is not very useful in practical network, as UE may rarely use such information.

And

- No need to Introduce that stationary network broadcasts indication of supporting mobile-IAB camping. The iab-support is sufficient to allow mobile IAB camping on an IAB-capable cell

- Rel-16/Rel-17 IAB procedure with additionally mobile IAB cell reselection or redirection is sufficient to guarantee mobile IAB connection to network, without network broadcasting indication of supporting mobile-IAB

- The usage of the 1 bit mobile-IAB cell type indication by Rel-18 UE is left to implementation

- Solution Option 2, i.e., “The RRC Reconfiguration messages are sent to the UEs, where they are withheld until a condition has been met, which may be based on a broadcast by the logical source IAB-DU.” is deprioritized as it may not work for legacy UEs.

- For simplicity Solution Option 1 is considered as baseline.

- There is no need for any assistance information for UE search and measure for mIAB cells on different frequencies.

- References