3GPP TSG-RAN WG2 Meeting #121-bis electronic							   R2-2303350

E-Meeting, April 17-26, 2023   				    					revision of R2-2301116

Source: 	Xiaomi

Title:  	Capability sharing issue for SRS Tx switching capability

Agenda Item:	7.17.4

Document for:	Discussion and Decision

# Introduction

According to the WID [1] of the Rel-18 MUSIM, RAN2 needs to find solutions to for UE capability restriction issue of multiple SIMs:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

According to the discussion in the RAN2#119bis-e meeting, RAN2 made the following agreement:

- RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class.

In this contribution, we provide the capability sharing issue related to SRS Tx switching capability (e.g. srs-TxSwitch)

# Discussion

## UE capability restriction for srs-TxSwitch

The detailed capability descriptions for srs-TxSwitch from 38.306 are quoted as follows:

| srs-TxSwitch, srs-TxSwitch-v1610 Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE, which is mandatory with capability signaling. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas. supportedSRS-TxPortSwitch-v1610, which is optional to report, indicates downgrading configuration of SRS Tx port switching pattern. If the UE indicates the support of downgrading configuration of SRS Tx port switching pattern using supportedSRS-TxPortSwitch-v1610, the UE shall report the values for this as below, based on what is reported in supportedSRS-TxPortSwitch. supportedSRS-TxPortSwitch; supportedSRS-TxPortSwitch-v1610. t1r2; t1r1-t1r2. t1r4; t1r1-t1r2-t1r4. t2r4; t1r1-t1r2-t2r2-t2r4. t2r2; t1r1-t2r2. t4r4; t1r1-t2r2-t4r4. t1r4-t2r4; t1r1-t1r2-t2r2-t1r4-t2r4. - txSwitchImpactToRx indicates the entry number of the first-listed band with UL (see NOTE) in the band combination that affects this DL, which is mandatory with capability signaling; - txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL (see NOTE) in the band combination that switches together with this UL, which is mandatory with capability signaling. For txSwitchImpactToRx and txSwitchWithAnotherBand, value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. The entry number is the band entry number in a band combination. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities. NOTE: The first-listed band with UL includes a band associated with FeatureSetUplinkId set to 0 corresponding to the support of SRS-SwitchingTimeNR. | BC | FD | N/A | N/A |
| --- | --- | --- | --- | --- |

The details on how the capability of srs-TxSwitch is used can be found in the section 6.2.1.2 of 38.214. Some signalling procedures for “UE sounding procedure for DL CSI acquisition” are quoted from 38.214 as follows:

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r6' for 1T6R, 't1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R): …

Figure 1: Capability sharing for srs-TxSwitch for two SIMs

However, when SIM-A is configured with SRS-ResourceSet with usage field set as 'antennaSwitching', some antenna port could be occupied by SIM-B due to various use cases (e.g. SIM-B could also be configured with SRS-ResourceSet with 'antennaSwitching', or SIM-B could have the transmission/reception from/to one or more antenna ports in other carriers of the same band combination.). As the gNB serving SIM-A does not know the capability restriction at SIM-A for srs-TxSwitch, the gNB would still use the full capability of srs-TxSwitch (e.g. t1r4) while configuring the SRS transmission. The example of the capability sharing between two SIMs are illustrated in the above Figure. If the network configures t1r4 for SRS switching for SIM-A, and the UE only supports 1t2r for SIM-A due to the case that the two antennas are occupied by SIM-B, the DL CSI acquired from the SRS reception by the gNB serving SIM-A will be not correct. Then this will cause the DL throughput reduction due to the wrong DL CSI acquisition, which could cause more packet loss and retransmission.

Observation 1: The srs-TxSwitch capability from SIM-A can be changed temporarily due to the capability sharing of SIM-B.

## Field log for capability sharing for srs-TxSwitch for two SIMs

The field logging environment is given as follows:

The field logging environment: Logging the average DL throughput (within 3 minutes) for 1t2R and 1t4R separately for SIM-A for the same UE equipped with two SIMs both in RRC_CONECTED. Logging the average DL throughput when the UE equipped with two SIMs both in RRC_CONECTED reports 1t4R for SIM-A. Logging the average DL throughput after the same UE with two SIMs both in RRC_CONECTED changes its capability reporting to 1t2r via detach + attach. SIM-A is in full-buffer data transmission. SIM-B is in voice call. Band combination: N78+N78 Number of PRB: 217 MCS: [5, 29] dynamic

The following tables are the DL throughput logged for SIM-A for 1t2R and 1t4R separately.

|  | Average DL throughput for 1t4r (Mbps) | Average DL throughput for 1t2r (Mbps) | Throughput increase from 1t4r to 1t2r |
| --- | --- | --- | --- |
| N78+N78 | 147 | 183 | 24.5% |

Observation 2: Due to the wrong channel estimation for srs-TxSwitch capability between 1t4r and 1t2r, the DL throughput could be reduced by 24.5%.

Different from the UE capability of MIMO-layer (e.g. maxNumberMIMO-LayersPDSCH), multiple RF chains (or multiple antenna ports as used for SRS switching) could be mapped to one layer transmission. Then the change of the UE capability of srs-TxSwitch due to the reduction of the number of RF chains (or antenna ports) does not require the change of the UE capability for MIMO-layer.

Thus, we consider that the UE should be able to report its temporary srs-TxSwitch for SIM-A, due to the antenna capability shared by SIM-B.

Proposal: The UE should be able to indicate its temporary srs-TxSwitch capability.

# Conclusions

According to the analysis given above, we have the following observations and proposal:

Observation 1: The srs-TxSwitch capability from SIM-A can be changed temporarily due to the capability sharing of SIM-B.

Observation 2: Due to the wrong channel estimation for srs-TxSwitch capability between 1t4r and 1t2r, the DL throughput could be reduced by 24.5%.

Proposal: The UE should be able to indicate its temporary srs-TxSwitch capability.

# Reference

[1] RP-220955, vivo, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”

[2] 3GPP TS 38.306, “NR; User Equipment (UE) radio access capabilities”