3GPP TSG-RAN WG2 Meeting # 121-bis - Electronic	R2-2303411

Online, April 17 – April 26, 2023

Agenda Item:	[7.17.4] MUSIM Gap Priorities and other RAN4 impacts

Source: 	Apple Inc

Title:  	Views on RAN4 LS for MUSIM gap priorities

Document for:	Discussion and Decision

# Introduction

RAN4 has sent a LS to RAN2 in [1]

RAN4 has discussed and achieved the following agreements on priority for MUSIM gaps: - Introduction of priorities for periodic MUSIM gaps - Each periodic MUSIM gap can be assigned with a different priority - The priority level of MUSIM gap(s) shall be configured to be comparable to priority level of NW A’s Type-2 MGs - MUSIM gap and Type-2 MG cannot be configured with the same priority - The priority level of MUSIM gaps should be configured/allocated by NW A RAN4 further agrees that: - When requesting periodic MUSIM gap(s) UE can provide an assistance information for gap priority selection - UE can optionally indicate its preferred priority for all or a subset MUSIM gaps - It is up to NW A on how to use this information Definition of Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 RAN4 is still discussing whether priority for aperiodic MUSIM gap needs to be introduced.

In this contribution we would like to present our view on this LS.

# Discussion

From the LS the following observations are clear

Observation 1: Each periodic gap can be assigned with a different priority

Observation 2: Priority for periodic gaps are allocated by NW, based on optional UE assistance information. It is up to the NW how to use this assistance information

From the above observations it is not clear,

- On what basis UE is expected to request for priority for a given set(s) of periodic gaps.

- How the UE expected to use these priorities for the periodic gaps once assigned.

- Will there be an out of sync between the priority requested by the UE compared to the value assigned by the NW (i.e.) whether that is a possible scenario.

Observation 3: Some open questions exist with respect to handling of priority for periodic gaps.

Proposal 1: RAN2 to discuss the open questions and potentially draft a reply LS to RAN4 requesting for clarifications thereof.

On the issue of aperiodic gaps, the following observation is made from the LS

Observation 5: RAN4 is still discussing whether priority for aperiodic MUSIM gap needs to be introduced.

From our understanding, aperiodic gaps are one-time gaps. This is used by UE to request for a pre-determined one-time operation on the other SIM instance and should be treated at the highest priority. If this does not get handled, then the UE must keep requesting for aperiodic gaps, until it gets to use that requested aperiodic gap. Such repeated request leads to unnecessary signalling overload, and potentially impacting the protocol use case on the other SIM instance. So it is better if the aperiodic gaps have implicitly the highest priority.

Observation 5: Aperiodic gaps are one-time gaps that are time sensitive

Proposal 2: Clarify if there is any need to assign priorities for aperiodic gaps in the first place.

# Conclusion

In summary, we have the following observations and proposals

Observation 1: Each periodic gap can be assigned with a different priority

Observation 2: Priority for periodic gaps are allocated by NW, based on optional UE assistance information. It is up to the NW how to use this assistance information

Observation 3: Some open questions exist with respect to handling of priority for periodic gaps.

Observation 4: RAN4 is still discussing whether priority for aperiodic MUSIM gap needs to be introduced.

Observation 5: Aperiodic gaps are one-time gaps that are time sensitive

Proposal 1: RAN2 to discuss the open questions and potentially draft a reply LS to RAN4 requesting for clarifications thereof.

Proposal 2: Clarify if there is any need to assign priorities for aperiodic gaps in the first place.

# Reference

[1]    R4-2303249 - LS on priority for MUSIM gaps     Vivo