3GPP TSG-RAN WG2 #121-bis-e	R2-2303705

Online April 17th – 26th, 2023

Agenda Item:	7.2.3

Source:	Ericsson

Title:	On RAT-dependent positioning Integrity

Document for:	Discussion, Decision

# 1	Introduction

The following objective was agreed for the work of integrity support for RAT dependent positioning[1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

Integrity for RAT independent positioning has been supported by 3GPP since release 17, specifically for GNSS / RTK.  In this paper, we give our view on measurement error sources, LMF-based RAT dependent integrity and support for UE-based RAT dependent integrity.

# 2	Discussion

## 2.1 On integrity error sources

In TR 38.859[2], the identified error sources and candidates for distributions to model the errors was captured in Table 6.1.1-1 and Table 6.1.1-2.

Table 6.1.1-1: Error sources for LMF-based and UE-based positioning integrity modes

| Positioning Integrity Mode | DL TDOA | UL TDOA | Multi-RTT | UL AoA | DL AoD |
| --- | --- | --- | --- | --- | --- |
| LMF-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - RSTD measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - RTOA measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement - TRP location | - Angle of arrival measurement - TRP location - ARP location (e.g., ARPLocationInformation in TS 38.455 [17]) | - TRP location - DL-PRS RSRPP of the first path or RSRP |
| UE-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355 [16]) |  |  |  | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) |

Table 6.1.1-2: Identified candidates for distributions to model the errors due to different error sources

| Error source | Candidate(s) for distribution for error source |
| --- | --- |
| Timing measurement errors (NOTE 1, 2, 3) | Gaussian distribution |
| Inter-TRP synchronization errors | - Uniform distribution (NOTE 4) - Gaussian distribution |
| TRP location error (e.g., NR-TRP-LocationInfo in [16]) | - Uniform distribution (NOTE 5) - Gaussian distribution |
| TRP location error (e.g., Geographical coordinates in [17]) | - Uniform distribution - Gaussian distribution |
| ARP location error (e.g., ARPLocationInformation in [17]) | - Uniform distribution - Gaussian distribution |
| NOTE 1: Timing measurement errors are applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurements. NOTE 2: It is assumed that the timing measurement error is associated with the first path. NOTE 3: It is assumed that the timing measurement error contains TEG related TX/RX timing error if the TEG related information is provided NOTE 4: This may already be consistent with the uncertainty related to NR-RTD-Info in [16]. NOTE 5: This may already be consistent with the uncertainty related to NR-TRP-LocationInfo in [16]. |  |

The RAT dependent positioning error sources are thus in the following categories

- Measurement error sources (RSTD, UE Rx-Tx Time Difference, AoA DL-PRS RSRP/RSRPP, RTOA, gNB Rx-Tx Time Difference) and parts thereof,

- TRP Location Information (NR-TRP-LocationInfo);

- TRP Time Synchronization Information (NR-RTD-Info);

- Beam Bore-Sight Direction (NR-DL-PRS-BeamInfo);

- Beam Antenna Information (NR-TRP-BeamAntennaInfo);

These error sources are discussed in the following subsections

### 2.1.1	Measurement error sources

Measurements to be reported are typically performed during short time windows during which it is difficult to establish extensive statistics. Therefore, the measuring entity (UE or TRP) needs to establish error statistics based on measurements also outside such a time window. The measurements can be directly outside the time window, periodically, part of background measurement process or part of a calibration procedure. It seems not realistic that a UE or TRP would be able to establish sufficient statistics to assess measurement error statistics and bounds only based on a snapshot measurement. More measurements and calibration are needed. Exactly how statistics and bounds are established is up to implementation.

Complete UE/TRP measurement error statistics cannot be derived only from measurements confined to a short time window.

UE/TRP needs to establish measurement error statistics and bounds from extensive measurements and calibration efforts

It is up to implementation how the UE/TRP establishes measurement error statistics and bounds, where a UE/TRP can be expected to measure outside the response time window in order to establish sufficient statistics.

RAN2 can focus on defining the structure for relevant statistics and bounds that facilitate integrity calculations. Such structure needs to be based on a signal model of measurements and measurement error statistics and bounds that represents errors that are natural to handle in integrity determination, typically lumping multiple error contributions together for convenience and clarity.

In Rel 17, the NR timing measurements for RAT-dependent positioning were complemented with the notion of timing error groups (TEG). Such timing measurements were by supporting UE/TRPs associated to a TEG ID, where measurement associated to different TEG IDs can be subject to a timing offset. The device can also provide a timing error margin as an upper bound for the timing measurement offsets across all TEGs within one SignalMeasurementInformation.

For DL-TDOA, the RSTD reference TRP is associated to a timing quality attribute, representing the RSTD reference TRP timing measurement error. Furthermore, each neighbor TRP RSTD measurement is associated to a timing quality attribute. This timing quality can represent either only A or A combined with B, where

- the neighbor TRP timing measurement error

- any TEG timing offset between

  - the UE Rx TEG of the TRP reference timing measurement, and

  - the UE Rx TEG of the neighbor TRP timing measurement

The same applies to the DL-TDOA additional measurements, where the timing quality attribute of the RSTD difference can combine both the timing measurement error as well as any UE Rx TEG offsets or represent only the actual timing measurement error. From the NOTE 3 in Table 6.1.1-2 in [2], RAN1 makes the assumption that the timing quality attribute represents the combination of the two, which means that we have the following two observations

The RSTD timing quality for neighbor TRP combines a timing measurement error and a UE Rx TEG offset between RSTD reference TRP timing measurement and neighbor TRP timing measurements

The DL TDOA additional measurement timing quality combines a timing measurement error and a UE Rx TEG offset between RSTD PRS resource and additional PRS resource timing measurements

Therefore, any timing measurement error bounds would also be defined for the combination of timing measurement error and UE Rx TEG offset combined.

DL TDOA timing quality and bounds refers to the combination of timing measurement error and any UE Rx TEG offset

Similar to the DL TDOA timing measurement errors, the Multi RTT timing measurements also can be subject to offsets between UE Rx TEGs, UE Tx TEGs, and UE RxTx TEGs depending on what is defined. The RAN1 assumption for Multi RTT is that the timing quality represents the combination of an RxTx timing measurement error and any Rx/Tx/RxTx TEG offsets.The same assumption concerns also the UE RxTx additional measurements.

The UE RxTx difference timing quality combines a timing measurement error and any UE Rx/Tx/RxTx TEG offsets

The UE RxTx difference additional measurement timing quality combines a timing measurement error and a UE Rx TEG offsets

Therefore, any Multi RTT timing measurement error bounds would also be defined for the combination of timing measurement error and UE Rx/Tx/RxTx TEG offsets combined.

Multi RTT timing quality and bounds refers to the combination of timing measurement error and any UE Rx/Tx/RxTx TEG offsets

### 2.1.2	TRP and ARP location errors

The TRP and ARP location errors can be represented by either a uniform or a Gaussian distribution according to [2]. However, it is only a Gaussian over-bounding that facilitates the integrity calculations

Represent the TRP and ARP location errors by a Gaussian over-bounding or paired over-bounding.

### 2.1.3	RTD errors

The relative time difference error between two different DL PRS resources can represent either A or the combination of A and B, where

- is the relative time difference between DL PRS resources

- TRP Tx TEG offsets between the TRP Tx TEG of the DL PRS resources

Similar to the measurement error discussion, it is natural to consider that a relative time difference error bound represents the combination of A and B.

The error bound of the relative timing difference between two DL PRS resources combines the relative time difference error and any TRP Tx TEG offsets

The relative time difference error distribution can be represented by either a uniform or a Gaussian distribution according to [2]. However, it is only a Gaussian over-bounding that facilitates the integrity calculations

Represent the RTD errors by a Gaussian over-bounding or paired over-bounding.

### 2.1.4	Beam Bore-Sight Direction and Beam Antenna Information

Both these are listed as FFS by RAN1 in the TR [2] since no consensus was reached. However, it is natural that errors in the antenna and beam directions will directly impact the performance and thus any antenna/beam bore sight direction errors should be bounded to support integrity assessments

Add antenna/beam direction error sources to the set of considered error sources and define corresponding error bounds

## 2.2

Measurements to be reported are typically performed during short time windows during which it is difficult

Measurements to be reported are typically performed during short time windows during which it is difficult

## 2.3	Support for UE-based integrity

As discussed in Section 2.1.1, integrity calculations considering measurement error characteristics benefits from regular and frequent measurements to assess measurement error statistics and bounds. This is most naturally the case for UE-based RAT dependent positioning, where the UE more of less continuously monitors the measurements. The integrity calculations based on a target integrity risk (TIR) to derive a protection level (PL) can therefore be monitored frequently. The main beneficiary of the information about the UE based RAT dependent positioning availability is typically higher layers in the UE. Examples include autonomous guided vehicles (AGV) etc. In such applications, the UE will not only assess the PL but also compare to an alert limit (AL) in consideration of  a time to alert (TTA) to make the availability or non-availability declaration to the UE higher layers which uses the UE-based position estimates provided that they are considered reliable and available. The UE will therefore need a full set of TIR, AL and TTA in order to make such assessments.

For key use cases of UE-based RAT dependent positioning the UE needs TIR, AL and TTA to make integrity calculations and be able to determine reliability and availability of the positioning estimates to UE higher layers

The discussions at RAN2#120 led to an agreement [4] to send an LS [3] to SA2 about integrity parameters that can be configured by the LCS client/UE/AF:

RAN2 would like to provide the following answer to SA2's question on the parameters that are needed: - LCS client/UE/AF sends TIR, AL, TTA to the LMF - LMF returns the system available/unavailable indication to the LCS client/UE/AF

This agreement now implies that LMF can be configured with the integrity parameters TIR, AL and TTA by an application function. LMF can also be configured with these parameters via OAM.

LMF can be configured with the integrity parameters TIR, AL and TTA.

If a UE is not pre-configured or provided from UE higher layers with TIR, AL and TTA, then the UE would need to be provided with these parameters in order to properly assess integrity – reliability and availability of the UE-based positioning estimates. It is therefore natural to let the UE request these parameters as part of assistance data in a similar way as it requests other assistance data for UE-based positioning such as the IE NR-PositionCalculationAssistance.

UE can request the integrity parameters TIR, AL and TTA as part of assistance data in a similar way as it requests for positioning calculation assistance from LMF.

Based on the discussion, we have the following proposal

Add TIR, AL and TTA to the integrity assistance data that the UE can request for on a need basis to support UE-based integrity calculations

Agree to the text proposal in Annex A.

## 2.3	LMF-based positioning integrity signalling

For LMF-based positioning integrity mode, the identified error sources from UE are all associated with UE’s measurements, thus it would be natural to support LMF requesting UE to send error source statistics of error source using LPP RequestLocationInformation message and UE reporting the error source result to LMF in LPP in ProvideInformation message for each positioning method.

For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects and agree to the text proposal as in Annex:

- UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

- LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

- UE performs positioning measurements and computes the error (same as legacy)

- UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

- LMF computes the Integrity.

### 2.2.2 LMF requests UE to send error source model statistics

For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source and such request can be sent in the RequestLocationInformation for each RAT positioning method.

For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source in the RequestLocationInformation for each RAT positioning method.

Agree to the LPP text proposal in Annex B.1

### 2.2.3 UE send error source model statistics

For LMF-based positioning integrity mode, the timing measurement error sources include RSTD measurement in DL-TDOA and UE Rx-Tx time difference measurement Multi-RTT. In TR 38.859, it is concluded that the timing measurement error can be modeled as Gaussian/Normal distribution. UE may send the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

For LMF-based positioning integrity mode, UE sends the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

Agree to the LPP text proposal in Annex B.2

# Conclusion

In the previous sections we made the following observations:

Observation 1	Complete UE/TRP measurement error statistics cannot be derived only from measurements confined to a short time window.

Observation 2	UE/TRP needs to establish measurement error statistics and bounds from extensive measurements and calibration efforts

Observation 3	The RSTD timing quality for neighbor TRP combines a timing measurement error and a UE Rx TEG offset between RSTD reference TRP timing measurement and neighbor TRP timing measurements

Observation 4	The DL TDOA additional measurement timing quality combines a timing measurement error and a UE Rx TEG offset between RSTD PRS resource and additional PRS resource timing measurements

Observation 5	The UE RxTx difference timing quality combines a timing measurement error and any UE Rx/Tx/RxTx TEG offsets

Observation 6	The UE RxTx difference additional measurement timing quality combines a timing measurement error and a UE Rx TEG offsets

Observation 7	For key use cases of UE-based RAT dependent positioning the UE needs TIR, AL and TTA to make integrity calculations and be able to determine reliability and availability of the positioning estimates to UE higher layers

Observation 8	LMF can be configured with the integrity parameters TIR, AL and TTA.

Observation 9	UE can request the integrity parameters TIR, AL and TTA as part of assistance data in a similar way as it requests for positioning calculation assistance from LMF.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It is up to implementation how the UE/TRP establishes measurement error statistics and bounds, where a UE/TRP can be expected to measure outside the response time window in order to establish sufficient statistics.

Proposal 2	DL TDOA timing quality and bounds refers to the combination of timing measurement error and any UE Rx TEG offset

Proposal 3	Multi RTT timing quality and bounds refers to the combination of timing measurement error and any UE Rx/Tx/RxTx TEG offsets

Proposal 4	Represent the TRP and ARP location errors by a Gaussian over-bounding or paired over-bounding.

Proposal 5	The error bound of the relative timing difference between two DL PRS resources combines the relative time difference error and any TRP Tx TEG offsets

Proposal 6	Represent the RTD errors by a Gaussian over-bounding or paired over-bounding.

Proposal 7	Add antenna/beam direction error sources to the set of considered error sources and define corresponding error bounds

Proposal 8	Add TIR, AL and TTA to the integrity assistance data that the UE can request for on a need basis to support UE-based integrity calculations

Proposal 9	Agree to the text proposal in Annex A.

Proposal 10	For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects and agree to the text proposal as in Annex:

	UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

	LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

	UE performs positioning measurements and computes the error (same as legacy)

	UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

	LMF computes the Integrity.

Proposal 11	For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source in the RequestLocationInformation for each RAT positioning method.

Proposal 12	Agree to the LPP text proposal in Annex B.1

Proposal 13	For LMF-based positioning integrity mode, UE sends the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

Proposal 14	Agree to the LPP text proposal in Annex B.2

# References

RP-223549, “New WID on Expanded and Improved NR Positioning”, December 12 - 16, 2022

TR 38.859, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on expanded and improved NR positioning;(Release 18)”, v18.0.0, December 2022

R2-2213320, “Reply LS on GNSS integrity requirement provisioning”, Huawei, Nov 2022

RAN2#120 Chairman’s notes

# Annex A – Text Proposal to 37.355

### 6.4.1	Common Lower-Level IEs

[…]

#### –	HorizontalWithVerticalVelocityAndUncertainty

The IE HorizontalWithVerticalVelocityAndUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255),

horizontalUncertaintySpeed	INTEGER(0..255),

verticalUncertaintySpeed	INTEGER(0..255)

}

-- ASN1STOP

#### –	IntegrityRequirementsSup

The IE IntegrityRequirementsSup is used to indicate support of position estimate reliability determination based on integrity requirements attributes provided in the assistance data.

-- ASN1START

IntegrityRequirementsSup-r17::= SEQUENCE {

...

}

-- ASN1STOP

### 6.4.2	Common Positioning

[…]

#### –	CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]] ,

[[

integrityRequirementsSup-r17	IntegrityRequirementsSup-r17	OPTIONAL	-- Cond IntReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| IntReq | This field is mandatory present if the target device supports IntegrityRequirements; otherwise it is not present. |

CommonIEsProvideCapabilities field descriptions

segmentationInfo This field indicates whether this ProvideCapabilities message is one of many segments, as specified in clause 4.3.5.

lpp-message-segmentation This field, if present, indicates the target device's LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the target device supports receiving segmented LPP messages; if bit 0 is set to value 0 it indicates that the target device does not support receiving segmented LPP messages. If bit 1 is set to value 1, it indicates that the target device supports sending segmented LPP messages; if bit 1 is set to value 0 it indicates that the target device does not support sending segmented LPP messages.

integrityRequirementsSup This field specifies that position estimate reliability determination based on provided integrity requirements is supported by the target device.

#### –	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]] ,

[[

integrityRequirementsReq-r17	NULL					OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PerADreq | The field is mandatory present if the target device requests periodic assistance data delivery. Otherwise it is not present. |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |
| IntReqReq | This field is mandatory present if the target device requests integrity requirements for position estimate reliability determination; otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

segmentationInfo This field indicates whether this RequestAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceDataReq This field indicates a request for periodic assistance data delivery, as specified in clause 5.2.1a.

integrityRequirementsReq This field, if present, indicates a request for integrity requirements..

#### –	CommonIEsProvideAssistanceData

The CommonIEsProvideAssistanceData carries common IEs for a Provide Assistance Data LPP message Type.

-- ASN1START

CommonIEsProvideAssistanceData ::= SEQUENCE {

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

periodicAssistanceData-r15	PeriodicAssistanceDataControlParameters-r15

OPTIONAL	-- Cond PerAD

]] ,

[[

integrityRequirements-r17	IntegrityRequirements-r17	OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerAD | The field is mandatory present in a periodic assistance data delivery session. Otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

segmentationInfo This field indicates whether this ProvideAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceData This field indicates a periodic assistance data delivery, as specified in clauses 5.2.1a and 5.2.2a.

integrityRequirements This field provides integrity requirements which the device shall use for UE-based position estimate reliability deterimation if the device has indicated support via IntegrityRequirementsSup or requested the integrity requirements via integrityRequirementsReq.

[…]

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	DL-PRS-ID-Info

The IE DL-PRS-ID-Info provides the IDs of the reference TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-ID-Info-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-DL-PRS-ResourceID-List-r16	SEQUENCE (SIZE (1..nrMaxResourceIDs-r16)) OF

NR-DL-PRS-ResourceID-r16

OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16

OPTIONAL  -- Need ON

}

-- ASN1STOP

DL-PRS-ID-Info field descriptions

nr-DL-PRS-ResourceID-List This field provides a list of DL-PRS Resource IDs under the same DL-PRS Resource Set.

#### –	IntegrityRequirements

The IE IntegrityRequirements provides the requirement parameters the device needs to assess positioning integrity

-- ASN1START

IntegrityRequirements-r17 ::= SEQUENCE {

targetIntegrityRisk-r17     		INTEGER (10..90),

horizontalAlertLimit-r17			INTEGER (0..50000),

verticalAlertLimit-r17				INTEGER (0..50000)				OPTIONAL, -- Need ON

timeToAlert-r17			        	INTEGER (1,2000)				OPTIONAL, -- Need ON

...

}

-- ASN1STOP

IntegrityRequirements field descriptions

targetIntegrityRisk This field indicates the Target Integrity Risk (TIR) for the integrity principle of operation by the device. The TIR is calculated by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

horizontalAlertLimit This field indicates the horizontal alert limit for the integrity principle of operation by the device along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres.

verticalAlertLimit This field indicates the vertical alert limit for the integrity principle of operation by the device. Scale factor 0.01 metre; range 0 – 500 metres. To be compared to the horizontal protection level determined by the device.

timeToAlert The maximum allowable elapsed time from when the protection level (PL) exceeds the Alert Limit (AL) until the function providing positioning integrity annunciates a corresponding alert. Scale factor 0.1 second.

# Annex B – Text proposals to 37.355

## B.1 For RequestLocationInformation for each RAT positioning method.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

[[

nr-RSTD-Error-Request-r18					ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested } 		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-RSTD-Error-Request This field indicates whether the target device is requested to report error source statistics of the RSTD measurements.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1) } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17

ENUMERATED { requested, ... }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL, -- Need ON

multiMeasInSameReport-r17			ENUMERATED { requested }			OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

[[

nr-UE-RxTxTimeDiff-Error-Request-r18 ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 	INTEGER (0..5)					OPTIONAL  -- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiff-Error-Request This field indicates whether the target device is requested to report error source statistics of the UE Rx-Tx time difference measurements.

-- ASN1START

NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-AoD-ReportConfig-r16					NR-DL-AoD-ReportConfig-r16,

...,

[[

multiMeasInSameReport-r17					ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

[[

nr-DL-PRS-RSRPP-Error-Request-r18 			ENUMERATED { requested }	OPTIONAL  -- Need ON

nr-DL-PRS-RSRP-Error-Request-r18 			ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

}

NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16		INTEGER (1..8)				OPTIONAL, -- Need ON

...,

[[

maxDL-PRS-RSRP-MeasurementsPerTRP-r17		INTEGER (9..24)				OPTIONAL, -- Need ON

maxDL-PRS-RSRPP-MeasurementsPerTRP-r17		INTEGER (1..24)				OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17			SEQUENCE {

type-r17	LOS-NLOS-IndicatorType1-r17,

granularity-r17

LOS-NLOS-IndicatorGranularity1-r17,

...

}							OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }

OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

nr-DL-PRS-RSRPP-Error-Request This field indicates whether the target device is requested to report error source statistics of the DL-PRS RSRPP of the first path.

nr-DL-PRS-RSRP-Error-Request This field indicates whether the target device is requested to report error source statistics of the DL-PRS RSRP.

## B.2 Error source statistics in the SignalMeasurementInformation message for the corresponding positioning method

#### –	nr-Timing-Error-Source

The IE nr-Timing-Error-Source characterizes the mean and standard deviation of a normal distribution.

-- ASN1START

nr-Timing-Error-Source-r18 ::= SEQUENCE {

mean 							INTEGER(FFS)

Standard-deviation      		INTEGER(FFS)

}}

-- ASN1STOP

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]]

[[

nr-RSTD-Error-Source-r18			nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]]

[[

nr-RSTD-ResultDiff-Error-Source-r18 nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-RSTD-Error-Source This field specifies the mean and standard deviation of the nr-RSTD for RAT-dependent positioning integrity.

nr-RSTD-ResultDiff-Error-Source This field specifies the mean and standard deviation of the nr-RSTD-ResultDiff for RAT-dependent positioning integrity.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]]

[[

nr-UE-RxTxTimeDiff-Error-Source-r18	nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16 				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]]

[[

nr-UE-RxTxTimeDiffAdditional-Error-Source-r18	nr-Timing-Error-Source-r18	OPTIONAL,

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-UE-RxTxTimeDiff-Error-Source This field specifies the mean and standard deviation of the nr-UE-RxTxTimeDiff for RAT-dependent positioning integrity.

nr-UE-RxTxTimeDiffAdditional-Error-Source This field specifies the mean and standard deviation of the nr-UE-RxTxTimeDiffAdditional for RAT-dependent positioning integrity.