3GPP RAN WG2 Meeting #121bis-e	R2-2303731

eMeeting April 17th – 26th, 2023

Agenda Item:	7.8.3

Source:	InterDigital

Title:	Flight path notification and reporting for UAV

Document for:	Discussion, Decision

# Introduction

A new work item on NR support for aerial UEs (UAVs) was agreed for Rel-18. Since aerial UEs are supported via LTE as from Rel-15, work done in LTE is a starting point and NR solutions should be harmonized as much as possible. Among detailed objectives for Rel-18 is the introduction of flight path reporting to NR.

RAN2#119e has captured the following relevant agreements [2]:

- As in LTE, flight path plan reporting will be introduced.  Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report.  FFS if timestamp is mandatory or optional for NR.  FFS if further enhancements are needed

RAN2#119bis-e has captured the following [3]:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

RAN2#120 has captured the following [4]:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS.

And RAN2#121 has further captured the following [5]:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Furthermore, email discussion [POST121][314][UAV] Flight path reporting [6] was triggered after RAN2#121 to continue discussion on flight path reporting aspects like update notification triggering and support for delta signaling. This document addresses remaining aspects for UAVs, specifically the content and procedure for updated flight path notification and reporting.

# Flight path reporting for UAV

Recent agreements regarding the contents of a UAV flight path and reporting procedure largely align with LTE. One exception is support for updating a previously reported flight path, which is a new enhancement unique to NR. Summarizing agreements from past meetings, once an update to a previously reported flight path is available, the UE can notify the network via the UE Assistance Information (UAI) message. An updated flight path is retrieved via the UE information request/response procedure like the initial flight path. Support and definition of trigger conditions for flight path update are currently FFS in RAN2 and are under discussion in [6].

## Flight path update notification assistance information.

Based on current agreements, UE indicates to the network a new flight path is available in the UE. However, what constitutes a “new” flight path can vary significantly. For example, a new flight path could be a modification to a previously reported value or an entirely new value. It could be one change or a change to the entire flight path, and could impact only time information, location information, or both. How the flight path changes can affect resource provisioning and mobility planning or have no impact at all. A simple flag is insufficient to differentiate between these cases.

Observation 1:	How and to what degree a flight path has changed can vary significantly and can impact how the update notification is treated by the network.

Instead, the UE can include additional assistance information to support NW prioritization. Whether or not to include additional assistance information could be configured by the network (e.g., within the flight path reporting configuration), as well as what type of information to include.

Proposal 1:	NW can configure UE to report assistance information about the flight path update.

For example, an extra waypoint appended to the end of the existing flight path may be considered with lower priority than a significant course correction to a previously reported flight path. A UE may optionally indicate whether the update is due to modification of an existing value or availability of new information in the update notification.

Proposal 2:	Flight path update assistance information can include the update cause (i.e., a modification to previously reported information or the availability of new flight path information).

Similarly, a large modification to a previously reported flight path would likely be treated with higher priority than a minor change. A UE may optionally indicate the number of changed and/or new waypoint/timestamps in the update notification.

Proposal 3:	Flight path update assistance information can include the number of updated waypoints/ timestamps available.

A UE arriving late to a waypoint may be treated differently (e.g., with lower priority) than arriving at an entirely different location. If a UE is configured to include timestamp information, a UE may optionally indicate that only time information has changed in an update notification.

Proposal 4:	Flight path update assistance information can include if only time information has changed.

## Flight path update notification triggering

One reason flight path reporting is under network control is to minimize possible interference caused by frequent and unnecessary flight path updates. A UE sending frequent update notifications (and thus triggering a flight path reporting procedure) over minor changes is contrary to the intention of the current procedure.

Observation 2:	Update notifications triggered by minor changes can cause unnecessary flight path reporting, defeating the purpose of having flight path reporting under NW control.

Unnecessary flight path updates can be controlled by limiting when a notification is triggered. For example, a waypoint may only be considered changed if the updated waypoint deviates greater that a distance threshold. A similar condition can also be defined for timestamps. If the network wishes to be notified of any deviation from a previous flight path report, a threshold can be configured to zero.

Proposal 5:	A waypoint/timestamp must change by a configured threshold to trigger a flight path update notification.

The network may similarly restrict a flight path update notification to only being triggered if a certain number of waypoints and/or timestamps are considered changed or invalid. This may be configured independently or jointly with the above distance/time thresholds.

Proposal 6:	NW can configure the number of updated waypoint/timestamps needed to trigger a flight path update notification.

## Reporting an updated flight path

When a flight path is updated/changed, it is likely only a portion of the overall flight path is impacted. For example, the UAV may encounter a delay (e.g., turbulence, collision avoidance etc.), where the waypoints remain the same and only the timestamp information has changed. In another example, the existing flight path may remain valid and one or more additional waypoints may be known to the UE and can be appended to the previously reported flight path.

If only a subset of a previously reported flight path has changed (which is likely the case), resending the entire flight path is inefficient from a signalling perspective and can introduce interference.

Observation 3:	Sending an entire flight path report is inefficient if only a subset of the previously reported information has changed.

A UE may instead indicate one or more waypoints/timestamps (e.g., the ones which have been updated, added, or removed) without sending a full flight path.  Another option to further reduce signalling overhead is to report only the delta change from a previous flight path report. However, additional considerations may be needed to ensure successful reception of a previously reported flight path to avoid the UE and network becoming out of sync and subsequent error propagation.

Proposal 7:	UE can report one or more waypoints/timestamps which have been updated, added, or removed, without a sending full flight path.

Whether the UE reports full or partial flight path should ultimately be decided by the network (e.g., based on the assistance information provided in the update notification). The network may explicitly indicate whether to report a full or partial flight path (e.g., within the information request message). Alternatively, the ability to report partial flight path update may be enabled via configuration, where UE will by default only report the modified/new flight path information.

Proposal 8:	NW decides whether UE reports full or partial flight path. FFS if partial flight path reporting is enabled by configuration or explicit request.

# Conclusion

In this contribution the following proposals are made concerning updated flight path notification and reporting for aerial UEs in NR:

Observation 1:	How and to what degree a flight path has changed can vary significantly and can impact how the update notification is treated by the network.

Observation 2:	Update notifications triggered by minor changes can cause unnecessary flight path reporting, defeating the purpose of having flight path reporting under NW control.

Observation 3:	Sending an entire flight path report is inefficient if only a subset of the previously reported information has changed.

Proposal 1:	NW can configure UE to report assistance information about the flight path update.

Proposal 2:	Flight path update assistance information can include the update cause (i.e., a modification to previously reported information or the availability of new flight path information).

Proposal 3:	Flight path update assistance information can include the number of updated waypoints/ timestamps available.

Proposal 4:	Flight path update assistance information can include if only time information has changed.

Proposal 5:	A waypoint/timestamp must change by a configured threshold to trigger a flight path update notification.

Proposal 6:	NW can configure the number of updated waypoint/timestamps needed to trigger a flight path update notification.

Proposal 7:	UE can report one or more waypoints/timestamps which have been updated, added, or removed, without a sending full flight path.

Proposal 8:	NW decides whether UE reports full or partial flight path. FFS if partial flight path reporting is enabled by configuration or explicit request.

# References

RP-223545 – WI on NR support for UAV

R2-2209301 – RAN2#119e Meeting Report

R2-2211101 – RAN2#119bis-e Meeting Report

Draft_R2_120_meeting_report_v1

Draft_R2_121_meeting_report_v1

R2-230xxxx – [POST121][314][UAV] Flight path reporting (Intel)