3GPP TSG-RAN WG2 Meeting #121bis	R2-2303770

E-meeting, 17th -26th Apri.,2023

Agenda item:		6.1.3.1

Source:		Lenovo

Title:		Discussion on CHO with T346g in MUSIM

Document for:		Discussion and Decision

- Introduction

In this contribution, we discuss the association between CHO and timer T346g. For example, whether to perform CHO when T346g is running?

- Discussion

In RAN2#117-e meeting, RAN2 made the following agreement on UE behaviors while T346g is running:

RAN2 will not specify any new behaviour if the wait timer for switching notification to leave RRC connected state is running, and UE detects RLF, triggers re-establishment, receives HO command or triggers CHO. No specification changes are needed.

According to the above agreement, a UE can follow legacy behaves for initiation of re-establishment and handover. That means whether performing re-establishment and handover does not depend on on T346g.

Observation 1: In RAN2#117-e meeting, RAN2 agreed that a UE behaves as in legacy i.e. detection of RLF and initiation of re-establishment is not dependent on the T346g running condition.

Based on the discussion on RAN2#119bis meeting, RAN2 agreed to add the following NOTE in clause 5.3.7.2: “NOTE: It is up to UE implementation whether to initiate the procedure while T346g is running.”. The compromised agreement of adding a NOTE is used to address the concern as follows [1].

- UE is NOT allowed to abort on-going re-establishment procedure by itself for any reason. Entering RRC_IDLE state during re-establishment may only happen as a consequence of following re-establishment procedure as specified in TS 38.331.

- Current procedure text does NOT allow a UE to stop running timer T346g during re-establishment procedure.

- Leaving to UE implementation whether to stop running timer T346g during re-establishment may cause potential issues on network side i.e. RRC state mismatch problems or inability to identify whether to cease on-going re-establishment procedures due to T346g expiry.

Observation 2: RAN2 agreed to add the following NOTE: “It is up to UE implementation whether to initiate the procedure while T346g is running.”

Regarding the case of traditional HO command, the network can avoid transmitting HO command when T346g is running since the network is aware of that timer T346g is running. The reason is that UE starts timer T346g Upon transmitting UEAssistanceInformation message with musim-PreferredRRC-State.

Observation 3: The network can avoid transmitting HO command when T346g is running since the network is aware of that timer T346g is running.

The same situation as re-establishment happens on CHO case. It is possible that the execution condition for CHO is met when T346g is running. Then, UE shall perform CHO when T346g is running. Once the timer expires, UE goes into IDLE state. That means UE autonomously stops CHO procedure which is not aligned with current specification.

Observation 4: The same situation as re-establishment happens on CHO case.

Proposal 1: To add a NOTE: “It is up to UE implementation whether to execute the CHO procedure while T346g is running.”

[The corresponding CR sees R2-2303771]

- Conclusion

In this contribution, we have the following proposals:

Observation 1: In RAN2#117-e meeting, RAN2 agreed that a UE behaves as in legacy i.e. detection of RLF and initiation of re-establishment is not dependent on the T346g running condition.

Observation 2: RAN2 agreed to add the following NOTE: “It is up to UE implementation whether to initiate the procedure while T346g is running.”

Observation 3: The network can avoid transmitting HO command when T346g is running since the network is aware of that timer T346g is running.

Observation 4: The same situation as re-establishment happens on CHO case.

Proposal 1: To add a NOTE: “It is up to UE implementation whether to execute the CHO procedure while T346g is running.”

References

R2-2211770 Finalizing re-establishment procedure handling while T346g is running. Samsung etc.