3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303870

April 17-26, 2023

Agenda item:	7.4.4

Source:	Samsung

Title:	Considerations on CHO with CPA/CPC

Document for:	Discussion & Decision

Introduction

From RAN2#119e to RAN2#121, the follow agreements were made on CHO with CPA/CPC.

RAN2#119e

Agreements:

- Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17.

- CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported.

- FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

RAN2#120

Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation)

RAN2#121

RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18

The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above

In this contribution, we will further discuss some issues of CHO with CPA/CPC.

Discussion

On evaluation mode

Regarding the evaluation mode, whether CHO evaluation and CPC/CPA evaluation is simultaneous or sequential can depend on how the CPA/CPC candidate PSCells are prepared and how the mapping relation between candidate PCells and PSCells is determined.

In one case, CPAC can be determined by the candidate MN, and the candidate PSCells of CPA/CPC can be prepared and determined without the source MN involvement. In this case, the candidate PSCells associated with a PCell can be determined by the candidate MN exchanging messages with candidate SNs, and the corresponding CPAC configuration is provided by the candidate MN to the source MN encapsulated in the CHO configuration of PCell. The structure for the configuration of CHO with CPA/CPC can be illustrated in Figure 1. Each encapsulated RRCReconfiguration* message from the candidate MN for CHO configuration can contain the associated CPA/CPC configuration.

Figure 1. Embedded structure for the configuration of CHO with CPA/CPC

If the configuration of CHO with CPA/CPC is provided in such an embedded structure, sequential operation of CHO with CPA/CPC can be performed. The UE first performs CHO evaluation and execution and upon successful accessing the selected target PCell the UE performs CPA/CPC evaluation and execution according to the CPA/CPC configuration included in the RRCReconfiguration*.

Proposal 1: If the configuration of CHO with CPA/CPC is provided in an embedded structure, CHO evaluation and execution is first performed, then CPA/CPC evaluation and execution for candidate PSCells configured for the target PCell is performed.

In another case, CPAC can be determined by the source MN. In this case, the source MN can suggest candidate PSCells for CPA/CPC to the candidate MN. The structure for the configuration of CHO with CPA/CPC can be illustrated in Figure 2. Each CHO execution is associated with multiple CPA/CPC execution condition and each CPA/CPC execution is identified by CondReconfigId of candidate PSCell. And each RRCReconfiguration* message from the candidate MN for CHO configuration contains the paired MCG and SCG configurations which is associated with the CHO execution condition and CPA/CPC execution condition by the CondReconfigId, as illustrated in Figure 2.

Figure 2.  Referring structure for the configuration of CHO with CPA/CPC

If the configuration of CHO with CPA/CPC is provided with such explicit association between configuration IDs of CHO candidate PCell and CPA/CPC candidate PSCell, simultaneous or sequential evaluation of CHO and CPA/CPC can be performed based on configuration. If simultaneous evaluation is configured, the UE evaluates a candidate PCell and the associated candidate PSCells simultaneously. If sequential evaluation is configured, the UE first evaluates candidate PCells. Upon the execution condition for one candidate PCell is fulfilled, the UE starts to evaluate the associated candidate PSCells of the selected candidate PCell.

Proposal 2: If the configuration of CHO with CPA/CPC is provided with explicit association between configuration IDs of CHO candidate PCell and CPA/CPC candidate PSCell(referring structure), the evaluation of CHO and CPA/CPC, i.e., simultaneous or sequential, can be configured by the NW.

On execution condition

For legacy CHO, when source MN decides to use CHO, it selects the candidate PCells and determines CHO execution conditions, which seems still feasible in CHO with CPA/CPC.

Proposal 3： In CHO with CPA/CPC, source MN selects candidate PCells and determines CHO execution condition.

For the embedded structure of CHO with CPA/CPC configuration, CPA/CPC can be determined by the candidate MN, and the candidate PSCells of CPA/CPC can be prepared and determined without the source MN involvement. Therefore, the execution condition for CPA/CPC should be determined by the candidate MN.

Proposal 4: For the embedded structure of CHO with CPA/CPC configuration, CPA/CPC execution condition can be determined by the candidate MN.

For the referring structure of CHO with CPA/CPC configuration, CPA/CPC can be determined by the source MN, and the source MN can suggest or determine the candidate PSCells for CPA/CPC to the candidate MN or candidate MN can provide the selected candidate PSCells to the source MN. Therefore the source MN could obtain the information of candidate PSCells and could determine the CPA/CPC execution condition. For the candidate MN, it will perform SN addition procedure with the candidate SNs for CPA/CPC preparation, which could obtain the information of the candidate PSCells. So the candidate MN could determine the CPAC execution condition either.

Proposal 5: For the referring structure of CHO with CPA/CPC configuration, CPA/CPC execution condition can be determined by the source MN or the candidate MN.

On simultaneous evaluation

In CHO with CPA/CPC,the candidate PCells and the associated candidate PSCells, will be preconfigured. When simultaneous evaluation is configured, UE should evaluates the execution conditions for all candidate PCells and candidate PSCells simultaneously.

But CHO execution condition and CPA/CPC execution condition for PCell and the associated PSCell might not be fulfilled simultaneously, and the potential scenarios are shown in table1.

Table1: Scenarios for fulfillment of CHO/CPAC execution condition

| scenarios | Fulfillment of CHO execution condition | Fulfillment of CPA/CPC execution condition | PSCell associated with PCell |
| --- | --- | --- | --- |
| 0 | √ | √ | √ |
| 1 | √ | √ | x |
| 2 | √ | x | - |
| 3 | x | √ | - |

There are different scenarios for fulfilment of CHO/CPAC execution condition in simultaneous evaluation which will have different impact on UE’s behaviour. According to the agreement from RAN2#120, the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation), for scenario 3, UE should do nothing, just keep evaluating CHO execution condition if only CPAC execution condition of the candidate PSCells is satisfied.

For the other scenarios UE’s behavior need to be defined. And our suggestions are given below:

- For Scenario 0, since CHO execution condition and CPA/CPC execution condition for the candidate PCell and the associated candidate PSCell are fulfilled simultaneously, UE will execute CHO and CPA/CPC simultaneously. Therefore, the MR-DC mobility is optimized and the throughput during handover will be ensured.

- For scenario1 and 2, UE’s behavior is the same. UE will firstly perform CHO to access the target PCell, and keep evaluating execution condition of the associated candidate PSCells of the target PCell. Therefore, UE’s connectivity with network will be ensured.

Proposal 6: In simultaneous evaluation, when CHO execution condition is fulfilled, and the CPA/CPC execution condition is also fulfilled, UE will select the target PCell and the associated PSCell to execute CHO and CPA/CPC simultaneously.

Proposal 7: In simultaneous evaluation, when CHO execution condition is fulfilled without associated candidate PSCell fulfilled CPA/CPC execution condition, UE will firstly select the target PCell to perform CHO, and keep evaluating the associated candidate PSCells of the target PCell.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: If the configuration of CHO with CPA/CPC is provided in an embedded structure, CHO evaluation and execution is first performed, then CPA/CPC evaluation and execution for candidate PSCells configured for the target PCell is performed.

Proposal 2: If the configuration of CHO with CPA/CPC is provided with explicit association between configuration IDs of CHO candidate PCell and CPA/CPC candidate PSCell(referring structure), the evaluation of CHO and CPA/CPC, i.e., simultaneous or sequential, can be configured by the NW.

Proposal 3: In CHO with CPA/CPC, source MN selects candidate PCells and determines CHO execution condition.

Proposal 4: For the embedded structure of CHO with CPA/CPC configuration, CPA/CPC execution condition can be determined by the candidate MN.

Proposal 5: For the referring structure of CHO with CPA/CPC configuration, CPA/CPC execution condition can be determined by the source MN or the candidate MN.

Proposal 6: In simultaneous evaluation, when CHO execution condition is fulfilled, and the CPA/CPC execution condition is also fulfilled, UE will select the target PCell and the associated PSCell to execute CHO and CPA/CPC simultaneously.

Proposal 7: In simultaneous evaluation, when CHO execution condition is fulfilled without associated candidate PSCell fulfilled CPA/CPC execution condition, UE will firstly select the target PCell to perform CHO, and keep evaluating the associated candidate PSCells of the target PCell.

Reference

[1] RP-222332 New WID on Further NR mobility enhancements