3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2303912

E-Meeting, 17th – 26th April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4047 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on sl-MaxTransPower |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  |  | Date: |  |  | 2023-4-10 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.101-1, the defined transmitted power for PSFCH is as follows: - For the total transmitted power PCMAX,PSFCH, PEMAX,c is the value given by IE sl-maxTransPower when single resource pool configured is transmitted at a given time and sum of the IEs sl-maxTransPower when multiple resource pools configured are transmitted at a given time, defined by TS 38.331. For the PSFCH configured power with multiple resource pools, RAN4 send an LS to RAN1 and RAN1 reply with R1- 2302231, and the CR R1-2302232 was agreed in RAN1. Based on RAN4/RAN1 agreement, the current description for sl-MaxTransPower in TS 38.331 is not accurate. The sl-MaxTransPower can only indicate the maximum value of transmission power for PSFCH when it is transmitted in a single pool. When PSFCH is transmitted in multiple pools, the transmission power in one of the pools may exceed the configured sl-MaxTransPower (as per RAN1 latest CR), and there would be only one restriction for the total transmission power, based on sum of the IEs sl-maxTransPower when multiple resource pools configured are transmitted, as specified in TS 38.101-1. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the field description of sl-MaxTransPower, it is clarified that when the parameter is used for PSFCH transmitted in multiple pools, it should be the sum of sl-MaxTransPower that indicates the maximum value of the UE's sidelink transmission power for simultaneously transmitted PSFCHs. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR Sidelink Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When PSFCH is transmitted in multiple pools, the transmission power in one of the pools may exceed the configured sl-MaxTransPower. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.3.5	Sidelink information elements

SL-PowerControl field descriptions

sl-MaxTransPower Indicates the maximum value of the UE's sidelink transmission power on this resource pool for PSSCH, PSCCH and PSFCH (When only transmitting in a single pool). The unit is dBm. When this field is used for the sidelink tranmission power for PSFCH tranmistted in multiple pools, the sum of sl-MaxTransPower indicates the maximum value of the UE's sidelink transmission power for simultaneously transmitted PSFCHs as specified in TS 38.101-1[15].

END OF CHANGE