3GPP TSG-RAN WG2 Meeting #121-bis electronic	R2-2303945

April 17-26, 2023

Agenda Item:	7.12.2.1

Source:	AT&T

Title:	Enhancements for IAB-node mobility and onboard UEs

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Work Item on Mobile IAB for NR [1], 3GPP has agreed to specify enhancements for the mobility of IAB nodes including the following objectives:

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

This contribution discusses solutions for configuring and enabling group mobility for served UEs of a mobile IAB node.

# Mobile IAB and Group Mobility Enhancements

An example of a deployment of vehicle-mounted mobile-IAB-nodes providing 5G coverage/capacity to onboard and/or surrounding UEs is shown in Figure 1:

Figure 1. Mobile IAB with Group Mobility

One of the main advantages of group mobility is the ability to minimize the service interruption during a mobile IAB topology change since the UEs are moving in a correlated manner with the mobile IAB node and also will typically remain connected to the same serving cell of the mobile IAB node after the topology change and handover of the IAB-MT is complete. During RAN2#119bis it was discussed whether during a full migration with “dual DUs” at the mobile IAB node, whether there would be any impacts on the serving UEs during the migration process when either the same PCI is used on the two virtual DUs versus separate PCIs:

From the QC tdoc, The following options O1 O2 O3 are considered:

1) message withholding by the logical source IAB-DU with conditional delivery, e.g., upon on MT migration,

2) conditional execution by the UE based on, e.g., a broadcast indication such as SIB indication of service time or DCI indication of MT-migration, (includes CHO with new trigger).

3) legacy CHO (with implementation specific behaviour, e.g. using source-cell power down or target cell power up triggering the actual HO)

RAN2 assumes that O1 and O3 above could work, and FFS if O2 above (new trigger etc) is needed.

While the details of the full migration still need to be finalized by RAN3, at least at a high level, it could be possible that one impact on UEs during the migration process would be if a UE would try to perform connection (re)establishment either for initial access or coming out of IDLE mode. In this case, it would be beneficial for the newly connected onboard UEs to avoid establishing bearers with the virtual DU which is connected to the source donor CU, since that traffic will need to be quickly migrated to the virtual DU which is connected to the target donor CU and this will incur additional signaling and delay. This issue is novel to mobile IAB compared to the migration solutions considered in Rel-17, which had a longer timeline for migration to support load balancing across stationary IAB nodes.

Proposal 1: RAN2 should consider explicit mechanisms to trigger CHO to the logical target cell and avoid connection (re)establishment of onboard UEs to a virtual source cell of a mobile IAB node during a full migration procedure.

Additionally, during RAN2#119bis the following agreement was made:

Regarding moving status/mode indication, R2 observes that legacy reporting of mobility state (e.g. mobilityState-r16) could be reused, and maybe also current location reporting from the UE. FFS whether any of this need to be enhanced or complemented, e.g. for the potential purpose of predictive mobility.

One mobility enhancement for the IAB-MT function of a mobile IAB node would be to support cell-level predicted trajectory reporting to the donor CU if available. Especially for use cases where the path of the mobile IAB can be planned in advance or is very predictable (e.g. a fixed bus route), the knowledge of the future location of the mobile IAB can be used to optimize the triggering of topology updates via features such as CHO.

Proposal 2: RAN2 should consider support for cell-level predicted trajectory reporting to the donor CU by the IAB-MT of a mobile IAB node.

# Conclusion

This contribution discussed solutions for configuring and enabling group mobility of an IAB node together with its served UEs. The following proposals were made:

Proposal 1: RAN2 should consider explicit mechanisms to trigger CHO to the logical target cell and avoid connection (re)establishment of onboard UEs to a virtual source cell of a mobile IAB node during a full migration procedure.

Proposal 2: RAN2 should consider support for cell-level predicted trajectory reporting to the donor CU by the IAB-MT of a mobile IAB node.

# Reference

[1] RP-221815 New WID on Mobile IAB