3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2304144

# Online, 17 –26 April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4018 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.331 correction on carrier frequency for SL-RSRP measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According RAN5 TS 38.508-1, clause 4.6.6, for sidelink IE "SL-MeasObjectList ", the Value/remark of frequencyInfoSL-r16 is set as "ARFCN-ValueNR of the DMRS used for SL-RSRP measurement. Note: how to set frequencyInfoSL-r16 is FFS". And in TS 38.331, there is no field description for field frequencyInfoSL-r16 within IE SL-MeasObjectList, which is the cause of the confusion. Therefore, it can be clarified by adding the FD that field frequencyInfoSL-r16 is use to indicate the SL carrier frequency in the measurement object configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sub clause 6.3.5, add the field description of frequencyInfoSL in the SL-MeasObjectList, that it "indicates the lowest usable subcarrier on this carrier, determined according to sl-AbsoluteFrequencyPointA in SL-FreqConfig/SL-FreqConfigCommon and offsetToCarrier in SCS-SpecificCarrier configured for sl-SCS-SpecificCarrierList in SL-FreqConfig/SL-FreqConfigCommon. See TS 38.211 [16], clause 4.4.2." Impact analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: Sidelink. Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue because change is for sidelink-specific issue and network is not involved. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue because change is for sidelink-specific issue and network is not involved. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The frequency used for one measurement is not clearly described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.3.5	Sidelink information elements

<<<<<<<<Skipped>>>>>>>>

#### –	SL-MeasObjectList

The IE SL-MeasObjectList concerns a list of SL measurement objects to add or modify for a destination.

SL-MeasObjectList information element

-- ASN1START

-- TAG-SL-MEASOBJECTLIST-START

SL-MeasObjectList-r16 ::=               SEQUENCE (SIZE (1..maxNrofSL-ObjectId-r16)) OF SL-MeasObjectInfo-r16

SL-MeasObjectInfo-r16 ::=               SEQUENCE {

sl-MeasObjectId-r16                     SL-MeasObjectId-r16,

sl-MeasObject-r16                       SL-MeasObject-r16,

...

}

SL-MeasObjectId-r16 ::=                 INTEGER (1..maxNrofSL-ObjectId-r16)

SL-MeasObject-r16 ::=                   SEQUENCE {

frequencyInfoSL-r16                     ARFCN-ValueNR,

...

}

-- TAG-SL-MEASOBJECTLIST-STOP

-- ASN1STOP

SL-MeasObjectList field descriptions

sl-MeasObjectId It is used to identify a sidelink measurement object configuration.

sl-MeasObject It specifies information applicable for sidelink DMRS measurement.

frequencyInfoSL It indicates the lowest usable subcarrier on this carrier, determined according to sl-AbsoluteFrequencyPointA in SL-FreqConfig/SL-FreqConfigCommon and offsetToCarrier in SCS-SpecificCarrier configured for sl-SCS-SpecificCarrierList in SL-FreqConfig/SL-FreqConfigCommon. See TS 38.211 [16], clause 4.4.2.

END OF CHANGES