3GPP TSG-RAN WG2 #121bis-e	R2-2304159

GTW, 17th April – 26th April 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on relations between LCM and Data collection

Document for:	Discussion and decision

Agenda Item:	7.16.2.2

- Introduction

In the RAN2 #121 meeting, RAN2 made following assumption [1]. In this contribution describes the relationship between LCM and data collection frameworks.

Data Collection Proposal 1 RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity. Proposal 2 Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc. Proposal 3 RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted. Proposal 4 Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics. Proposal 5 When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy. Proposal 6 Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed. Proposal 7 Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required. Proposal 8 For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary. P1-P8 are loosely endorsed with the understanding that we can also go beyond, e.g. analyse other methods. Endorse the table (below) as a starting point (e.g. can add more columns if needed later, modify, add rows etc). Content shall be interpreted as current content. Chair: There is significant support to aim for evaluating the data collection methods per LCM purpose R2 may consider including the existing EVEX framework for this SI, FFS exactly what this means, can discuss next meeting. Involved Network entity; RRC state to generate data; Max payload size per reporting*; Contents to be collected; End-to-End report latency**; Report type; Security and Privacy. Logged MDT; TCE/OAM (It can be utilized by gNB); RRC_IDLE/RRRC_INACTIVE; <9kbyte; L3 cell/beam measurements, location info, sensor info, timing info; Procedure latency***: Latency to enter CONNECTED state Latency to receive gNB request signaling (~20ms) Air interface signaling latency****: ~20ms (RRC) Other latency: Forwarding latency between gNB and TCE; Upon gNB request after entering RRC_CONNECTED; AS security via RRC message, Privacy via user consent. Immediate MDT; TCE/OAM (It can be utilized by gNB); RRC_CONNECTED; <9kbyte; L3 cell/beam measurements, location info, sensor info; Procedure latency: Report interval: l20ms~30min for periodic report TTT for event triggered report Air interface signaling latency: ~20ms (RRC) Other latency: Forwarding latency between gNB and TCE; Event triggered report, Periodic reporting; AS security via RRC message, Privacy via user consent. L3 measurements; gNB; RRC_CONNECTED; <9kbyte; L3 cell/beam measurements; Procedure latency: Report interval: l20ms~30min for periodic report TTT for event triggered report Air interface signaling latency: 20ms (RRC); Event triggered report, Periodic reporting; AS security via RRC message.. L1 measurement (CSI reporting); gNB; RRC_CONNECTED; <1706bit in PUCCH, <3840bit in PUSCH; L1 CSI measurement; Procedure latency: Report interval: 4-320 slot for periodic report and semi-persistent report 0-32 slot after reception of DCI for aperiodic report Air interface signaling latency: 1 TTI (PUCCH); Aperiodic report, Semi-persistent report, Periodic report; No AS security. UAI; gNB; RRC_CONNECTED; <9kbyte; Assistance information to show UE preference; Procedure latency: Upon generation of UE's preference Air interface signaling latency: ~20ms (RRC); Up to UE implementation when to report; AS security via RRC message. Early measurements; gNB; RRC_IDLE/RRC_INACTIVE; <9kbyte; L3 cell/beam measurements; Procedure latency: Latency to enter CONNECTED state Latency to receive gNB request signaling (~20ms) Air interface signaling latency: ~20ms (RRC); Upon gNB request after entering RRC_CONNECTED; AS security via RRC message. LPP; LMF; RRC_CONNECTED; <9kbyte; Location info; Procedure latency: Latency to get upper layer trigger (for UE triggered) Or latency to receive NW request message (~20ms) Air interface signaling latency: ~20ms (RRC) Other latency: Forwarding latency between gNB and LMF; UE-triggered, NW-triggered; AS security via RRC message. Note: * The payload size doesn't consider signaling overhead. ** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification. *** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g. entering CONNECTED state, report interval) ****Air interface signaling latency is the latency to transmit one report, e.g. RRC signaling latency or PUCCH signaling latency.

- Discussion

The following techniques are currently being discussed for LCM in RAN1.

- Training phase

  - Data collection

  - Model training/update

  - Model delivery/transfer

  - Model identification

- Inference phase

  - Model activation/deactivation

  - Model monitoring

RAN2 has analysed and summarized the existing frameworks for data collection.

The data collected by these data collection frameworks are expected to be used in other LCM methods, and different LCM require different data and different data characteristics.

Therefore, the relationship with data collection should be summarized by studying whether each LCM requires data and the data requirements of each LCM.

First, the following is a description of the data requirements for each LCM.

- Model training/update

A data set used for model training is required. Enough dataset, either the model input data only or a set of data with output data, must be prepared in advance before training is executed.

- Model delivery/transfer

No other data collection is required since models are only transmitted between NW entities and between NW and UE.

- Model identification

No data collection is required since it only registers and shares models that can be used between UEs and NWs.

- Model activation/deactivation

Based on the results of model monitoring, more effective models are selected. In other words, data collected from model monitoring is necessary.

- Model monitoring

Data that can be used as an indicator for performance assurance when using models is necessary.

From the above, it is expected that data collection will be used mainly for model training/update and model monitoring.

By analysing the requirements for each LCM technique, such as the entity using the data, the expected data size, the contents of the data to be used, and the latency, it will be possible to determine an appropriate data collection framework.

Table: relations between LCM methods and appropriate data collection framework

| LCM methods | Entity Using data | Expected data size | Contents of the data to be used | Latency requirements | Appropriate data collection framework |
| --- | --- | --- | --- | --- | --- |
| Model training | NW entity performing the training | Enough data for model training | Dataset containing model inputs and outputs. Detail depends on use case. | Data set is used after enough data has been collected, so latency is not a consideration | FFS |
| Model update | NW entity performing the training | Amount of data needed to update the model, assuming less than training | Dataset containing model inputs and outputs. Detail depends on use case. | If online training is performed, latency within which data can be provided as a stream | FFS |
| Model monitoring | NW entity determine model activation/deactivation | Amount of information needed to guarantee performance, assuming less than training | Performance Indicators. Details depends on use case. | Low latency to follow quality changes | FFS |

RAN2 should study the relationship between LCM and data collection framework, using the above table as a baseline. In addition, further detailed analysis (e.g., KPIs for each use case, data sets, specific NW entity, etc.) and the need for additional items should be discussed. It is especially desirable to be able to provide details on the specific contents of the data. This point is also under discussion in RAN1 and should be updated in line with RAN1 discussions.

Proposal 1: 	The relationship with data collection should be summarized by studying whether each LCM requires data and the data requirements of each LCM.

Observation1: 	It is expected that data collection will be used mainly for model training/update and model monitoring.

Proposal 2: 	RAN2 should study the relationship between LCM and data collection framework, using the above table as a baseline.

- Conclusion

Our observations and proposals are summarized below.

Proposal 1: 	The relationship with data collection should be summarized by studying whether each LCM requires data and the data requirements of each LCM.

Observation1: 	It is expected that data collection will be used mainly for model training/update and model monitoring.

Proposal 2: 	RAN2 should study the relationship between LCM and data collection framework, using the below table as a baseline.

Table: relations between LCM methods and appropriate data collection framework

| LCM methods | Entity Using data | Expected data size | Contents of the data to be used | Latency requirements | Appropriate data collection framework |
| --- | --- | --- | --- | --- | --- |
| Model training | NW entity performing the training | Enough data for model training | Dataset containing model inputs and outputs. Detail depends on use case. | Data set is used after enough data has been collected, so latency is not a consideration | FFS |
| Model update | NW entity performing the training | Amount of data needed to update the model, assuming less than training | Dataset containing model inputs and outputs. Detail depends on use case. | If online training is performed, latency within which data can be provided as a stream | FFS |
| Model monitoring | NW entity determine model activation/deactivation | Amount of information needed to guarantee performance, assuming less than training | Performance Indicators. Details depends on use case. | Low latency to follow quality changes | FFS |

- References

[1]RP-230661