3GPP TSG-RAN WG2 Meeting #121-bis-e		R2-230XXXX

- Online, 17 – 26 April, 2023

Agenda item:	6.5.3

Source:	Samsung

Title:	Summary of Relay-17 relay UP CR (Samsung)

Document for:	Discussion & Decision

# Introduction

This is the report of the offline discussion.

- [AT121bis-e][426][Relay] Rel-17 relay UP CR (Samsung)

Scope: Check the CR in R2-2304036 and determine whether/how to integrate the TP from P4 of R2-2304191

Intended outcome: Agreed CR (without CB if possible)

Deadline: Monday 2023-04-24 2359 UTC

# Discussion

As per the summary in [1], this is to check companies view on the CR in R2-2304036[2] and the TP for DL data transfer which is associated with the change for UL data transfer in R2-2304036[2].

2.1 TP for UL data transfer

| R2-2304036[2] | Corrections on SRAP for SL relay | NEC | - In clause 5.3.1.2, - clarify how to determine the egress PC5 relay RLC channel for sending SRB1 via default SL-RLC1 - else if the SRAP Data PDU is for SRB1 and if there is not an entry in sl-SRAP-ConfigRemote, whose sl-RemoteUE-RB-Identity matches the SRB identity of the SRAP Data PDU, or if there is an entry in sl-SRAP-ConfigRemote without the corresponding sl-EgressRLC-ChannelPC5: - Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3]; |
| --- | --- | --- | --- |

Q1. Do companies agree the CR in R2-2304036?

| Companies | Agree or Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Apple | Agree | There is one additional issue with the coversheet of 4036, The clause number 5.3.3.2 in summary of change is wrong. |
| China Telecom | Agree |  |
| CATT | Agree |  |
| vivo | Agree |  |

2.2 TP for DL data transfer

The TP (underlined red text) is to be added in clause 5.2.2.2 in TS 38.351. Rapporteur would like to check the views of companies on the TP and if it is agreeable then the TP can be merged into the CR in R2-2304036[2] or a separate CR.

#### 5.2.2.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received): - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3]; - else if the SRAP Data PDU is for SRB1 and if there is not an entry in sl-SRAP-ConfigRelay, whose sl-RemoteUE-RB-Identity matches the SRB identity of the SRAP Data PDU, or if there is an entry in sl-SRAP-ConfigRelay without the corresponding sl-EgressRLC-ChannelPC5: - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3]; - else if there is an entry in sl-SRAP-ConfigRelay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received): - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

Q2.1 Do companies agree the TP for clause 5.2.2.2 in 38.351?

| Companies | Agree or Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| China Telecom | Agree |  |
| vivo | Agree |  |

Q2.2 If agreed in Q2.1, which option do companies support for the TP?

option 1: merge the TP into the CR in R2-2304036[2]

option 2: a separate CR

| Companies | Option 1/Option 2 | Comments |
| --- | --- | --- |
| vivo | No strong view |  |

# Conclusion

We have the following proposals.

# References

[1] R2-2304191, Summary of agenda item 6.5.3, Samsung (Rapporteur)

[2] R2-2304036, Corrections on SRAP for SL relay, NEC