3GPP TSG-RAN WG2 Meeting #121bis-e	R2-2304450

Electronic, 17th – 26th April, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0896 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous Correction on UE capability-R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-4-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The mimo-CB-PUSCH defined as below for the codebook based PUSCH MIMO Transmission was missed in TS 38.306. In reference to the existing definition in TS 38.331, only two sub-elements (maxNumberMIMO-LayersCB-PUSCH, maxNumberSRS-ResourcePerSet) were included. mimo-CB-PUSCH SEQUENCE { maxNumberMIMO-LayersCB-PUSCH MIMO-LayersUL OPTIONAL, maxNumberSRS-ResourcePerSet INTEGER (1..2) } OPTIONAL, Besides, it’s incorrect to describe the prerequisite only for the first sub-element (i.e. maxNumberMIMO-LayersCB-PUSCH). Furthermore, the prerequisite shall be changed to pusch-TransCoherence. Similarly, for the non-CB based PUSCH MIMO transmission, the mimo-NonCB-PUSCH was also missed though it was referenced in a few places. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add mimo-CB-PUSCH to indicate whether the UE supports codebook based PUSCH MIMO Transmission, and fold two existing sub-parameters (maxNumberMIMO-LayersCB-PUSCH, maxNumberSRS-ResourcePerSet) into it, meanwhile change the prerequisite to pusch-TransCoherence. - Add “mimo-NonCB-PUSCH” to indicate whether the UE supports non-codebook based PUSCH MIMO Transmission, and also fold two existing sub-parameters (maxNumberSimultaneousSRS-ResourceTx and maxNumberSRS-ResourcePerSet) into it. Impact analysis Impacted 5G architecture options: SA, MR-DC Impacted functionality: PUSCH MIMO Transmission Inter-operability: If the Network is implemented according to the CR but the UE is not, the UE may report mimo-CB-PUSCH (e.g. with only maxNumberSRS-ResourcePerSet) even it doesn’t support pusch-TransCoherence, which may cause the misunderstanding of the codebook based PUSCH transmission capability at network side. If the UE is implemented according to the CR but the Network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may report mimo-CB-PUSCH even it doesn’t support pusch-TransCoherence, which may cause the misconfiguration at network side for the codebook based PUSCH transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2303878 |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.2.7.8	FeatureSetUplinkPerCC parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| channelBW-90mhz Indicates whether the UE supports the channel bandwidth of 90 MHz. For FR1, the UE shall indicate support according to TS 38.101-1 [2], Table 5.3.5-1. | FSPC | CY | N/A | FR1 only |
| mimo-CB-PUSCH Indicates whether the UE supports codebook based PUSCH MIMO Transmission. If supported, it includes 2 parameters as follows: - maxNumberMIMO-LayersCB-PUSCH defines supported maximum number of MIMO layers at the UE for PUSCH transmission with codebook precoding. - maxNumberSRS-ResourcePerSet defines the maximum number of SRS resources per SRS resource set configured for codebook based transmission to the UE. A UE indicating support of this feature shall also indicate support of pusch-TransCoherence. | FSPC | No | N/A | N/A |
| maxNumberMIMO-LayersNonCB-PUSCH Defines supported maximum number of MIMO layers at the UE for PUSCH transmission using non-codebook precoding. UE supporting non-codebook based PUSCH transmission shall indicate support of maxNumberMIMO-LayersNonCB-PUSCH and mimo-NonCB-PUSCH together. | FSPC | No | N/A | N/A |
| mimo-NonCB-PUSCH Indicates whether the UE supports non-codebook based PUSCH MIMO Transmission. If supported, it includes 2 parameters as follows: - maxNumberSimultaneousSRS-ResourceTx defines the maximum number of simultaneous transmitted SRS resources at one symbol for non-codebook based transmission to the UE. - maxNumberSRS-ResourcePerSet defines the maximum number of SRS resources per SRS resource set configured for non-codebook based transmission to the UE. | FSPC | No | N/A | N/A |
| mTRP-PUSCH-RepetitionTypeB-r17 Indicates whether the UE supports multi-TRP PUSCH repetition for non-codebook based PUSCH repetition type B with sequential mapping for repetitions larger than 2 and cyclic mapping for 2 repetitions by indicating the supported number of SRS resources in one SRS resource set. The UE shall also support two SRS resource sets with usage set to 'nonCodebook'. The UE indicating support of this feature shall also indicate support of mimo-NonCB-PUSCH and pusch-RepetitionTypeB-r16. | FSPC | No | N/A | N/A |

END OF CHANGES