3GPP TSG RAN WG2#121bis-e	R2-2302408

Online, 17th - 26th April, 2023

3GPP TSG RAN WG1 #112	R1-2302144

Athens, Greece, February 27th – March 3rd, 2023

Title:	LS to RAN2 on K2 indication for multi-PUSCH scheduling

Release:	Release 17

Work Item:	NR_ext_to_71GHz-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact person:	Seonwook Kim

seonwook.kim@lge.com

# 1	Overall description

RAN1 observed the inconsistency between 38.214 and 38.331 specifications. The following two types of multi-PUSCH scheduling are supported:

- Type 1: Rel-16 multi-PUSCH scheduling, where a row of the TDRA table can indicate 2 to 8 contiguous PUSCHs

- Type 2: Rel-17 multi-PUSCH scheduling, where a row of the TDRA table can indicate 2 to 8 non-contiguous PUSCHs

According to 38.214 specification, UE determines K2 from k2-r16 (rather than extendedK2-r17) for Type 1 while UE determines K2 from extendedK2-r17 for Type 2. However, according to 38.331 specification, if the size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present, which implies UE shall determine K2 from extendedK2-r17 regardless of Type 1 or Type 2.

To resolve this inconsistency issue, the following TP for 38.331 specification is provided as RAN1’s recommendation.

| Conditional Presence | Explanation |
| --- | --- |
| MultiPUSCH | In case size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present for all n if any two PUSCHs are non-contiguous. Otherwise, it is optionally present, Need S. |

# 2	Actions

To TSG RAN2

ACTION:	RAN1 respectfully asks RAN2 to take the above information into account and capture it in TS 38.331.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#112bis-e	17 - 26 April 2023	e-meeting

RAN1#113	22 - 26 May 2023	Incheon, KR