3GPP TSG RAN WG2#121bis-e	R2-2302431

Online, 17th - 26th April, 2023

3GPP TSG-RAN4 Meeting #106 		R4-2303306

Athens, Greece, February 27 – March 3, 2023

Title:	LS on measurements without gap

Response to:

Release:	Rel-18

Work Item:	NR_MG_enh2-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact person:

#### Name:	Rui Huang , Qiuge Guo

#### E-mail Address:	rui.huang@intel.com , guoqiuge@CATT.cn

Attachments: None

# 1	Overall description

In R18 measurement gap enhancement WI (NR_MG_enh2-Core), RAN4 discussed RRM requirements for measurement without gaps for the following cases:

- NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

- NR SSB-based inter-frequency and intra-frequency measurements without gaps

Firstly, for the case of the NR SSB based inter/intra-frequency measurement without gap when UE report “no-gap” via NeedForGapsInfoNR (Rel16), RAN4 has discussed this issue and reached the following agreement:

<Agreement in R4#105> - Introduce additional Rel-18 UE signalling to differentiate UE supporting no gap with interruption

Therefore, RAN4 would like to ask RAN2 to introduce additional Rel-18 UE signalling to enable the UE to indicate to the NW whether interruption is needed for the case of NR SSB based inter/intra-frequency measurement without gap.

- Inter-RAT measurements without gaps

Secondly, RAN4 classified inter-RAT measurements without gaps to the three subcases below.

  - Case a-1: inter-RAT NR measurements without gap as there is vacant RF chains for UE measurements

  - Case b-1: inter-RAT LTE measurements without gap as there is vacant RF chains for UE measurements

  - Case b-2: inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP

RAN4 has reached the following agreement (given below).

< Agreements in R4#105>: - ONLY on top of ‘interRAT-NeedForGapsNR-r16’ capability to support case a-1.

For Case a-1, RAN4 agreed to introduce additional Rel-18 signalling from UE to indicate the inter-RAT NR measurements without gap but interruption needed.

The other two subcases (Case b-1 and Case b-2) are still in pending discussion in RAN4.

RAN4 would like to respectively ask RAN2 to take the above information into account when designing corresponding RRC signalling in R18.

# 2	Actions

To RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account and design Rel18 signalling for measurements without gap.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #106-bis-e	Apr. 17 – Apr. 26, 2023	Electronic Meeting

TSG RAN WG4 Meeting #107	May 22 – May 26, 2023	Korea