3GPP TSG-RAN WG2 #122	R2-2304671

Incheon, Korea, May 2023

Agenda Item:	7.25.2

Source:	OPPO

Title:	Discussion on R18 UL Tx switching

Document for:	Discussion, Decision

Introduction

This is to discuss the left issues for Tx switching.

Discussion on UE Capability

Issues in [Post121][045]

Current RRC allows UE to report band-pair(s) supporting R16/R17 Tx-switching for a BC via

ULTxSwitchingBandPair-r16 ::=       SEQUENCE {

bandIndexUL1-r16                    INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r16                    INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingPeriod-r16         ENUMERATED {n35us, n140us, n210us},

uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL

}

If we reuse the r16 band pair indication, e.g., for a 3-band BC of band-A/B/C, if UE report

a/ A 3-band FSC, where MIMO layer = 1 or 2 for all band-A/B/C, and

b/ three band-pairs for Tx switching, i.e., A+B, B+C, A+C

It may have different cases:

Case-1: UE only supports 2-band Tx switching, i.e., the one defined in R16/17;

Case-2: UE only support 3/4-band Tx switching, i.e., the one defined in R18;

Case-3: UE support both 2-band Tx switching and 3/4-band Tx switching, i.e., the one defined in R16/17 and the one defined in R18.

Based on R4 LS R4-2306623

The indication of supported switching band combinations of the UE capability can also imply the capabilities of the fallbacks in accordance with the current 38.331 fallback framework.

It ruled out case-2 above.

Given the new parameters defined for R18, e.g., the per-band-pair option, the differentiation between Case-1 and Case-3 can be achieved.

As captured in P6 of [Post-R2#121bis-E][030], one left issue is whether there is a need to mandate the reporting of R18 switching period (although finally the question was removed)

Question 2. RAN2 respectfully asks RAN4 whether the UE needs to:

-	report a length of switching period for 1T-2T switching via Rel-18 signalling even when it is the same value as the switching period being reported via Rel-16 UE capability for the same band pair.

-	report a length of switching period for 2T-2T switching via Rel-18 signalling even when it is the same value as the switching period being reported via Rel-17 UE capability for the same band pair.

Given R4 LS R4-2306623

Rel-18 Tx switching capability will include all sufficient capabilities for two-band switching. In addition, UE may also declare two-band Tx switching capabilities with Rel-16 and/or Rel-17 capabilities.

Our understanding is the optionality should not be on R16/17 capability, but should be on R18 capability, for backward compatibility (for legacy gNB).

But on the other hand, it seems R4 is also thinking about saving the capability reporting when the value is the same.

- UE saves the reporting of a length of switching period for 1T-2T (or 2T-2T) switching via Rel-18 signalling when it is the same value as the switching period being reported via Rel-16 (or Rel-17) UE capability for the same band pair.

Another issue is the feasibility of 3/4 band FSC entry, as captured in [Post-R2#121] [045] summary

Continue discussion

Proposal 1.  RAN2 wait for RAN4 conclusion on fallback of Rel-18 Tx switching to Rel-16/17 Tx switching. In parallel, RAN2 continue following discussion:

-	If it is possible that UE supports both Rel-18 and Rel-16/17 UL Tx switching for the same band combination,

	Approach 1: the 3/4 FeatureSetUplink are reported in one row in FSC for the 3/4 UL bands involved in Rel-18 UL Tx switching;

	Note: If Approach 1 is down-selected, the UE needs to guarantee the FeatureSetUplinks reported for Rel-18 UL Tx switching are applicable to Rel-16/Rel-17 Tx switching if the Rel-16/Rel-17 switching period is reported for that band pair and the same switching option of the band pair is supported for Rel-16/Rel-17 switching.

	Note: If Approach 1 is down-selected, discuss if UEs are allowed to report feature sets for Rel-16/17 UL Tx switching apart from that for Rel-18 UL Tx switching.

	Approach 2: the FeatureSets reported for Rel-16/17 Tx switching between 2 bands can be combined to indicate UL capabilities on the 3/4 UL bands for Rel-18 UL Tx switching;

Please note that opponents may argue that it would lead to restriction of UE capability, e.g.,

In the example above, the band 2 can be paired with band 1 or band 3 and assuming:

- the carrier in band1 is 100MHz;

- the carrier in band 3 is 50MHz;

- the UE can process 150MHz in total;

The carrier in band 2 can only be set to less than 50MHz considering it might be paired with band1 in dual UL transmission. However, when it paired with band 3, the actual capability for band 2 can be extended to 100MHz while it can’t be reported in this scheme. therefore, this report scheme would restrict the report flexibility from UE side.

And thus argue this issue can be solved by always reporting 2-band FSC, so that network cannot misunderstand, but this would lead to unnecessary signaling overhead.

Firstly, this issue only comes from concurrent transmission, i.e., if we limit to switched-UL, there is no dependency between the capability of each FS entry.

Secondly, even for dual-UL, this issue exists since R16, i.e., even for a 2-band FSC, if

- UE can support 150MHz on band-A alone, i.e., for 2T on band-A, or

- UE can support 50MHz on band-B and 100MHz on band-A, i.e., for 1T-1T on band-A/B

If we follow this logic, it means even for 2-band Tx switching, UE should not use a single 2-band FSC, with MIMO-layer = 1 on band-B and MIMO-layer = 2 on band-A for the 2-band Tx switching, since that may lead to lower BW when UE is configured to transmit via 2T on band-A only. A similar issue exists for R17 2T-2T Tx switching.

But please note that we concluded previously for R16 Tx switching, at R2#111 that

[019] Confirm that apart from the agreed 1T+2T UE capability there is no need to report 1T+1T UE capability in new BC list specific for inter-band CA/EN-DC option2.

So there seems no need to revisit this issue again.

- R2 concluded not to handle the capability difference between dual-UL and switched-UL, since R16 2-band Tx switching.

- For P1 from [Post121][045], R2 adopt Approach-1, and clarify UE is not required to but can report an additional 2-band FSC entry (as in legacy if it is with higher capability than parent FSC entry).

Other issues

At 120, it was agreed that

R2 assumes to reuse the per band per BC capability, uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17, on UL-MIMO coherence for the 2Tx-capable UL band(s) for Rel-18 UL Tx switching (fallback description FFS).

For R18 Tx switching involving 2 bands, it is just legacy R16/17 Tx switching, so the delta part is on 3/4 band switching, for which it is assumed the legacy spec is sufficient to handle the fallback issue.

| UplinkTxSwitchingBandParameters-v1700 Contains the UL Tx switching specific band parameters for a given band combination. The capability signalling comprises of the following parameters: - bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination. bandIndex xx refers to the xxth band entry in the band combination. - uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 indicates support of the uplink codebook subset for the carrier(s) on a band capable of two antenna connectors on which UE supports dynamic UL 2Tx-2Tx switching with another band in the band combination. UE indicating support of full coherent codebook subset shall also support non-coherent codebook subset. If this field is absent, the per BC UE capability reported in uplinkTxSwitching-PUSCH-TransCoherence-r16 is applied, and if this field and uplinkTxSwitching-PUSCH-TransCoherence-r16 are both absent, the UE capability reported in pusch-TransCoherence is applied when uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| uplinkTxSwitching-PUSCH-TransCoherence-r16 Indicates support of the uplink codebook subset when uplink 1Tx-2Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. UE indicating support of full coherent codebook subset shall also support non-coherent codebook subset. If the field is absent, the supported uplink codebook subset indicated by pusch-TransCoherence applies when the uplink switching is triggered between last transmitted SRS and scheduled transmission. | BC | No | N/A | FR1 only |

- For R18 Tx switching involving 3/4 band, if uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is absent, the per BC UE capability reported in uplinkTxSwitching-PUSCH-TransCoherence-r16 is applied, and if this field and uplinkTxSwitching-PUSCH-TransCoherence-r16 are both absent, the UE capability reported in pusch-TransCoherence is applied.

Conclusion

We have the following proposals:

Proposal 1	UE saves the reporting of a length of switching period for 1T-2T (or 2T-2T) switching via Rel-18 signalling when it is the same value as the switching period being reported via Rel-16 (or Rel-17) UE capability for the same band pair.

Proposal 2	For P1 from [Post121][045], R2 adopt Approach-1, and clarify UE is not required to but can report an additional 2-band FSC entry (as in legacy if it is with higher capability than parent FSC entry).

Proposal 3	For R18 Tx switching involving 3/4 band, if uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is absent, the per BC UE capability reported in uplinkTxSwitching-PUSCH-TransCoherence-r16 is applied, and if this field and uplinkTxSwitching-PUSCH-TransCoherence-r16 are both absent, the UE capability reported in pusch-TransCoherence is applied.

Reference

- R1-2210724, LS on UE capability and gNB configuration for UL Tx switching across 3 or 4 bands in Rel-18

- R4-2217741, LS on Rel-18 UL Tx switching

- R4-2306623, LS on Rel-18 Tx switching across 3/4 bands