3GPP TSG-RAN WG2 #122	R2-2304672

Incheon, Korea, May 2023

Agenda Item:	7.25.1

Source:	OPPO

Title:	Discussion on MSD Capability

Document for:	Discussion, Decision

Introduction

This is to discuss the MSD Capability.

Discussion

Issue-1: How to understand “inherit”

In the LS, it is stated that

•	Lower MSD capability for higher order combination is inherited from lower order fallback combinations

-	For 2-bands combination, the MSD indication of corresponding threshold (or capability class) are supposed to be reported separately as per victim band per MSD type per band combination

-	For 3-bands combination, the MSD indication of corresponding threshold (or capability class) are only reported for IMD of dual UL falling into the third band DL whose UL band is not configured with, other kinds of Lower MSD capability (harmonic/ harmonic mixing/cross band isolation/IMD due to dual UL falling into own DL) could inherit from 2-band combinations with the same power class.

-	For combination with more than 3 bands, no need to report the Lower MSD capability any more, the capability could inherit from the fallback combinations with the same power class.

It implies that the reporting framework is

- MSD capability is reported for child BC (e.g., 2-band BC)

- MSD capability can be simplified for parent BC (e.g., 3-band BC)

- MSD capability can be saved for grand BC (e.g., BC of more than 3 bands)

This design collides with R2 capability signaling framework of “fallback BC”, i.e., typically parent BC would be included, while reporting of child BC can be saved, if the capability is the same or lower than parent BC, rather than saving the reporting of parent BC.

Following R2 capability signaling framework, the design should be: for a BC

- Include IMD-related MSD for each victim DL band, assuming the dual-UL-transmission, if this BC includes dual UL; and

- Include per-band-pair MSD (one band of UL, and one band of DL) for all MSD types

In this way, child BC can inherit the MSD capability of parent BC, and it is compatible with R2 capability signaling framework.

- In the LS reply, R2 clarifies MSD capability reporting framework should align with the legacy capability signaling framework for the fallback concept.

Issue-2: Per-Power-Class MSD

In the LS, it is stated that

-	For combination with more than 3 bands, no need to report the Lower MSD capability any more, the capability could inherit from the fallback combinations with the same power class.

•	UE is allowed to report lower MSD capability for different power classes

I.e., the MSD may be power-class dependent.

Yet there are some unclear aspects.

Firstly, whether the dependency is on per-BC power class, or other types of power class, since we have per-UE, per-band, per-FSPC power class, etc.

Secondly, given the following statement in the LS

-	For 2-bands combination, the MSD indication of corresponding threshold (or capability class) are supposed to be reported separately as per victim band per MSD type per band combination

It is not super clear whether the dependency of power class means the MSD capability should be reported per-power-class as well. And even if so, it is unclear how to interpret the “per-power class”.

- On the one hand, UE only has a single power-class value for the per-UE power-class, a single value for a specific band for the per-band power-class, or a single value for a specific BC for the per-BC power-class, etc. From this perspective, seems there should be a single MSD value for the per-UE value, a single MSD value for a specific band for the per-band power-class, or a single MSD for a specific BC for the per-BC power-class, etc;

- On the other hand, there is a view that UE may fallback to a lower power class (lower than the PC it supports), depending on different reasons. From this angle, even for a single power-class value of per-UE power class, the UE may report multiple MSD to cover the case that UE fallback to lower PC levels. Whether it is a valid case or not is not clear at the current stage.

Our understanding is that 1) is the correct understanding, while it requires R4 clarification.

- In the LS reply, R2 further asks R4 to clarify, for a specific victim band and a specific MSD type and a specific BC, whether a single MSD capability is sufficient, or multiple MSD capabilities are needed for different power-class values.

Conclusion

We have the following proposals:

Proposal 1	In the LS reply, R2 clarifies MSD capability reporting framework should align with the legacy capability signaling framework for the fallback concept.

Proposal 2	In the LS reply, R2 further asks R4 to clarify, for a specific victim band and a specific MSD type and a specific BC, whether a single MSD capability is sufficient, or multiple MSD capabilities are needed for different power-class values.

Reference

[1] R4-2306594 LS on lower MSD capability.