3GPP TSG-RAN WG2 Meeting #122	R2-2304680

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.9.2

Source:	NEC

Title:	SRAP design and Connection establishment

Document for:	Discussion, Decision

# Introduction

At last RAN2 meeting, there was an initial discussion on the solutions for L2 UE to UE relay. The following agreements are reached:

Agreements @RAN2#121bis:

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.  This does not imply support of all PC5-RRC procedures between the remote UEs.

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID and egress RLC Channel for a particular pair between source remote UE and target remote UE.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

WA: E2E bearer ID (i.e., configuration index in the list of SLRB configurations) is used as input for the L2 U2U relay ciphering and deciphering at PDCP.

LS to SA3 to confirm the feasibility of using the configuration index.

This paper discusses the design on the identification at the adaptation layer and the connection establishment procedure for L2 UE to UE relay.

# Discussion

## SRAP for bearer mapping and SRAP header

As for L2 U2N Relay, for L2 U2U Relay, the SRAP sublayer should be only for the purpose of bearer mapping. At Relay UE, The SRAP sublayer performs bearer mapping between ingress PC5 Relay RLC channels for relaying and egress PC5 Relay RLC channels. For relaying traffic, the different end-to-end PC5 Radio Bearers (SRBs or DRBs) of the same L2 U2U Remote UE and/or different L2 U2U Remote UEs can be multiplexed over the same egress PC5 Relay RLC channel.

Proposal-1: The U2U SRAP sublayer is only for the purpose of bearer mapping.

At Remote UE, from transmission perspective, the SRAP sublayer supports bearer mapping between Remote UE end-to-end Radio Bearers and outgoing PC5 Relay RLC channels. At Remote UE, from reception perspective, it correlates the received packets for the specific PDCP entity associated with the right end-to-end Radio Bearer based on the identity information included in the PC5 SRAP header. As same as U2N relay, a local Remote UE ID should be included in the SRAP header in order to make the SRAP header design consistent between U2N and U2U Relay architecture.

Proposal-2: The identity information of source U2U Remote UE end-to-end Radio Bearer and a local Remote UE ID are included in the SRAP header in order for the target Remote UE to correlate the received packets for the specific PDCP entity associated with the right end-to-end Radio Bearer.

However according to the discussion at the last meeting, it is unclear if the identity information for both Source Remote UE and Destination Remote UE should be included the adaptation layer header.

In the latest SA2 agreement, the sharing PC5 link for the different destination UEs per RSC is supported for the first U2U hop. This means if the U2U Relay UE serves multiple Remote UEs, it is possible that one Source Remote UE may establish one single connection with two separate Destination Remote UEs.

As the same PC5 link applies to both RRC connections, if the data of LCHs supporting the transmission towards the two Destination Remote UEs can be multiplexed within the same SL MAC PDU at the first hop, the Relay UE cannot distinguish which Destination Remote UE the data is supposed to arrive. The following options can be discussed to handle this case:

- LCHs supporting the transmission towards the two Destination Remote UEs is not allowed to multiplexed within the same SL MAC PDU at the first hop

- Specific LCH is allocated for each Destination Remote UE

- Destination Remote UE ID or a local ID is included at SRAP data header

As there are unnecessary restrictions put on the LCP because of option-1/2, we think option-3 may be the best, which enables the PC5 adaptation layer to support bearer mapping for end-to-end UE-to-UE traffic.

Proposal-3:	Destination Remote UE ID or a local ID is included at SRAP data header to support bearer mapping for end-to-end UE-to-UE traffic for first hop.

In addition, it should be clarified if one RLC channel towards Relay UE at first hop can contain multiple source UE ID data, our current assumption is no. The different RLC channel should serve different connection pair between Source Remote UE and Relay UE, and the Relay UE can do a bearer/RLC mapping between the first hop and second hop. There is not a need to include the identity information of source Remote UE in the adaptation layer header.

Proposal-4: The identity information of source Remote UE is not included in the adaptation layer header of first hop assuming there is mapping at Relay UE.

In case of L2 UE-to-Network Relay case, the network controls the configuration of the UEs including both Relay UE and Remote UE. That is why in Rel-17, RAN2 agreed that the network is responsible for the allocation of local UE identities for Remote UE. However, in the UE-to-UE Relay case, the U2U relay UE is the only node that has the full knowledge of the configurations, since there may be no network coverage and the Remote UEs including both source and target may be only aware of their own configurations. Hence we think Relay UE is in a better position to assign and maintain the local UE identities for peer Remote UEs. In addition, the U2U Relay UE may hold a bearer mapping table for each peer of Remote UEs.

Proposal-5: The U2U Relay UE configures source and target Remote UEs with the local UE identities.

## Connection establishment

In legacy handling, the PC5-RRC connection between two peer UEs is implicitly established based on the establishment of the PC5-S connection.

For UE-to-UE relay case, three PC5-RRC connections need to be established: between the source Remote UE and the Relay UE, between the Relay UE and the target Remote UE, and between the source Remote UE and the target Remote UE. We suggest RAN2 to keep the existing connection model to establish the PC5-RRC connection between the source Remote UE and the target Remote UE. This means when the PC5-S connection between the source Remote UE and the target Remote UE is established, the corresponding PC5-RRC connection is automatically established. Then there is no RRC state management between source Remote UE and the target Remote UE as legacy Relay handling. In this way, the additional spec change is avoided to support U2U relay architecture in Rel-18.

Proposal-6: PC5-RRC connection state management is not needed for UE-to-UE Relay.

From end-to-end PC5 RRC connection establishment perspective, it can be only established after the hop-by-hop PC5 RRC connections are established. Since only when the hop-by-hop channel is available, can the end-to-end SL-SRBs (i.e., SL-SRB0 and SL-SRB1) between source Remote UE and the target Remote UE be forwarded. The necessary end-to-end security context can then be established. This aspect may be obvious but we prefer to confirm it for UE-to-UE Relay at high level.

Proposal-7: End-to-end PC5 RRC connection establishment can be only established after the hop-by-hop PC5 RRC connections are established for UE-to-UE Relay.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: The U2U SRAP sublayer is only for the purpose of bearer mapping.

Proposal-2: The identity information of source U2U Remote UE end-to-end Radio Bearer and a local Remote UE ID are included in the SRAP header in order for the target Remote UE to correlate the received packets for the specific PDCP entity associated with the right end-to-end Radio Bearer.

Proposal-3:	Destination Remote UE ID or a local ID is included at SRAP data header to support bearer mapping for end-to-end UE-to-UE traffic for first hop.

Proposal-4: The identity information of source Remote UE is not included in the adaptation layer header of first hop assuming there is mapping at Relay UE.

Proposal-5: The U2U Relay UE configures source and target Remote UEs with the local UE identities.

Proposal-6: PC5-RRC connection state management is not needed for UE-to-UE Relay.

Proposal-7: End-to-end PC5 RRC connection establishment can be only established after the hop-by-hop PC5 RRC connections are established for UE-to-UE Relay.

# Reference

- RP-213585, WID on NR sidelink relay enhancements

- R2-22122321	Connection management and procedures for L2 UE-to-UE relay