3GPP TSG-RAN WG2 Meeting #122	R2-2304682

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.19.2

Source:	NEC

Title:	Fallback behaviour for eRedcap UE

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meetings, RAN2 agreed the basic handling of the fallback behaviour for eRedcap UE:

Agreement @RAN2#121bis:

	Introduce an optional UE capability with signalling for Rel-18 enhanced eDRX in RRC_INACTIVE.

	UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_IDLE eDRX. TBD if it must also support Rel-17 RRC_INACTIVE eDRX.

	A cell can allow Rel-18 INACTIVE eDRX, only if eDRX-AllowedIdle is configured. TBD if it must also configure Rel-17 RRC_INACTIVE eDRX.

	UEs configured with Rel-18 enhanced INACTIVE eDRX should apply Rel-18 enhanced INACTIVE eDRX if Rel-18 enhanced INACTIVE eDRX is allowed in the serving cell, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell.

	UEs configured with Rel-18 enhanced INACTIVE eDRX should apply INACTIVE DRX if both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX are not allowed in the serving cell.

	Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

	Following cases are invalid:

Case 1: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle but not configured with the IDLE eDRX cycle.

Case 2: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle longer than the IDLE eDRX cycle.

However, within the agreement, there are two issues for TBD and one Working assumption, this paper further discusses these remaining issues for the fallback behaviour for eRedcap UEs supporting the enhanced INACTIVE eDRX.

# Discussion

For the first TBD issue, it seems unclear if the UE must also support Rel-17 RRC_INACTIVE eDRX if it can support Rel-18 enhanced eDRX in RRC_INACTIVE. Our understanding is that from UE capability perspective, the support of Rel-18 enhanced eDRX in RRC_INACTIVE should be decoupled from its support of Rel-17 RRC_INACTIVE eDRX, since different UE may benefit from different features in different 3GPP release. For example, for some particular device, they may just apply the very long DRX in RRC_INACTIVE as supported by Rel-18 enhanced eDRX. Here we have the following proposal:

Proposal-1: The UE support of Rel-17 RRC_INACTIVE eDRX is decoupled from its support of Rel-18 enhanced eDRX in RRC_INACTIVE.

For the second TBD issue, it discusses the cell behaviour, it seems unclear if a cell that can allow Rel-18 INACTIVE eDRX must also configure Rel-17 RRC_INACTIVE eDRX. Our understanding is that from network implementation perspective, 3GPP may not limit the cell capability. What RAN2 needs to do is to follow different cell configurations on eDRX, which will be reflected within the system information. We would suggest to leave this flexibility to network implementation. Hence we have the following proposal:

Proposal-2: Whether a cell that can allow Rel-18 INACTIVE eDRX must also configure Rel-17 RRC_INACTIVE eDRX is up to network implementation.

The following work assumption is reached at the last RAN2 meeting:

Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

We did not see the specification complexity and network complexity to allow the UEs configured with Rel-18 enhanced INACTIVE eDRX to fall back to use Rel-17 INACTIVE eDRX if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell.

From UE perspective, this case may need to be considered during UE movement among different cells, which means the UE may need to switch between Rel-18 enhanced INACTIVE eDRX mechanism and  Rel-17 INACTIVE eDRX mechanism during its inactive mobility depending on the updated serving cell configuration in this period. However we did not see the additional complexity because of this configuration. Then we suggest to turn the work assumption into formal agreement. Based on the discussion above, we have the following proposal:

Proposal-3: RAN2 turns the following work assumption into formal agreement .

Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: The UE support of Rel-17 RRC_INACTIVE eDRX is decoupled from its support of Rel-18 enhanced eDRX in RRC_INACTIVE.

Proposal-2: Whether a cell that can allow Rel-18 INACTIVE eDRX must also configure Rel-17 RRC_INACTIVE eDRX is up to network implementation.

Proposal-3: RAN2 turns the following work assumption into formal agreement .

Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

# Reference

- R2-2303561	Discussion on enhanced eDRX in RRC inactive

- R2-2304063	Extending eDRX cycles in RRC_INACTIVE