3GPP TSG-RAN WG2 Meeting #122	   R2-2304697

Incheon, Korea, May 22-26, 2023

Agenda Item:		6.2.1

Source: 	vivo

Title: 	Discussion on MBS Broadcast Reception on SCell

Document for:	Discussion and decision

1 Introduction

In this contribution, we will discuss MBS broadcast reception on SCell.

2 Discussion

In the RAN2#121bis meeting, RAN2 discussed the MBS broadcast reception on SCell and achieved the following agreement [1]:

- Postpone discussion on whether the change is needed for plmn-Index with MBS broadcast reception on SCell in Rel-17. If we agree to fix it, we do this in ASN.1 BC way.

In R17, for receiving MBS broadcast data on SCell, the dedicated RRC signaling is used to provide the SIB20 of the SCell (i.e., via sCellSIB20 IE). Then UEs can acquire the MCCH content (i.e., broadcast MBS session info) directly from the SCells without reading broadcast SIB20 from the SCells. If the PLMN index is used as a part of the TMGI for the broadcast MBS session, UEs need to translate the PLMN index into the PLMN identity or SNPN identity which is broadcasted in the SIB1 of the SCell. However, based on the current specification, UE is not able to read the SIB1 of the SCell (i.e. all the required system information of the SCell is provided via the dedicated RRC signaling to UE). As a result, the PLMN-IdentityInfoList IE and the npn-IdentityInfoList IE in the SIB1 of the SCell are not provided to UE according to the following signaling structure [2]. In other words, UEs have no way to translate the PLMN index into the PLMN identity or SNPN identity due to the lack of PLMN-IdentityInfoList IE and the npn-IdentityInfoList IE.

Observation 1: Due to the lack of the list of PLMN/SNPN in the SIB1 of SCell, UEs have no way to translate the PLMN index into the PLMN identity or SNPN identity if the PLMN index is used as a part of the TMGI of the broadcast MRB session provided via SCell.

To get rid of this, three options can be used to support the MBS broadcast reception on SCell as below:

- Opt 1: The dedicated signaling is extended to provide the list of PLMN/SNPN in SIB1 of the SCell to UE;

- Opt 2: SCell must always provide the PLMN identity as part of TMGI of the broadcast MRB session;

- Opt 3: MBS broadcast reception on SCell within PLMN can be supported based on NW implementation.

For Option 1, it is exactly that this option can widely support all use cases which allow the MBS broadcast reception on SCell. However, this kind of spec modification shouldn't be introduced at such a late stage of Rel-17. In addition, RAN2 has discussed which system information of the SCell can be provided via the dedicated signaling in Rel-15, the final decision is that the list of PLMN in SIB1 of SCell is not provided. If Rel-17 re-opens to discuss whether the list of PLMN/SNPN in SIB1 of SCell can be provided, we are not sure if any other unpredictable issue will occur. At this stage, we do not have enough time to check if any potential issues will be brought.

For Option 2, it is obvious that the MBS broadcast reception on SCell within SPNN cannot be supported as RAN2 has agreed that only the PLMN index can be used for the TMGI of the broadcast MBS session provided within SNPN. In addition, as the SCell of one specific UE can be the PCell of the other UEs, this option means that this cell shall always provide the PLMN identity when the TMGI is indicated. As a result, the gain of overhead reduction by introducing the PLMN index as part of the TMGI via the Uu interface will no longer exist. Moreover, if Option 2 is adopted, it is confusing why RAN2 spends several meetings to discuss the translation of the PLMN index into PLMN identity or SNPN identity in MII handling.

For Option 3, NW can configure UE to receive the MBS broadcast data on SCell if the PLMN identity is used for the TMGI which is interested in the UE. Otherwise, NW will not configure UE to receive the MBS broadcast data on SCell. As a solution, it can be left to UE implementation to establish a unicast PDU session for receiving the MBS broadcast data provided in SCell if UE is not configured by NW to receive the MBS broadcast data on SCell by NW. Same with Option 2, this option will not support the MBS broadcast reception on the SCell within SNPN. Even though this option is only applicable to limited use cases. However, based on the above analysis, this is the safest solution which does not need stage 3 spec change.

Based on the above, we propose,

Proposal 1: MBS broadcast reception on SCell within SNPN is not supported.

Proposal 2: MBS broadcast reception on SCell within PLMN is supported based on NW implementation (i.e., no stage 3 spec impact).

If proposals 1&2 are agreeable, the spec change of stage 2 is needed to clarify the scenario about MBS broadcast reception on SCell. It is proposed to adopt the corresponding spec change of TS 39.300 in the Annex.

Proposal 3: If proposal 1 is agreeable, RAN2 adopts the TP of TS 38.300 in the Annex.

3 Conclusions

In this contribution, we discussed the issues for CP and UP. Based on our discussion, we conclude with the following observation and proposals:

Observation 1: Due to the lack of the list of PLMN/SNPN in the SIB1 of SCell, UEs have no way to translate the PLMN index into the PLMN identity or SNPN identity if the PLMN index is used as a part of the TMGI of the broadcast MRB session provided via SCell.

Proposal 1: MBS broadcast reception on SCell within SNPN is not supported.

Proposal 2: MBS broadcast reception on SCell within PLMN is supported based on NW implementation (i.e., no stage 3 spec impact).

Proposal 3: If proposal 1 is agreeable, RAN2 adopts the TP of TS 38.300 in the Annex.

4 References

- RAN2#121BIS chairman note.

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

5 Annex

TP of the 38.300

16.10.6.3	Support of CA

UE can be configured to receive MBS broadcast data and MCCH either from a PCell or a single SCell at a time. MBS broadcast reception on SCell within SNPN is not supported in this release of the specification. MBS broadcast reception on SCell within PLMN can be configured if the PLMN identity is used as part of the TMGI of the MBS broadcast session. Meanwhile, dedicated RRC signalling is used for providing SIB20 of the SCell i.e., while in RRC_CONNECTED state, UEs need not acquire broadcast SIB20 directly from the SCells.

NOTE:	The UE may be able to receive MBS broadcast also from a non-serving cell, which is transparent to the network, e.g., it does not require UE capability or MBS Interest Indication to be sent by the UE.