3GPP TSG-RAN WG2 Meeting #122	   R2-2304705

Incheon, Korea, May 22-26, 2023

Agenda item:		7.23.2

Source:	vivo

Title:	Discussion on RAN feedback for Upstream Scheduling

Document for:		Discussion and Decision

1 Introduction

In RAN#99 meeting, the new WID on NR Timing Resiliency and URLLC enhancements had been approved. The working objective regarding RAN feedback for low latency communication are listed below [1],

Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a.RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication. Note 3: Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

Further, in RAN2#121bis-e meeting, a tentative discussion on whether and how to support RAN feedback for upstream was initiated. There was no consensus on whether to support it eventually, and the corresponding agreements are listed below [2]:

RAN2#121bis-e agreements: We will wait for XR to progress and see if we can use the existing mechanism. RAN2 will strive to re-use existing mechanism or rely on gNB to determine the information (i.e. aim to not introduce new UE specific BAT reporting)

In this contribution, we will further provide our understanding on RAN feedback for upstream scheduling based on the agreements regarding delay reporting in the XR session given in the Annex.

2 Discussion

Based on the data traffic model, buffer delay for the UL data occurs when the arrival time of data is not aligned with the transmission timing of UL resources granted by the network. To get rid of this, on one hand, the application layer of UE may autonomously adjust the arrival time of UL data based on the delay offset obtained from the lower layer. On the other hand, if RAN feedback reporting is supported, then UE may report the burst arrival time offset and/or data periodicity to the network so that the network can be aware of the UE’s date state and allocate UL resources that are matched with the upstream pratten.

In our understanding, the former solution based on UE internal implementation may help to reduce the buffered delay, however, it is not suitable for some cases, such as time-sensitive transmission. This is because the network currently cannot know the precise upstream pratten. As a result, the UE cannot transmit the UL data precisely at a given time without a timely UL resource. Therefore, for time-sensitive UL transmission, it is beneficial to allow UE upstream feedback reporting to the RAN node.

Observation 1: Buffer delay can be reduced by UE internal implementation to match the UL burst arrival time and the timing of UL resources, but it may not be helpful for time-sensitive transmission.

Observation 2: It can be beneficial to allow RAN feedback (e.g. BAT offset and traffic periodicity) for upstream in the time-sensitive network.

Proposal 1: RAN2 confirms RAN feedback from UE to RAN for upstream scheduling is supported.

Regarding how to support RAN feedback from UE to RAN for upstream, in the previous meeting,  a common understanding was that RAN2 should strive to re-use XR mechanism. For the XR, delay reporting (e.g. reporting the queued time of UL data before transmission)  might be adopted. With that, the network can calculate the actual BAT offset based on the queued time reported and the round-trip time of UL grant allocation and PUSCH transmission. Thus, we think the design of delay reporting in XR precisely matches the motivation of RAN feedback for upstream. Reusing it in Rel-18 URLLC should be adopted.

Proposal 2: Reuse the delay reporting mechanism defined in XR session for RAN feedback.

Further, in XR, the delay reporting is associated with BSR reporting for one or more than LCG or LCH. Technically, the BSR is used for data volume reporting when data arrive for a logical channel that has higher priority than the other logical channels, or when the first data arrives and the UE’s buffer is empty, or when the periodicBSR-Timer expires.

Associating BSR reporting and delay reporting seems not so necessary in the case of TSN network, as the data volume information is not needed for the network to some degree. To be efficient,  RAN2 should consider decoupling BSR reporting and delay reporting for that valid use case (i.e. the UE can only report the delay information, or report both BSR and delay information). Therefore, we propose,

Proposal 3: RAN2 considers decoupling BSR reporting and delay reporting for Rel-18 RAN feedback.

3 Conclusion

In this contribution, we have further provided our understanding on RAN feedback for upstream scheduling based on the agreements. All the observations and proposals are summarized below:

Observation 1: Buffer delay can be reduced by UE internal implementation to match the UL burst arrival time and the timing of UL resources, but it may not be helpful for time-sensitive transmission.

Observation 2: It can be beneficial to allow RAN feedback (e.g. BAT offset and traffic periodicity) for upstream in the time-sensitive network.

Proposal 1: RAN2 confirms RAN feedback from UE to RAN for upstream scheduling is supported.

Proposal 2: Reuse the delay reporting mechanism defined in XR session for RAN feedback.

Proposal 3: RAN2 considers decoupling BSR reporting and delay reporting for Rel-18 RAN feedback.

4 References

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements.

- Draft_R2_121bis_meeting_report_v2.

5 Annex Agreements:

RAN2#119bis-e meeting agreements:

Delay information consists of at least “remaining time”.

2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

RAN2#120 meeting agreements:

RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.