3GPP TSG-RAN WG2 Meeting #122	   R2-2304707

Incheon, Korea, May 22-26, 2023

Agenda item:		7.18.3

Source:	vivo

Title:	Discussion on Supporting MT-SDT from UP Perspective

Document for:		Discussion and Decision

1 Introduction

In the last RAN2 meeting [1], agreements related to MT-SDT support were achieved as follows,

Include a one-bit indication in paging to trigger MT-SDT. We will ensure that the CCCH message can be transmitted over CG. Indication is per UE. FFS on signalling. In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS. Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17) - FFS 3a: Check for DVT (if UL data becomes available in UL) - 3b: Check for SDT RSRP threshold - 3c: Check for TA validation before selecting CG (if applicable) - 3d: Check for SSB level RSRP threshold for CG resource (if applicable) When UE resumes for MT-SDT, UE resumes all RBs configured for SDT. RBs configured for SDT are common for MO-SDT and MT-SDT. If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement. Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT). UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures. If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy).

In this contribution, we would like to further provide our considerations on the MT-SDT procedure in terms of MT-SDT triggering, resource selection, and DL data reception.

2 Discussion

  - 2.1	MT-SDT Triggering

After the reception of a matched paging message triggering MT-SDT procedure, additional conditions need to be checked for the applicable of MT-SDT procedure. In the previous meeting, a tensive discussion was initiated about this. And an FFS is left that whether data volume check is needed when MO-SDT may be triggered.

In our understanding, based on the agreement:

UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures.

The UE-implementation fallback from MT-SDT to MO-SDT, as shown in Figure 1, is feasible even when after reception of MT-SDT paging but before CCCH transmission. To allow such kind of fallback, when the RRC request the MAC to do the SDT check, the MAC should first check whether the MO-SDT conditions are satisfied. If not, then it should subsequently check the MT-SDT specific trigger (e.g. no data volume is needed). If all the conditions for SDT is not fulfilled, the UE should only use the legacy RA resource for the RRC resumption procedure.

Figure 1: MO-SDT trigger after MT-SDT paging

Based on the above understanding, we propose,

Proposal 1: In response to MT-SDT paging, the MAC entity shall,

- if the conditions for MO-SDT are satisfied, trigger MO-SDT procedure as legacy;

- elseif the conditions for MT-SDT are satisfied, trigger MO-SDT procedure;

- else using legacy RA resource (i.e. non RA-SDT resource) for RRC resumption procedure.

Further, for the conditions for MT-SDT, as agreed, no data volume is needed. And based on our modelling for resource configuration, the UE can further check whether the MT-SDT specific CG resources can be used during MT-SDT procedure (e.g. the the TA for MT-SDT specific CG resources is valid and the RSRP of any SSB associated the MT-SDTspecific CG resources is above a configured threshold). If there is no available MT-SDT specific CG resources, then legacy RA resource is used with setting resume cause to MT-SDT specific cause, as mentioned in our companion paper R2-2304706.

Proposal 2: UE can initiate MT-SDT procedure as long as the following are met,

- There is at least one RB configured as SDT-RB;

- Downlink pathloss reference is higher than a configured threshold, if applicable.

Proposal 3: After triggering MT-SDT procedure, UE initiates CG-based MT-SDT procedure if the MT-SDT specific CG resources are valid. Otherwise, the UE initiates RA-based MT-SDT procedure with MT-SDT specific resume cause.

  - 2.2	Resource selection

If the it was agreed that MT-SDT specific CG resources can be configured on both NUL and SUL carriers. Then the next question comes to whether the legacy UL carrier and BWP selection mechanism can be reused for MT-SDT. Similar questions are also discussed in Rel-17 SDT session and CovEnh sessions. And the conclusion is that the legacy UL carrier and BWP selection mechanism is reused and is not impacted by the newly introduced feature for the sake of UE simplicity. No optimization is adopted for some unusual configuration cases. For CG-based MT-SDT procedure using MT-SDT specific CG resources, we understand the design logic is the same as Rel-17. The UE should anyway select a UL carrier and BWP as legacy and then further check whether MT-SDT triggers are fulfilled on the selected BWP of the selected UL carrier.

Proposal 4: UL carrier selection mechanism is not impacted by MT-SDT (i.e., legacy UL carrier selection mechanism is re-used).

Proposal 5: BWP selection mechanism is not impacted by MT-SDT (i.e., legacy BWP selection mechanism is re-used).

  - 2.3	DL data reception

Technically, if the RA procedure is used for MT-SDT procedure, the first DL data delivery can be done via,

- scheduling by paging message,

- Msg2,

- Msg4/MsgB,

- the other DL scheduling after Msg4/MsgB.

Obviously, including a UE ID and the corresponding scheduling information for DL data delivery into the paging message will largely increase the paging overhead and may incur additional complexity on the CU operation. Thus, it should be excluded from the MT-SDT scope.

If multiplexing the DL data into the Msg2 message, more attention is required to investigate the security issue, the new MAC PDU structure design. For simplicity, we suggest ruling out this solution.

For the other two solutions, it is already supported in RA-SDT. They can be reused in MT-SDT procedure.

Having said the evaluation above, similar to the RA-SDT, we think the first DL data delivery shall not be performed before the transmission of Msg4/MsgB.

Proposal 6: For RA-based MT-SDT procedure,  the first DL data delivery shall not be performed before the transmission of Msg4/MsgB.

3 Conclusion

In this contribution, we have provided our considerations on the MT-SDT procedure in terms of MT-SDT paging, MT-SDT triggering, resource selection, DL data reception, and fallback. All the observation and proposals are given as follows,

MT-SDT triggering:

Proposal 1: In response to MT-SDT paging, the MAC entity shall,

- if the conditions for MO-SDT are satisfied, trigger MO-SDT procedure as legacy;

- elseif the conditions for MT-SDT are satisfied, trigger MO-SDT procedure;

- else using legacy RA resource (i.e. non RA-SDT resource) for RRC resumption procedure.

Proposal 2: UE can initiate MT-SDT procedure as long as the following are met,

- There is at least one RB configured as SDT-RB;

- Downlink pathloss reference is higher than a configured threshold, if applicable.

Proposal 3: After triggering MT-SDT procedure, UE initiates CG-based MT-SDT procedure if the MT-SDT specific CG resources are valid. Otherwise, the UE initiates RA-based MT-SDT procedure with MT-SDT specific resume cause.

Resource selection:

Proposal 4: UL carrier selection mechanism is not impacted by MT-SDT (i.e., legacy UL carrier selection mechanism is re-used).

Proposal 5: BWP selection mechanism is not impacted by MT-SDT (i.e., legacy BWP selection mechanism is re-used).

DL data reception:

Proposal 6: For RA-based MT-SDT procedure,  the first DL data delivery shall not be performed before the transmission of Msg4/MsgB.

4 References

[1] R2-121 Meeting Report.