3GPP TSG-RAN WG2 Meeting #122	R2-2304708

Incheon, Korea, May 22 – 26, 2023

Agenda item:	7.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on XR awareness

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss what should be included in traffic assistance information from UE to network.

# Discussions

## UL traffic information

At the last RAN2 meeting, the following agreements were made:

UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting. FFS on whether EoDB signalling is needed.

UL jitter information

Among the usual statistics of a traffic parameters (e.g. min, max, mean, std, etc), since the main use of jitter is to help network configure UL radio resources such as configured grants, its range probably is the most useful one. For example, it can help network better choose the interval between PUSCH occasions in a multi-PUSCH CG. Or the start offset of a configured grant. Similar use of DL jitter range was the reason why SA2 agreed that CN should provide it to RAN [1]. Therefore, we think jitter range is also the right information for UE to report to RAN.

Proposal 1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the same way as the one for N6 jitter.

On DL, CN provides jitter range associated with each DL periodicity. However, this reporting granularity may not be possible, because logical channel is the finest granularity for scheduling in AS. Hence, for UL jitter, its range can be reported per logical channel. And network can configure for which logical channel UE should report jitter information.

Proposal 2. 	Range of UL jitter is reported per logical channel. Network can configure for which logical channel UE should report jitter information.

Jitter in traffic arrivals can be dynamic. Hence jitter information should be reported periodically or when there is a significant change.

Proposal 3. 	Network can configure whether UE reports UL jitter information periodically or only when the range exceeds a threshold.

End of Data Burst (EoDB) indication

In our view, EoDB indication is useful for RAN in UE power savings, especially for XR applications with uplink-centric traffic (e.g. AR). Network typically does not know when a data burst on uplink ends, because size of data bursts in XR traffic varies between frames and is not predictable. However, UE often is able to get indication from XR application when a data burst ends or identify that by itself through traffic inspection (e.g. as what SA2 have agreed for DL EoDB). If UE is able to provide this indication to network, then network can use it to determine whether to terminate UE’s DRX active time early. That will give UE more time to sleep before the next data burst.

Observation 1. 	End of burst indication by UE can help network determine whether to terminate DRX active time early and thus save UE more power.

In the last RAN2 meeting, there was discussion on whether UL EoDB indication should be a new signal or legacy padding BSR can be used for the same purpose. In our view, padding BSR is not a reliable way to indicate end of data burst. For example, where there is jitter in UL traffic, there can be temporary gap between PDUs. If a padding BSR with zero buffer is sent before all PDUs in a data burst have been received, it can cause network to prematurely terminate UE’s DRX active time. The remaining PDUs in the data burst may have to trigger SR to be scheduled again. On the other hand, if an explicit EoDB indication is supported, a smart network implementation can continue scheduling UL grants to UE even after receiving a padding BSR with zero buffer. Therefore, an explicit EoDB indication can help network schedule UE with more certainty and avoid extra latency.

Observation 2. 	Padding BSR may not be a reliable way to indicate EoDB when there is jitter in UL traffic. It may cause premature termination of DRX active time and interruption in scheduling.

Observation 3.	An explicit EoDB indication can help network schedule UE with more certainty.

There can be different ways for UE to indicate an EoDB. For example, to help network act promptly on an EoDB indication (e.g. terminate DRX active time), this indication can be sent via L1 signaling such as a UCI. Or if reliability of an EoDB indication is more important, it can be sent via L2 signaling, e.g. in a MAC CE, a L2 protocol header, or a L2 control PDU. These options perhaps can be FFS.

Proposal 4.	Introduce EoDB indication explicitly signaled by UE. FFS the type of signal used to send the indication.

Another issue is whether any additional UE behavior after sending an EoDB needs to be specified.

Network may have the following options after receiving an EoDB indication:

- Do nothing and let DRX inactivity timer expire by itself; or

- Schedule a PDCCH skipping longer than the DRX inactivity timer; or

- Send a DRX MAC CE to terminate UE’s DRX active time.

With XR applications’ short traffic periodicities (e.g. 16.6 msec), if network replies on DRX inactivity timer to terminate DRX active time, there will not be much time left for UE to sleep between two data bursts. And it loses the benefits of having EoDB indication.

In the current specification, PDCCH skipping is signaled only in a scheduling DCI. If there is no more data on both DL and UL (e.g. both DL and UL EoDB have been received) and network is to terminate UE’s DRX active time, there will be no data left for network to send PDCCH skipping.

The downside with DRX MAC CE is that there is a fixed amount of delay (3msec) for it to effect. This delay may not be an issue when DRX cycle is long (e.g. VoNR or web browsing). But for XR applications with short traffic periodicities, this delay can take away a considerable portion of sleep opportunity between two data bursts.

Therefore, we think a method for UE to end DRX active time faster than the above three options is useful. For instance, if UE does not receive any new or retransmission DCI when the HARQ retransmission timer associated with the PUSCH transmission containing the EoDB indication expires, UE can autonomously end its DRX active time. Otherwise, UE follows the legacy behavior without change.

Proposal 5. 	Specify a method for UE to terminate its DRX active time faster than the legacy ones after it sends a UL EoDB indication. FFS details of this method.

Other UL traffic information

It is anticipated that network may configure one or more CGs to support XR traffic, as transmission over CG can effectively reduce the latency of UL access for UE. That makes the stringent delay requirements of some XR QoS flows (e.g. pose updates) more manageable.

When configuring a CG for UE, one of the key parameters that the network needs to know is the traffic periodicity, such as the frame rate. While it has been agreed by SA2 that the CN provides both UL and DL traffic periodicity of a QoS flow to the RAN during session establishment, we believe that it would still be useful for UE to provide this information directly to RAN. It is not intended to replace the signaling by CN, but rather to complement it. For example, in case there is a change in UL traffic periodicity, signaling from UE directly to RAN has much lower latency than having it propagate through AF, PCF, SMF, and then to the RAN.. It is up to RAN to decide how to use information on traffic periodicity from both CN and UE (e.g. always apply the latest one).

Proposal 6.	UE can report UL traffic periodicity of a logical channel to RAN. This information is a complement, not a replacement, to the traffic periodicity provided by CN to RAN.

When configuring a CG for a UE, the network may face difficulty in determining the appropriate start offset, as typically network is not aware of the nominal arrival time of a UL traffic flow or the start time of a UL data burst in advance. However, this information is crucial, as any mismatch between the start time of traffic and the start offset of its associated CG could result in additional access latency, which would reduce the benefits of using a CG. We therefore think it is useful for UE to indicate its preferred start offset for a CG in UE assistance information to RAN.

Proposal 7. 	UE can report its preferred start offset for a CG to RAN.

## Signaling options

At the last RAN2 meeting, different options for signaling UL traffic information were discussed, such as PINE [2] and TSCAI [3].

We do not think PINE is a good option for UE to report UL jitter (or traffic information in general). First, the signaling is from UE to 5GC, so that 5GC needs to rely it back to RAN. However, UL jitter information, at least so far, has not been identified as required by 5GC. It hence makes more sense for UE to report UL traffic information directly to RAN. Second, generally speaking, it is usually not a good idea to couple two separate features in the same release, because that would create undesired dependency in their deployment.

We do not think TSCAI is good option either, at least from UE’s perspective. TSCAI so far is defined for use in 5GC DL only. Copying it for UL traffic information report requires UE to implement a new feature in AS signaling. On the other hand, the RRC message, UE Assistance Information, has been used since Rel-15 to signal UE’s preferred configuration parameters and other information to RAN. Considering that UL traffic information for XR is primarily used in RRC configurations, we believe that using the existing UAI message is an effective way for UE to communicate this information to the RAN, while minimizing UE implementation efforts.

Proposal 8. 	UL traffic information can be signaled via the RRC message UE Assistance Information.

# Conclusion

Based on the discussions, we’d like to suggest RAN2 discuss and agree the following the proposals:

UL jitter information

Proposal 1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the same way as the one for N6 jitter.

Proposal 2. 	Range of UL jitter is reported per logical channel. Network can configure for which logical channel UE should report jitter information.

Proposal 3. 	Network can configure whether UE reports UL jitter information periodically or only when the range exceeds a threshold.

End of data burst indication

Observation 1. 	End of burst indication by UE can help network determine whether to terminate DRX active time early and thus save UE more power.

Observation 2. 	Padding BSR may not be a reliable way to indicate EoDB when there is jitter in UL traffic. It may cause premature termination of DRX active time and interruption in scheduling.

Observation 3.	An explicit EoDB indication can help network schedule UE with more certainty.

Proposal 4.	Introduce EoDB indication explicitly signaled by UE. FFS the type of signal used to send the indication.

Proposal 5. 	Specify a method for UE to terminate its DRX active time faster than the legacy ones after it sends a UL EoDB indication. FFS details of this method.

Other UL traffic information

Proposal 6.	UE can report UL traffic periodicity of a logical channel to RAN. This information is a complement, not a replacement, to the traffic periodicity provided by CN to RAN.

Proposal 7. 	UE can report its preferred start offset for a CG to RAN.

Proposal 8. 	UL traffic information can be signaled via the RRC message UE Assistance Information.

# References

- S2-236201, EN resolving for KI#8 Except TSCAC and clarifications on AF inputs, Tencent, Tencent Cloud, Huawei, HiSilicon, April 2023.

- R2-2303986, Discussion on UL jitter handling, Samsung, April 2023.

- R2-2303800, Considerations on PDU sets and Traffic assistance information for XR, CMCC, April 2023.