3GPP TSG-RAN WG2 Meeting #122	R2-2304710

Incheon, Korea, May 22 – 26, 2023

Agenda item:	7.5.4.2

Source:	Qualcomm Incorporated, Huawei, HiSilicon, Meta

Title:	Discussion on SFN wrap around

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In the RAN2#121bis-e meeting, the following agreement was made:

- To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

In this paper, we follow up on this agreement and discuss design options for the remaining open issues.

# Discussions

We first discuss how to implement the counter used in the DRX formula.

We think that there are two main options for implementing the counter:

•	Option 1: The counter is broadcast in system information (as with the legacy SFN). It can be either H-SFN or a new SFN-based counter (E-SFN);

•	Option 2: The counter is maintained independently by the UE. Its initial value and/or maximum can be either pre-defined in the spec or initialized via dedicated signalling (RRC).

For Option 1, we do not think it is a good idea to use H-SFN for the new counter. The reason is that the maximum value of H-SFN, which is 1024 (H-SFN is 10-bit long), can’t be fully divided by some XR frame rates. For example, for 60 fps, 1024 x 10240 / (1000/60) is not an integer. In addition, use of H-SFN would mandate a serving cell that supports XR to support eDRX as well. Coupling two independent features requires joint deployment of these two features and thus is not desirable.

An alternative is to introduce a new counter (E-SFN) similar to H-SFN in system information (SI) but is only for supporting XR. UE acquires the value of this counter from SI whenever its DRX is re-/configured. In our view, the downside of this approach is that if an acquisition of E-SFN takes multiple attempts that span across SFN boundaries, then there can be different understandings between UE and network on what the initial value of the counter is.

On the other hand, Option 2 does not have the above issues. In term of impact on the MAC spec, Option 2 is on par with Option 1, i.e. in both options, the MAC spec only needs to specify that UE increments the counter when SFN wraps around. The two options differ only in how the initial value and maximum (if applicable) of the counter are set.

Therefore, based on the above analysis, we’d propose that Option 1 should not be given further consideration.

Proposal 1. 	The counter in DRX formula for addressing SFN wrap around is not obtained from system information.

Next, we discuss how the counter in Option 2 should be defined/configured.

The first issue is whether the counter needs a maximum value or not. There were views at the last RAN2 meeting that the maximum of the counter does not need to be specified, i.e. it can keep incrementing without wrapping around, because a RRC connection will not last forever. If that is agreed, then it is up to UE/NW implementation to make sure that the storage for the counter is large enough so that it never overflows in any cases. However, if overflow does happen and NW and UE allocate different field length for the counter, there will be inter-operability issue.

Observation 1. 	If maximum value of the counter is not specified, there can be inter-operability issue between different UE/NW implementations of the counter.

Since defining or configuring a maximum value for the counter does not create much complexity, we think it is better to have it defined/configured.

To ensure that DRX cycles always perfectly align with the boundary of SFN, this maximum should be chosen such that 10240 x M is an integer multiple of any DRX cycles that UE supports (which directly relates to frame rates supported by UE). Considering the uncertainty around new DRX cycle values at the moment (e.g. integer versus rational numbers) and beyond, we think it is better to have the maximum value configured by network during UE’s DRX re-/configuration than to predefine it in the spec. That will provide better flexibility and forward compatibility.

Proposal 2. 	The maximum value of the counter is RRC configured by network together with DRX re-/configuration.

The initial value of the counter may depend on how the DRX reference SFN is used. We see two possible options:

- Option A: the DRX reference SFN is directly added to the DRX formula, in the same fashion as how time reference SFN is used in the Rel-16 configured grant formula;

- Option B: the DRX reference is not directly used in the DRX formula. Instead, UE uses the DRX reference SFN signalled by network to set the initial value of the counter.

The details of these two options are discussed in the following.

In the Option A, there is no coupling or interaction between the counter (denoted by NSFN) and DRX reference SFN (denoted by drx-ReferenceSFN):

- The counter NSFN is applied to ensure boundaries of DRX cycles and SFN always are aligned, for all possible DRX cycles. In the MAC spec, NSFN can be added to the left hand side of the DRX formula;

- The DRX reference SFN drx-ReferenceSFN is used to avoid the RRC configuration ambiguity problem. In the MAC spec, drx-ReferenceSFN can be added to the right hand side of the DRX formula.

The enhanced formula for long DRX cycle can be the following:

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) =

(drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-LongCycle)

And similarly for short DRX cycle:

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) =

(drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-ShortCycle)

In this option, the counter NSFN can be initialized to 0 when DRX is configured.

The Option B is motivated by the question whether there is an alternative that has less impact on the legacy DRX formula.

We observe that the main purpose of DRX reference SFN is to add a “shift” to the start offset in the DRX formula if the RRC configuration is sent and received on different sides of a SFN boundary. This “shift” then allows both sides of the DRX formula to have a common “start”.

Note that the same effect can be achieved if the initial value of NSFN is set based on DRX reference SFN. More specifically, assume network assigns the values of DRX reference SFN in the same way as in Rel-16, i.e. drx-ReferenceSFN = 0 if RRC configuration is first sent between SFN 0 and 512. Otherwise, network sets drx-ReferenceSFN = 512. Then based on the received value of DRX reference SFN and the SFN in which UE successfully receives RRC configuration message, UE can determine if ambiguity may exist, i.e. if drx-ReferenceSFN = 512 and UE receives RRC configuration in a SFN between 0 and 512. In that case, UE can set the initial value of NSFN to 1. In other cases, ambiguity does not exist. Hence UE can set the initial value of NSFN to 0.

By setting the initial value of NSFN based on the value of DRX reference SFN, a “shift” is applied to the left hand side of the DRX formula when RRC configuration is sent and received across SFN boundary. As a result, DRX reference SFN does not need to be added to the DRX formula anymore. The resulting DRX formulas thus are:

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset;

[(NSFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) =

(drx-StartOffset) modulo (drx-ShortCycle).

These changes obviously have less impact on the current spec than those in Option A.

Based on the above discussion, we’d thereby propose that

Proposal 3. 	Network sets DRX reference SFN drx-ReferenceSFN to either 0 or 512, in the same way as in Rel-16 IIoT.

Proposal 4.	RAN2 discuss and select one of the following options:

- Option A: both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to DRX formula. The initial value of NSFN =0;

- Option B: only NSFN is added to DRX formula. However, the initial value of NSFN is set according to drx-ReferenceSFN as follows:

  - If UE successfully receives RRC configuration in SFNUE, UE initializes the counter NSFN to 1 if 0 ≤ SFNUE < 512 and drx-ReferenceSFN = 512 (i.e. there may be ambiguity);

  - Otherwise (i.e. there is no ambiguity), UE initializes NSFN to 0.

# Conclusion

Based on the above analysis, we’d suggest RAN2 to discuss and agree the following proposals:

Proposal 1. 	The counter in DRX formula for addressing SFN wrap around is not obtained from system information.

Observation 1. 	If maximum value of the counter is not specified, there can be inter-operability issue between different spec implementations.

Proposal 2. 	The maximum value of the counter is RRC configured by network during DRX re-/configuration.

Proposal 3. 	Network sets DRX reference SFN drx-ReferenceSFN to either 0 or 512, in the same way as in Rel-16 IIoT.

Proposal 4.	RAN2 discuss and select one of the following options:

- Option A: both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to DRX formula. The initial value of NSFN =0;

- Option B: only NSFN is added to DRX formula. However, the initial value of NSFN is set according to drx-ReferenceSFN as follows:

  - If UE successfully receives RRC configuration in SFNUE, UE initializes the counter NSFN to 1 if 0 ≤ SFNUE < 512 and drx-ReferenceSFN = 512 (i.e. there may be ambiguity);

  - Otherwise (i.e. there is no ambiguity), UE initializes NSFN to 0.