3GPP TSG-RAN WG2 Meeting #122	R2-2304711

Incheon, Korea, May 22 – 26, 2023

Agenda item:	7.5.4.1

Source:	Qualcomm Incorporated

Title:	BSR and delay status reporting for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss design options for new BSR tables and enhancements on delay status reporting.

# Discussion

## New BSR Tables

At the last RAN2 meeting, the following agreements were made:

- New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size

According to the above agreements, RAN2 should design new BSR table(s) such that network can schedule UL grants whose sizes better match UE’s buffered data. For XR traffic, the size range of UE’s buffered data is mostly determined by the size range of video frames, which can be known beforehand based on the bit rate (denoted by R) and frame rate (denoted by F) used by the application. For example, suppose an XR application applies AVC codec with 4K resolution, which produces a bit rate in the range of 20~150 Mbps [2]. Then at 60 fps, the size range of a data burst = R/8/F = 20/8/60 ~ 150/8/60 = 41~312 KB.

Since quantization error matters more at high end of buffer sizes, the maximum data burst size should be covered by the new BSR table(s). We thus should set it as the maximum buffer size for new BSR table(s) (denoted by Bmax). It can be derived by the following steps:

- Since size of a data burst = R/8/F, it is easy to see that maximum burst size is associated with maximum bit rate and lowest frame rate. From Table 4.5-1 in [2], which is copied below for reference, we can see that AVC 8K high-quality produces the highest bit rate of 150 Mbps.

Table 1. Expected Video coding standards performance and bitrate target

| # Codec | # Coding performance # (Random-Access) |  | # Targeted bitrate # (Random Access) |
| --- | --- | --- | --- |
|  | Objective | Subjective |  |
| AVC |  |  | 4k: - Statmux: 20-25 Mbps - CBR: 35 - 50 Mbps 8k: - CBR: 80 - 100 Mbps - High quality: 100 - 150 Mbps [33][34][35] |
| HEVC | -40% vs AVC [33][34][35] | -60% vs AVC [33][34][35] | 4k: - Statmux: 10-13 Mbps - CBR: 18-25 Mbps 8k: - CBR: 40-56 Mbps - High quality: 80-90 Mbps [33][34][35] |

- Since the set of common frame rates are 24, 25, 30, 50, 60, 90, 120, 144 and 240, we thus can derive the maximum frame size to be 150Mbps/8/24 = 781,250 B.

Next, we determine the minimum of new BSR table(s) (denoted by Bmin) as follows.

- First, given the relatively small value of the maximum frame size, we can tentatively assume that only one new BSR table is needed. As we will show below, this assumption works well.

- Second, we need to choose number of code points (or length of BS field) for the table. In our view, the legacy 8 bit BS field (256 code points) already provide a sufficiently fine granularity, especially given the fact that new BSR table(s) has narrower range than legacy. Moreover, since both new and legacy BSR tables may be used in new BSR MAC CE, it is desirable for new BSR table(s) to have the same BS field length as that in the legacy table. That will help keep the format of new BSR MAC CE simple.

- Third, with linear distribution, quantization error is the largest at Bmin and then decreases with buffer size. Since the objective of new BSR table(s) is to reduce quantization error, then at least at Bmin, the quantization error should be no worse than that of the legacy table. Since the quantization error of the legacy BSR table is 6.5% across all code points, we then have the following requirement for the new BSR table:

step sizeBmin= (Bmax-Bmin)/256Bmin≤6.5% → Bmin ≥ Bmax1+256 ×6.5%

Solve the above inequality for Bmin, we obtain Bmin ≥ 44,289 B.

At Bmax, the quantization error is step size divided by Bmax, which is (781,250 - 44,289)/256/781,250 = 0.37%. The arithmetic average of quantization errors across all code points is 1.1%. In our view, this result is quite good. Since average frame size is around 100KB for the most common bit rates and frame rates, this result means the mismatch between reported BS and actual BS differs only about one IP packet.

Observation 1. 	It is possible to define a single new BSR table that achieves an average quantization error of ~1% as follows:

- It covers all common bit rates and frame rates;

- Use linear distribution and 8-bit BS field length;

- Choose maximum BS based on maximum bit rate and minimum frame rate;

- Choose minimum BS such that no BS in the new table has a quantization error worse than the legacy one.

If we take a closer look at the above result, we can discover that the new table (Bmin, Bmax, each BS in the table) generated based on the method described above is scales with maximum bit rate. More specifically,

- Bmax = R/8/F is a linear function of maximum bit rate;

- Bmin = Bmax / (1 + 256 x 6.5%) = R/8/F/ (1 + 256 x 6.5%), which is also a linear function of maximum bit rate.

- Since all code points are generated based on linear distribution, each of them then is a linear function of maximum bit rate too.

The implication of the above observations is that this new BSR table is forward compatible, e.g. if in the future new maximum bit rate is introduced or there is no need to support 8K bit for AR applications, we can simply introduce a scaling factor that UE can use to update the table. There is no need to go through the excise of defining a new table again.

Observation 2. 	The new BSR table generated by the method in Observation 1 is forward compatible because the entire table scales linearly with maximum bit rate.

Since a single pre-defined BSR table can achieve satisfactory performance, we think it should be adopted instead of RRC configured tables, for its lower impact on UE implementation.

Proposal 1. 	Introduce a single pre-defined BSR table, e.g. derived based on the method described in Observation 1.

If companies still prefer to have UE generate a new BSR table based on parameters configured by RRC, it is important that the parameters and the formula used in the calculation yield the same results across different UE and network implementations. For example, division and power operation should be avoided, because different implementations may use different floating-point precisions in those calculations.

Observation 3.	If new BSR table(s) are RRC configured, then parameters and formula used to generate a new BSR table should be defined in a way that different UE/NW implementations can produce the same table.

Although RAN2 have agreed that linear distribution should be used as the baseline for generating new BSR table(s), we think exponential distribution can be considered too, if new BSR table(s) is RRC configured. For example, if the range of a BSR table is relatively small and linear steps can provide sufficient quantization accuracy, then linear distribution can be used. Otherwise, exponential distribution can provide lower quantization errors. We thus think whether to use linear or exponential distribution can be left to network configuration. More specifically,

- If network specifies that step sizes are linearly distributed, the buffer size Bk can be generated according to the following formula: B1 = Bmin, and Bk = Bk-1 + floor(Bmin x p), for k=2, …, N.

- If network specifies that step sizes are exponential, the buffer size Bk can be generated according to the following formula: B1 = Bmin, and Bk = Bk-1 + floor(Bk-1 x p), for k=2, …, N.

Proposal 2.	 If new BSR table(s) is RRC configured,

- Network configures UE with minimum buffer size Bmin, distribution type (linear or exponential), and step size factor p.

- Buffer size Bk can be generated according to the following formula: B1 = Bmin, and Bk = Bk-1 + floor(BS x p), for k=2, …, N, where BS = Bmin linear distribution and BS = Bk-1 for exponential distribution.

Proposal 3.	The length of BS field is 8 bits, regardless of whether new BSR table(s) is pre-defined or RRC configured.

The motivation of using new BSR table(s) is to reduce quantization error in BS reporting. Therefore, if there are multiple BSR tables (including the legacy one) for UE to use, then UE should always choose the one which has the least quantization error for the buffer size to be reported.

Proposal 4.	If there are multiple BSR tables (including the legacy one) for UE to use, then UE should always choose the one which has the least quantization error for the buffer size to be reported.

Since multiple BSR tables may be used by UE, network needs to know which BSR table UE has used when reporting. Hence UE has to provide an indication for that in the new BSR MAC CE.

Proposal 5. 	The new BSR MAC CE should include an indication by UE on which BSR table it used for reporting.

## Delay status reporting (DSR)

We may first discuss which entity should report DSR, e.g. whether it should be per LCG or per LCH. For the same reason why LCG is used for BSR reporting, we think network likely would only group LCHs with compatible priorities (delay requirements) into the same LCG. And per-LCG reporting can allow the possibility that delay status is reported together with buffer status. Hence LCG can be a good choice for the report entity for DSR.

An XR application may generate different types of flows. Some flows have tight delay requirements, while others do not. For those flows that do not have tight delay requirements, then there probably is no need for them to report their delay status, even when such a report is triggered by another flow. Therefore, network can configure which LCG(s) should report its delay status. Not every LCG needs to report their delay status in a DSR.

Observation 4.	It is not necessary for UE to report delay status of every QoS flow, e.g. those without stringent delay requirements.

Proposal 6.	Network can configure which LCG(s) should report its delay status.

For the level of urgence required by XR traffic (e.g. 15~30 msec), we do not think it is necessary for UE to include a DSR in every of its PUSCH transmission. Instead, network can configure a time threshold for an LCG to trigger DSR, i.e. UE triggers a DSR if the maximum remaining time among data buffered in the layer-2 buffers corresponding to the LCG drops below the configured time threshold.

Proposal 7.	UE triggers a DSR when an LCG configured for reporting and its associated L2 buffer has data whose remaining time drops below a configured triggering threshold.

As to the remaining time, it could be defined as the residual delay budget of a PDU or PDU Set, i.e. the duration from the current time till the delay deadline. However, a more direct way to define the remaining time of a PDU is the residual time of its associated PDCP discard timer [3], because ultimately gNB should decide when to schedule a PDU based on when the PDU may be discarded. Network can set PDCP discard timer based on, say, PSDB or something else. That can be left to network implementation.

Proposal 8. 	The remaining time of a PDU is defined as the residual value of its associated PDCP discard timer.

We think it is also useful to have timer triggered DSR, for the same reason that BSRs can be triggered by both events and timers.

Proposal 9.	Network can also configure an LCG to periodically report its delay status.

Depends on the preferred granularity of reporting, network may configure one or more reporting thresholds. UE then reports the amount of data whose remaining time is below a reporting threshold. For example, suppose network configures two reporting thresholds, T1 and T2, where T1 < T2. Then UE reports the amount of data whose remaining time is shorter than T2, and the amount of data whose remaining time is between T1 and T2.

Proposal 10.	Network can configure one or more reporting thresholds for an LCG. For each reporting threshold, UE reports the amount of data whose remaining time is below that threshold.

The remaining time used in determining the amount of data for each reporting threshold can be different from the remaining time which triggered the DSR, because typically there is additional delay between the time when a DSR is triggered and the time when a DSR is transmitted over PUSCH.

The reported remaining time hence should be defined as the duration between the time/slot when a DSR is transmitted and the delay deadline (which is the full duration of the corresponding PDCP discard timer). Since the time/slot when a DSR is transmitted can be determined ahead of time, there is no timeline issue.

Proposal 11.	The remaining time reported in a DSR is the duration between the time when the DSR is transmitted and the delay deadline of the corresponding data.

# Conclusion

Based on the above discussions, we’d suggest RAN2 discuss and agree to the following proposals:

New BSR table

Observation 1. 	It is possible to define a single new BSR table that achieves an average quantization error of ~1% as follows:

- It covers all common bit rates and frame rates;

- Use linear distribution and 8-bit BS field length;

- Choose maximum BS based on maximum bit rate and minimum frame rate;

- Choose minimum BS such that no BS in the new table has a quantization error worse than the legacy one.

Observation 2. 	The new BSR table generated by the method in Observation 1 is forward compatible because the entire table scales linearly with maximum bit rate.

Proposal 1. 	Introduce a single pre-defined BSR table, e.g. derived based on the method described in Observation 1.

Observation 3.	If new BSR table(s) is RRC configured, then parameters and formula used to generate a new BSR table should be defined in a way that different UE/NW implementations can produce the same table.

Proposal 2.	If new BSR table(s) is RRC configured,

- Network configures UE with minimum buffer size Bmin, distribution type (linear or exponential), and step size factor p.

- Buffer size Bk can be generated according to the following formula: B1 = Bmin, and Bk = Bk-1 + floor(BS x p), for k=2, …, N, where BS = Bmin linear distribution and BS = Bk-1 for exponential distribution.

Proposal 3.	The length of BS field is 8 bits, regardless of whether new BSR table(s) is pre-defined or RRC configured.

Proposal 4.	If there are multiple BSR tables (including the legacy one) for UE to use, then UE should always choose the one which has the least quantization error for the buffer size to be reported.

Proposal 5. 	The new BSR MAC CE should include an indication by UE that which BSR table it used for reporting.

Delay status reporting

Observation 4.	It is not necessary for UE to report delay status of every QoS flow, e.g. those without stringent delay requirements.

Proposal 6.	Network can configure which LCG(s) should report its delay status.

Proposal 7.	UE triggers a DSR when an LCG configured for reporting and its associated L2 buffer has data whose remaining time drops below a configured triggering threshold.

Proposal 8. 	The remaining time of a PDU is defined as the residual value of its associated PDCP discard timer.

Proposal 9.	Network can also configure an LCG to periodically report its delay status.

Proposal 10.	Network can configure one or more reporting thresholds for an LCG. For each reporting threshold, UE reports the amount of data whose remaining time is below that threshold.

Proposal 11.	The remaining time reported in a DSR is the duration between the time when the DSR is transmitted and the delay deadline of the corresponding PDU.

# References

- TR 26.998, Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices, V18.0.0, Jan 2023.

- TR 26.298, Technical Report on the Study on Extended Reality (XR) in 5G, v18.0.0, March 2023.

- R2-2304408, Discussion on BSR enhancement and delay information report, LG Electronics, April 2023.