3GPP TSG-RAN WG2 Meeting #122	R2-2304732

Incheon KR, May 22nd – 26th, 2023

Agenda Item:	7.6.2.2

Souce:	MediaTek Inc.

Title:	Enhancements on GNSS operation

Document for:		Discussion and Agreement

Introduction

In RAN#98e [1], the revised WID on IoT NTN enhancements has been endorsed for Release 18. The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks).

The WID considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective related to improved GNSS operations as follows:

Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

- NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

This contribution aims to discuss RAN2 aspects related to enhanced GNSS operations for IoT NTN.

2 Discussion

GNSS position fix time duration reporting

RAN2#121 has made the following agreement:

For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used: RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB, RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB, FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete. FS for Msg3

UE is in RRC connected mode when sending RRCConnectionReestablishmentComplete(-NB) and RRCConnectionReconfigurationComplete. The network should be able to know UE’s capability as well as the reported GNSS position fix time duration. Hence it is not necessary to report the GNSS position fix time duration in RRCConnectionReestablishmentComplete(-NB) and RRCConnectionReconfigurationComplete

Proposal 1: UE does not report GNSS position fix duration in RRCConnectionReestablishmentComplete(-NB) and RRCConnectionReconfigurationComplete.

During the connection, the GNSS position fix time duration may vary due to the GNSS state changes (cold start, warm start, hot start, etc.). But the possibility of GNSS state change is very small, and the connection should not last for a long time, we think it is not necessary to report the GNSS position fix time duration in connected mode.

Proposal 2: UE reporting GNSS position fix time duration in connected mode is not supported.

In Rel-17 the UE reports the remaining GNSS validity duration in Msg5. When the reported value is infinity, it implies two cases. The first case is the UE is able of performing the GNSS measurement during the DL/UL transmission. When the GNSS validity duration ends, UE can autonomously get a new GNSS position fix and it would not break the connection. Therefore, network can consider the GNSS validity duration of UE as infinity. The other case is that the UE is a stationary device, which is unlikely to move during the connection. There is no need to obtain a new GNSS position fix during the connection, hence network can consider the GNSS validity duration of the UE as infinity.

When it comes to Rel-18, in both cases mentioned above, the UE does not need to obtain a new GNSS position fix during the connection process, so there is no need to report the GNSS position fix time duration.

Proposal 3: UE does not report GNSS position fix time duration when the remaining GNSS validity duration in Msg5 is infinity.

Network aperiodically triggers UE to make GNSS measurement

RAN1#111 has made the following agreement:

Agreement - If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used.

In the last RAN2 meeting, some companies suggest that MAC CE may have security issue. The MAC CE is not protected by the ciphering and integrity protection. However, the signaling using MAC CE is not transferring the user data, the damage is very limited. The existing signaling using MAC CE like Timing Advance Command can cause the same level of damage. There is no AS security protection in NB_IoT CP solution, even if the RRC signaling is used to trigger GNSS measurement, it can cause the same level of damage.

Proposal 4: A new DL MAC CE is used by the network to trigger UE to make GNSS measurement in connected mode.

UE re-acquires GNSS autonomously (when configured by the network)

RAN1#111 has made the following agreement:

Agreement The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

RAN1#112bis has sent LS to RAN2:

| Conclusion From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2 Send an LS to RAN2 for the conclusion. |  |
| --- | --- |

According to RAN1’s agreement, besides the network trigger, UE may re-acquire GNSS autonomously when configured by the network. The timing of UE autonomously trigger can be:

- DRX inactive time before GNSS validity duration expiry

- After GNSS validity duration expiry

- Time X delay after GNSS validity duration expiry

For the case 1, as long as the inactive time of C-DRX is longer than GNSS position fix duration, and there is no overlapping PDSCH/PUSCH transmission, UE can utilize this period to autonomously start GNSS measurement without any interruption on the connection. Therefore, it is better to start GNSS measurement before any possible network trigger and before GNSS validity duration expiry. But it is also not good to start GNSS measurement too early. Ideally, UE should start GNSS measurement somewhere near the validity duration expiry. The exact time of UE start GNSS measurement during inactive state of C-DRX can be left for UE implementation. Because network has no knowledge that when the GNSS measurement was performed in this case, UE should somehow notify the network that UE has re-acquired the GNSS measurement autonomously. Network can cancel any following aperiodically trigger.

Proposal 5: UE can autonomously start GNSS measurement during the inactive state of C-DRX.

Proposal 6: The exact time of starting GNSS measurement during the inactive state of C-DRX can be left for UE implementation.

Proposal 7: UE notify network that UE has successfully re-acquired GNSS position fix during the inactive state of C-DRX and the new GNSS validity duration.

If network does not aperiodically trigger GNSS measurement, UE should be able to autonomously trigger GNSS measurement at the last minute of the duration that UE can make UL transmission to maintain a long connection.

For case 2, the expiry of GNSS validity duration only indicates the GNSS position fix is not valid anymore, it does necessarily mean UE cannot transmit UL since then. Therefore, the case 3 can be considered, to extend time X after GNSS validity duration. The time X can be a network configuration, or a time until Timing alignment timer expiry.

Proposal 8: Network can configure UE to allow UE autonomously to re-acquire GNSS measurement after GNSS validity duration expiry + X.  The detail of time delay X can be further discussed.

Similar like the measurement gap for network aperiodical trigger, a time duration for GNSS measurement should be configured by the network. The default value of time duration can be the UE reported GNSS position fix duration. To control the UE behavior, a new RRC timer should be introduced.

Proposal 9: A new RRC timer is configured by RRC signaling where UE can re-acquire GNSS position fix autonomously. FFS for details of the RRC signaling.

Leaving connected mode

When should the UE leave RRC connected and move to RRC idle? In general, UE should move to RRC idle when failed to acquire new GNSS position fix. The exact UE behavior should depend on how the GNSS measurement is triggered.

- If the GNSS measurement is started by network aperiodically trigger, UE moves to RRC idle when UE failed to re-acquire GNSS position fix. Because when network send the trigger, it means network found that it is necessary to re-acquire GNSS at a certain point of time, otherwise the UL timing/frequency synchronization may not be maintained. If it is failed to reacquire the GNSS position fix, there is no point to keep the connection.

- If UE was configured with UE autonomously acquisition after GNSS validity duration expiry + X, UE moves to RRC idle when UE failed to re-acquire GNSS position fix after GNSS validity duration expiry + X. The time of GNSS validity duration expiry + X is the last minute that UE think the UL timing/frequency synchronization can be maintained. If it is failed to reacquire the GNSS position fix, there is no point to keep the connection.

- If UE failed to re-acquire the GNSS position fix during the inactive state of C-DRX, UL timing/frequency synchronization can still be maintained, thus another attempt of GNSS measurement triggered by network or UE can be allowed. It is possible that the GNSS satellites were temporarily blocked, another attempt of GNSS measurement is beneficial. UE does not move to idle when UE failed to re-acquire the GNSS position fix during the inactive state of C-DRX.

- If there is no network aperiodically trigger and network configuration of UE autonomously acquisition of GNSS position fix after GNSS validity duration expiry + X, UE moves to idle when GNSS validity duration expired.

Proposal 10: If network aperiodically trigger the GNSS measurement, and UE failed to re-acquire the GNSS position fix, UE moves to RRC_IDLE.

Proposal 11: If UE failed to autonomously re-acquire GNSS position fix after GNSS validity duration expiry + X, UE moves to RRC_IDLE.

Proposal 12: If UE failed to autonomously re-acquire the GNSS position fix during the inactive state of C-DRX, UE does not move to idle.

Proposal 13: If there is neither network aperiodically trigger nor network configuration of UE autonomously acquisition of GNSS position fix after GNSS validity duration expiry + X, UE moves to RRC_Idle when GNSS validity duration expired.

3 Conclusions

Proposal 1: UE does not report GNSS position fix duration in RRCConnectionReestablishmentComplete(-NB) and RRCConnectionReconfigurationComplete.

Proposal 2: UE reporting GNSS position fix time duration in connected mode is not supported.

Proposal 3: UE does not report GNSS position fix time duration when the remaining GNSS validity duration is infinity.

Proposal 4: A new DL MAC CE is used by the network to trigger UE to make GNSS measurement in connected mode.

Proposal 5: UE can autonomously start GNSS measurement during the inactive state of C-DRX.

Proposal 6: The exact time of starting GNSS measurement during the inactive state of C-DRX can left for UE implementation.

Proposal 7: UE notify network that UE has successfully re-acquired GNSS position fix during the inactive state of C-DRX and the new GNSS validity duration.

Proposal 8: Network can configure UE to allow UE autonomously to re-acquire GNSS measurement after GNSS validity duration expiry + X. Time delay X can be further discussed.

Proposal 9: A new RRC timer is configured by RRC signaling where UE can re-acquire GNSS position fix autonomously. FFS for details of the RRC signaling.

Proposal 10: If network aperiodically trigger the GNSS measurement, and UE failed to re-acquire the GNSS position fix, UE moves to RRC_IDLE.

Proposal 11: If UE failed to autonomously re-acquire GNSS position fix after GNSS validity duration expiry + X, UE moves to RRC_IDLE.

Proposal 12: If UE failed to autonomously re-acquire the GNSS position fix during the inactive state of C-DRX, UE does not move to idle.

Proposal 13: If there is no network aperiodically trigger and network configuration of UE autonomously acquisition of GNSS position fix after GNSS validity duration expiry + X, UE moves to RRC_Idle when GNSS validity duration expired.

4 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.