3GPP TSG-RAN WG2 Meeting #122	 R2-2304733

Incheon KR, May 22nd – 26th, 2023

Agenda Item:	7.6.3.1

Souce:	MediaTek Inc.

Title:	Enhancements on neighbour cell measurement

Document for:		Discussion and Agreement

# 1 Introduction

Rel-18 enhancements for IoT-NTN Work Item was approved at RAN Plenary #96e [1]. The corresponding work item has identified IoT-NTN Mobility Enhancements as one key agenda. The work item objective (WID) has mentioned the following two enhancements:

- Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline.

- Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC.

In the RAN2 #119-e meeting, the following agreements has been made:

Agreements:

- IoT NTN can use the mechanism for neighbour cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g. triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g. the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

In the RAN2#119bis-e meeting, due to the limit time, only the following agreement has been made:

Agreements:

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events

In the RAN2#120 meeting, we had the following agreements:

Agreements:

1.	For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

2.	At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

3.	RAN2 will not specify the condition of stopping UE measurement before t-Service

4.	For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

5.	For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation

6.	CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.: UE is allowed to perform HO only during T1 to T2.

7.	For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR)

8.	For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR).

9.	FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

In the RAN2#121 meeting, we had the following agreements:

1. Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN) 2. Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN) 1. We don’t introduce any new low mobility criterion for enhanced mobility in Rel-18

In the RAN2-121bis meeting, RAN2 made the following agreements:

- New SIBxx is introduced to broadcast the neighbor cell/satellite information.

- Common TA parameters are broadcast as assistance information for neighbor cell measurements.

- Kmac is broadcast as neighbor cell assistance information.

- For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN)

- Introduce satellite ID for the satellite in a list in new SIB-xx. FFS on the details of the new IE

- For eMTC NTN, for fixed cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN)

- For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode

- SIB3 is extended to include the reference location and distanceThresh

This contribution will discuss the measurement triggers for IoT-NTN.

# 2 Discussion

Measurement based Serving cell coverage

In the RAN2-121 meeting, for quasi-earth-fixed cell and earth-moving cell, a serving cell reference location and a distance threshold/radius are specified for detecting when to trigger connected mode measurements. FFS on whether the R17 IEs are reused or not.

The Rel-17 footprint IE(footprintInfo-r17) was introduced for discontinuous coverage in SIB32. It includes referencePoint-r17(including longitude-r17 and latitude-r17), elevationAngles-r-17,radius-r17. Since the elevation angles is not used in rel-18, it is better not to reuse the footprintInfo-r17. But the referencePoint-r17 and radius-r17 can be reused.

Proposal 1: For earth fixed and earth moving cell, serving cell reference location uses the Rel-17 footprint IE (i.e. referencePoint-r17 and radius-r17)

In the RAN2-121bis meeting, it has been agreed that SIB3 is extended to include the reference location and distanceThresh. However, for earth moving cell, the refence location is actully changing erery minute. SIB3 will not be updated in RRC connected mode.For a UE staying RRC Connected state long enough, the reference location will become invalid and misleading.  For a UE in idle mode, a frequent SIB3 update is expected due to the varying reference location. It will cause undisired paging messages. In the onther hand, put the reference location and distanceThresh in SIB31 is more suitable, as SIB31 will be frequently updated without paging messages.And UE will update SIB31 in RRC connceted state.

Propoal 2: RAN2 revises the agreement to SIB31 is extended to include the reference location and distanceThresh.

The new SIBxx

In the RAN2-121bis meeting, new SIBxx is introduced to broadcast the neighbor cell/satellite information.

The SIBxx is used to provide neighbor cell information for neighbor cell measurement in RRC connected state, without SIBxx UE can still work, hence SIBxx is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled.

Proposal 3: SIBxx is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled.

The SIBxx contains the neighbor cell ephemeris information that varying with time especially for earth moving cell. Hence SIBxx will be update frequently just like SIB31. The update to SIBxx should not affect the system information value tag and does not trigger system information modification procedure. Otherwise, it will cause frequent paging messages to notify UE to update the system information.

Proposal 4: The update to SIBxx should not affect the system information value tag and does not trigger system information modification procedure.

In RAN2-121bis meeting, it has been agreed that for eMTC NTN, for fixed cell and earth moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes. Hence for eMTC, UE needs to acquire SIBxx in RRC idle state. UE should only acquire SIBxx when it is needed, for example, when a neighbour cell measurement has been triggered. The exact time of acquisition of SIBxx in idle state can be left for UE implementation.

Proposal 5: For eMTC, UE acquires SIBxx in RRC idle state. The exact time of acquisition of SIBxx in idle state can be left for UE implementation.

In Rel-17, UE acquires the SIB31 before accessing the cell. That’s because SIB31 is essential for UE to estimate the UL timing. Unlike SIB31, the new SIBxx is not essential for UE before accessing the cell. If UE always acquire the SIBxx before accessing the cell, it will increase the connection establishment latency, and SIBxx is not always needed in the connected state.However, if UE does not have a valid SIBxx before entering RRC connected state, when UE needs the assistance information in SIBxx, UE cannot autonomously acquire the SIBxx in connected state.

Proposal 6: RAN2 to discuss whether UE needs to acquire the SIBxx before connection establishment/resumption/re-establishment.

As the SIBxx contains the ephemeris of satellites, the SIBxx also needs epoch time and validity duration. The serving cell’s epoch time and validity duration can be used when it is possible. The epoch time and validity duration can be optional.

Proposal 7: SIBxx contains optional epoch time and validity duration. If absent, the epoch time and validity duration in SIB31 can be used.

Measurement trigger based Neighbor cell coverage

Besides UE starts measurements before the time of losing coverage of current cell, UE can also start measurements when entering the neighbor satellite’s coverage if assistance information is provided.

For quasi-earth fixed cell, if start time and footprint information is provided, for earth moving cell, if footprint information is provided, UE can know when it will enter neighbor cell’s coverage. It can help to improve mobility in RRC connected state.

Proposal 8: For earth-fixed neighbor cell, start time and footprint is broadcasted.

Proposal 9: For earth-moving neighbor cell, footprint is broadcasted.

Proposal 10: In connected mode, when a neighbor cell comes under coverage, UE may start neighbor cell measurement.

# 3 Conclusions

Proposal 1: For earth fixed and earth moving cell, serving cell reference location uses the Rel-17 footprint IE (i.e., referencePoint-r17 and radius-r17)

Propoal 2: RAN2 revises the agreement to SIB31 is extended to include the reference location and distanceThresh.

Proposal 3: SIBxx is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled.

Proposal 4: The update to SIBxx should not affect the system information value tag and does not trigger system information modification procedure.

Proposal 5: For eMTC, UE acquires SIBxx in RRC idle state. The exact time of acquisition of SIBxx in idle state can be left for UE implementation.

Proposal 6: RAN2 to discuss whether UE needs to acquire the SIBxx before connection establishment/resumption/re-establishment.

Proposal 7: SIBxx contains optional epoch time and validity duration. If absent, the epoch time and validity duration in SIB31 can be used.

Proposal 8: For earth-fixed neighbor cell, start time and footprint is broadcast.

Proposal 9: For earth-moving neighbor cell, footprint is broadcast.

Proposal 10: In connected mode, when a neighbor cell comes under coverage, UE may start neighbor cell measurement.

# 4 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.