3GPP TSG-RAN WG2 Meeting #122bis	 Tdoc R2- 2304734

May 22 - 26, 2023, Incheon, Korea

Agenda Item:	8.7.4

Souce:	MediaTek Inc.

Title:	Handover Enhancement in Earth-moving Cells

Document for:		Discussion and decision

# 1 Introduction

The work item objective (WID) for Rel-18 enhancements in mobility aspects has been discussed in WG2 Meeting #121bis-e. This contribution carries on the discussion for the potential group-based handover solution in improving the significant signalling overhead due to the large number of UEs under the broad LEO-satellite spot beam with frequent cell (beam) switching required.  This contribution also continues the discussion on PCI unchanged scenario.

# 2 Discussion

## 2.1 Broadcast/ Group-cast Common Configuration

As suggested in previous meetings, group-based handover can be a method to reduce signalling overhead in LEO mobility, whereby UEs can be grouped by location or UE capability, etc, and common configuration i.e., servingCellConfigCommon is broadcast or group-cast to UEs in groups, as in Figure 1 below.

| No Group | Group |
| --- | --- |
| UE specific config common config broadcast: Target cell config will be the same for all UEs UE1 HO Target Cell specific config UE specific config UE2 HO Target Cell specific config | UE specific config common config broadcast UE1 HO Target Cell specific config UEs grouped, with group-cast config UE specific config UE2 HO Target Cell specific config |

Figure 1: Common configuration for HO with grouping

Given the large number of UEs within the satellite coverage, the gain in broadcast or group-cast signalling comes down to the amount of common optional and common UE specific configuration; and whether this gain outweighs the configuration required for broadcast and/or group-cast. Grouping mechanisms can be based on UE characteristics, such as UE traffic types to find more common UE specific configuration or can be based on other parameters such as UE location as handover times can be predicted due to the deterministic satellite trajectory.

Each group can have a RNTI for scheduling PDCCH, and the legacy PDCP can be used for security. The deterministic satellite ephemeris can be used to estimate the target cell and predict trigger handover timing. This, however, can result in many RACH collisions, especially in high load scenarios. A potential solution is for UEs to back off randomly before initiating random access to overcome the potential large number of RACH collisions. Further study is to be conducted for RACH collision mitigation.

Proposal 1: Group-based handover can help to reduce some NTN HO signalling overhead by broadcasting or group-casting some common messages such as servingCellConfigCommon. Grouping gain depends on the amount of signalling saved in common signalling versus the configuration required for grouping.

Proposal 2: Further study is to be conducted on exact UE grouping mechanisms. Potential solutions can use common UE characteristics, such as UE traffic type, or other parameters such as UE location.

Proposal 3: To overcome the potentially large number of RACH collisions, UEs can first back off randomly before initiating random access. Further study is to be conducted for RACH collision mitigation.

## 2.2 Conditional Handover Enhancements

Group-based handover can be further enhanced by applying group-based or broadcast signalling in conditional handover RRC messages to all UEs belonging to the same RNTI. CHO can make use of the deterministic satellite ephemeris information and pre-configure the target cell to the UE, as per Figure 2, very far ahead of the handover time because it is possible to predict when the satellite will arrive and when handover is required to take place. It may also be possible to pre-configure multiple subsequent HO instances.

| No Group | Group |
| --- | --- |
| UE specific config UE1 HO Target cell config pre-configured UE specific config UE2 HO Target cell config pre-configured | UE specific config UE1 HO Target cell config pre-configured UE2 HO Target cell config pre-configured common config broadcast/ group-cast UE specific config |

Figure 2: Conditional handover configuration with grouping

Proposal 4: LEO NTN can group-cast or broadcast Conditional Handover Command messages to the UEs in the group very far ahead of the handover time, based on the deterministic satellite ephemeris to predict when and where handovers may take place.

## 2.3 PCI Unchanged

In the #121bis-e meeting, an agreement has been made that:

“In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1”, with pending feedback from RAN1.

Figure 3: NTN Handover with same feeder link

This scenario is drawn in Figure 3 above. If the PCI is unchanged when UEs handover from SAT1 to SAT2, there is no change from the UE perspective, as if it is still connected to the same cell with the same PCI. However, during handover there will be a service interruption time, which should be minimized, due to the hard satellite switch with an indication to the UE that this handover target is a PCI unchanged cell. In addition, each UE is still required to re-synchronise with the new SAT to acquire new NTN-config such as timing and ephemeris information which may extend the service interruption.

Also pending feedback from RAN1 is the feasibility of PCI unchanged with soft-satellite switch. Using a soft-satellite switch mitigates the service interruption time at the expense of interference with two cells projecting on the same geographical area in the same SSB frequency with the same PCI.

Observation 1: PCI unchanged removes L3 mobility, which is trade-off with service interruption time. In addition, each UE is still required to re-synchronise with the new SAT to acquire new NTN-config such as timing and ephemeris information which may extend the service interruption.

Observation 2: Unchanged PCI with soft satellite switch mitigates service interruption at the expense of interference between two cells projecting on the same geographical area in the same SSB frequency with the same PCI.

# 4 Conclusions

Proposal 1: Group-based handover can help to reduce some NTN HO signalling overhead by broadcasting or group-casting some common messages such as servingCellConfigCommon. Grouping gain depends on the amount of signalling saved in common signalling versus the configuration required for grouping.

Proposal 2: Further study is to be conducted on exact UE grouping mechanisms. Potential solutions can use common UE characteristics, such as UE traffic type, or other parameters such as UE location.

Proposal 3: To overcome the potentially large number of RACH collisions, UEs can first back off randomly before initiating random access. Further study is to be conducted for RACH collision mitigation.

Proposal 4: LEO NTN can group-cast or broadcast Conditional Handover Command messages to the UEs in the group very far ahead of the handover time, based on the deterministic satellite ephemeris to predict when and where handovers may take place.

Observation 1: PCI unchanged removes L3 mobility, which is trade-off with service interruption time. In addition, each UE is still required to re-synchronise with the new SAT to acquire new NTN-config such as timing and ephemeris information which may extend the service interruption.

Observation 2: Unchanged PCI with soft satellite switch mitigates service interruption at the expense of interference between two cells projecting on the same geographical area in the same SSB frequency with the same PCI.

# 4 References

- RP-220953, Thales, NR NTN (Non-Terrestrial Networks) enhancements, WID, RAN#95e, June 2022.

- TR 38.821 “Solutions for NR to support non-terrestrial networks”