3GPP TSG-RAN WG2 #122	R2-2304758

Incheon, Korean, May 2023

Agenda Item:	7.15.5

Source:	OPPO

Title:	Discussion on SL-FR2 impact

Document for:	Discussion, Decision

Introduction

This paper will further discuss the RAN2 work on SL-FR2, during RAN2 #121bis, the following agreements have been made

Agreement:

For beam failure detection, reuse Uu design of timer + counter based mechanism as baseline, and R2 further study how SL beam failure is detected. FFS on Tx or Rx UE based manner.

Upon beam failure is detection, support BFR signaling exchange between peer UEs, and further study e.g., RLF declaration due to beam failure.

Discussion

The following table has listed the main RAN2 work in Uu beam management

Table 1 Summary of main RAN2 work in Uu beam management

|  | Uu work |
| --- | --- |
| Beamforming-based initial access (initial Beam-pairing) | Initial beam-pairing is done during RACH procedure. |
| Beam maintenance | DL beam measurement/report (L1): L1 RSRP/SINR of configured CSI-RS/SSB, CSI-RS/SSB is configured/activated by RRC+MAC CE DL beam indication: TCI state indication by RRC+MAC CE+DCI |
|  | UL beam indication: signal indication by RRC+ MAC CE |
| Beam failure detection and recovery | Beam failure detected for SpCell: RACH-based BFR for CFRA, MAC CE based BFR report for CBRA; Beam failure detected for SCell: MAC CE based BFR report; |
| Beam-based RRM | Cell-level + beam-level L3 measurement report |

Initial Beam-pairing

In Uu, the initial beam-pairing is achieved during RACH, for the DL Tx beam,

- Firstly, the UE selects one ‘good’ SSB, and the DL Tx beam is selected accordingly;

- Then UE transmits the corresponding PRACH associated with the selected good SSB;

- gNB can be aware of the selected beam based on the association between SSB and the received PRACH.

In Uu, the initial beam-pairing is achieved during RACH, relying on the association between PRACH and SSB.

While in sidelink, there is no RACH procedure, the unicast link is established according to the following PC5-S signaling exchange[1]:

Figure 1: Layer-2 link establishment procedure[1]

In PC5, the link establishment procedure relies on discovery and PC5-S signalling exchange, without RACH-like procedure.

Therefore, the RACH-based beam access cannot be copied directly in sidelink, the initial beam-pairing during PC5 unicast link establishment needs to be further considered.

Recently, RAN1 has made the following agreements on the initial beam pairing

Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, including at least the following steps and how to determine UE2: - UE1 sends reference signals via different transmit beams - Note: multiple reference signals transmissions (e.g. repetitions) from each of the beams can be studied - FFS when reference signals are sent - FFS applicable reference signal - UE2 measures the reference signals and determines a UE1 transmit beam and/or a UE2 receive beam - FFS:whether/how to determine a UE2 transmit beam - UE2 indicates to UE1 the determined UE1 transmit beam - FFS how to indicate the determined transmit beam, including its feasibility - UE1 and UE2 set up sidelink unicast link using the determined beam, following existing link establishment procedure. Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed during sidelink unicast link establishment - UE1 sends PSCCH/PSSCH that carries unicast link establishment message (e.g., DCR message) via different transmit beams - Note: multiple - indication) - FFS: how to map between each PSCCH/PSSCH and UE1 transmit beam - FFS: how UE2 determines UE1 transmit beam(s) and/or UE2 transmit/receive beam(s) - UE1 uses one of the indicated beam(s) to finish the remaining sidelink unicast link establishment procedure with UE2 - FFS: how UE1 determines one of the indicated beam(s) - FFS: use of additional reference signal or additional messages or additional measurement for efficient beam pairing. Agreement RAN1 can study the following candidate procedure where initial beam pairing starts after sidelink unicast link establishment between UE1 and UE2, including studying whether and in which cases initial beam pairing after sidelink unicast link establishment is feasible. - UE1 and UE2 set up sidelink unicast link, following existing link establishment procedure - FFS the beams used for unicast link establishment. - UE1 and/or UE2 configure the resources for beam sweeping and/or beam reporting - FFS details of resources configuration - UE1 and/or UE2 use the configured resources to transmit reference signals and determine a pair of transmit beam and receive beam based on beam sweeping. - FFS applicable reference signal(s) - FFS whether/how to indicate the determined beams between UE1 and UE2 - FFS difference between initial beam pairing (after sidelink unicast link establishment) and beam maintenance

And in Uu, the initial beam selection is based on SSB measurement, while in sidelink, which signal/quantity should be measured and used boils down to RAN1 design on beam-based reference signal and measurement solution, so RAN2 should wait for RAN1’s progress on this issue first.

For SL initial beam-pairing, RAN2 wait for RAN1 progress first.

Beam management

The beam management after initial beam-pairing in Uu includes the following aspects:

- UE measures the Tx beam of the gNB and reports L1 SINR/RSRP based on network configuration;

- gNB determines the Tx beam based on UE’s report and indicates DL beam to the UE.

Following Uu principle, similar mechanism can be considered in sidelink, e.g.,

- Rx UE performs beam measurement and reports the measurement results based on Tx UE configuration;

- Tx UE determines the Tx beam based on Rx UE’s report and indicates the Tx beam to the Rx UE.

However, in Uu beam management, the mechanism design was mainly done in RAN1 and RAN2 work was on some MAC CE (e.g., TCI state activation/deactivation MAC CE) content design based on RAN1 agreements.

In Uu beam management, the L1 beam measurement and indication were designed by RAN1, while RAN2 work was on MAC CE design (e.g., TCI state activation/deactivation, etc.) as triggered by RAN1.

And RAN1 has made the following agreements on beam management

Agreement Consider one or more of the following items as sidelink beam reporting contents for beam maintenance: - Beam indication (e.g., CRI) - L1-RSRP - FFS: L1-SINR or other reporting contents if necessary enhancement is identified. - Note: This does not preclude performing beam maintenance without any beam reporting Agreement The container(s) of sidelink beam reporting for beam maintenance is at least selected from the following options: - Option 1: SL PHY layer signal (e.g., PSFCH, SCI) - Option 2: SL MAC CE - FFS: PC5-RRC, Signaling over Uu link (e.g., UCI) Agreement For UE1 and UE2 in a unicast link, RAN1 to study the selection of both UE1’s transmit beam and UE2’s corresponding receive beam. - Note: this applies for both PSCCH/PSSCH transmission/reception and PSFCH transmission/reception.

Therefore, RAN2 can wait for RAN1 progress on the beam measurement/report (L1) and indication now.

For SL beam management (including L1 measurement report and beam indication), RAN2 waits for RAN1 progress.

Beam-based RRM measurement

In Uu link, the RRM measurement is mainly used for mobility management. UE performs beam and cell measurement reports based on network configuration, and network may use the measurement result to determine the mobility-related configuration.

While in SL, the RRM measurement is mainly used for power control, and whether the per-beam based RRM measurement report is needed or not is related to whether the per-beam based power control is needed. Power control is designed by RAN1, so whether the per-beam based RRM measurement report is needed is pending on RAN1 design on the power control in FR2.

For SL beam-based RRM measurement report, RAN2 waits for RAN1 progress.

Beam failure detection and recovery

Issue-1: Which UE to declare BFR

For the issue of whether the beam failure is detected per-directional or bi-directional, it is related to whether there is a coupling relationship between the beam of the 2 directions (i.e., UE1->UE2 and UE2->UE1),

- If there is no coupling relationship between the beams of the 2 directions (e.g., there is no beam correspondence, and thus the two directions are using different paths), considering in legacy sidelink the behavior is always per-direction defined, and without correspondence, the beam is a naturally directional concept, so it is straightforward to have the beam failure detection/recovery in a per-directional manner;

- If there is a coupling/correspondence relationship between the beams of the 2 directional (e.g., there is beam correspondence, and the two directions are using the same path), whether the beam failure detection and recovery are bi-directional (i.e., if the beam from UE1 to UE2 is detected as failed then the correspondence-beam from UE2 to UE1 can also be considered as failed), can be further discussed based on RAN1 conclusion on whether/how the correspondence is defined in sidelink.

So at least the former case needs to be supported, and whether to support the latter case is still FFS and pending further discussion in R1 and R2.

In last RAN1 meeting, they have made the conclusion that the BFI-based beam failure detection (Rx-based manner) is to be studied:

Agreement RAN1 is to study sidelink Beam Failure Recovery (BFR) mechanism at least for the scheme where SL BFI is triggered based on the measurement of reference signal for BFD (if supported), including - candidate beam(s) identification - FFS details on reference signals for candidate beam identification, including structure, procedure, timing. - sidelink BFR request (BFRQ), including resources, transmit and/or receive beams, container, timing, etc. - sidelink BFR response (BFRR), including container, procedure, timing, etc. - FFS applicability to the scheme where SL BFI is triggered based on SL HARQ feedback (if supported).

In Uu, BFD-RS set is configured for the UE to detect beam failure. And the detection is also an Rx behavior, i.e., the beam failure is detected by RS signaling monitoring/measurement.

BFR in Uu is more of an Rx behavior.

RAN1 has agreed on the studying of BFI-based beam failure detection/recovery mechanism.

As RAN1 has confirmed on the BFI in SL-FR2, beam failure also needs to be detected and we can reuse the Uu beam failure detection mechanism, i.e., introduce both a timer and a counter in sidelink for beam failure detection in a Rx UE based manner as in Uu.

For beam failure detection, Rx UE based manner as in Uu is adopted as baseline.

Issue-2: BFR Granularity

When beam failure is detected in Uu, serval solutions have been discussed to help the UE to recover from beam failure. For different carriers/cells, the UE behaviors upon consistent LBT failure are different,

- If beam failure happens on SpCell, UE performs RACH, and may trigger RLF if random access is not successful for the beam failure recovery;

- If beam failure happens on SCell and SpCell (only in case of CBRA), UE reports beam failure via MAC CE.

In Uu, the UE performs RACH upon beam failure on SpCell and triggers RLF if the random access is not successful.

In Uu, the UE reports BFR MAC CE upon beam failure on SCell and SpCell (only in case of CBRA).

And in multiple-TRP case, the BF is detected/recovered per TRP.

While for SL, considering there be no CA in FR2 spectrum, there is no need to discuss the different BFR mechanisms for different carriers (cells). But considering there may be multi-UC link for a UE, similar to Uu multi-TRP case, the BF can be detected per-UC link.

There is no CA in SL FR2 spectrum now, so no need to consider different SL BFR mechanisms for different carriers.

RAN2 to discuss SL beam failure is declared per-unicast-link.

Issue-3: What signaling to report BFR

And for the detailed solution for beam failure recovery on PC5 link, RAN2 has made the agreement on signalling exchange between the 2 UEs, RAN1 also made the agreement on BFRR/BFRQ message and the possible content /shape of BFRR/BFRQ.

RAN1 made the conclusion on BFRR/BFRQ message and the possible content / shape of BFRR/BFRQ.

For the signalling to carry BFR in Uu, MAC CE/MSG1/MSGA are used to carry the BFR request to the network,

- In R15, the BFR is only defined for Pcell, and Preamble is used to identify the candidate beam for beam recovery;

- In R16/17, the BFR is defined for both Pcell and Scell, and MAC CE is used to indicate the failed-beams and the candidate beams;

By following the WID guidance, the Uu mechanism can be reused, the question is which Uu mechanism to select, i.e., physical signalling or MAC CE. Both the 2 signalings seem work,

- For the MAC CE based solution, the benefit is both the beam failure declaration and recovery are handled by MAC;

- For the physical signalling based solution, it also works considering there is no CA in SL FR2, i.e., more similar to the BFR at Pcell only scenario in R15.

Therefore, it is proposed to wait for R1 decision for the signaling exchange between the 2 UEs.

RAN2 waits for RAN1 decision before adopting MAC CE to carry SL beam failure indication from Rx-UE to Tx-UE.

Issue-4: BFR failure handling

And for the FFS point on the RLF triggered by BFR failure, since it is coupled with beam failure recovery mechanism/signalling exchange which depends on RAN1 progress, it is also preferred to wait for RAN1 progress first.

RAN2 waits for RAN1 decision before making the conclusion on BFR failure will trigger RLF.

Issue-5: gNB Involvement in BFR

Based on the above analysis, the general procedure of BFR in SL FR2 can be as follows:

Figure 2 General BFR procedure in SL FR2

As seen in the above figure, the general procedure is

From the direction from UE1 to UE2:

- UE1 sends RS signalling to UE2 (the RS signalling is up to RAN1 design);

- UE2 performs RS signaling monitoring/measurement (measurement relies on RAN1 design);

- UE2 detects beam failure and triggers BFR when the BFR counter defined for the UC-link reaches the maximum value before the BFR timer expires;

- UE2 indicates the BF info to UE1 (e.g., via MAC CE), and UE2 can consider the BFR complete upon the successful transmission of the BF indication.

From the direction from UE2 to UE1:

A similar procedure as above with the change of direction.

At last, for the gNB involvement of SL BFR procedure, considering beam is a direction/location-related concept, which is dynamic and not known by the network, so the gNB involvement for BFR seems not necessary and RAN2 can prioritize the study of beam failure recovery of RRC_IDLE/INACTIVE/OOC UEs first.

RAN2 to prioritize the study on beam failure recovery of RRC_IDLE/RRC_INACTIVE/OOC UEs, and wait for RAN1 progress before deciding whether/how network should be involved for RRC_CONNECTED Tx/Rx UEs.

Conclusion

We have the following observations:

Observation 1	In Uu, the initial beam-pairing is achieved during RACH, relying on the association between PRACH and SSB.

Observation 2	In PC5, the link establishment procedure relies on discovery and PC5-S signalling exchange, without RACH-like procedure.

Observation 3	In Uu beam management, the L1 beam measurement and indication were designed by RAN1, while RAN2 work was on MAC CE design (e.g., TCI state activation/deactivation, etc.) as triggered by RAN1.

Observation 4	BFR in Uu is more of an Rx behavior.

Observation 5	RAN1 has agreed on the studying of BFI-based beam failure detection/recovery mechanism.

Observation 6	In Uu, the UE performs RACH upon beam failure on SpCell and triggers RLF if the random access is not successful.

Observation 7	In Uu, the UE reports BFR MAC CE upon beam failure on SCell and SpCell (only in case of CBRA).

Observation 8	There is no CA in SL FR2 spectrum now, so no need to consider different SL BFR mechanisms for different carriers.

Observation 9	RAN1 made the conclusion on BFRR/BFRQ message and the possible content / shape of BFRR/BFRQ.

We have the following proposals:

Proposal 1	For SL initial beam-pairing, RAN2 wait for RAN1 progress first.

Proposal 2	For SL beam management (including L1 measurement report and beam indication), RAN2 waits for RAN1 progress.

Proposal 3	For SL beam-based RRM measurement report, RAN2 waits for RAN1 progress.

Proposal 4	For beam failure detection, Rx UE based manner as in Uu is adopted as baseline.

Proposal 5	RAN2 to discuss SL beam failure is declared per-unicast-link.

Proposal 6	RAN2 waits for RAN1 decision before adopting MAC CE to carry SL beam failure indication from Rx-UE to Tx-UE.

Proposal 7	RAN2 waits for RAN1 decision before making the conclusion on BFR failure will trigger RLF.

Proposal 8	RAN2 to prioritize the study on beam failure recovery of RRC_IDLE/RRC_INACTIVE/OOC UEs, and wait for RAN1 progress before deciding whether/how network should be involved for RRC_CONNECTED Tx/Rx UEs.

Reference

[1] TS 23.304 Proximity based Services (ProSe) in the 5G System