3GPP TSG-RAN WG2 #122	R2-2304759

Incheon, Korea, May 2023

Agenda Item:	7.24.2

Source:	OPPO

Title:	Discussion on emergency cause value for SL Relay

Document for:	Discussion, Decision

Introduction

For the support of emergency for UE-to-Network Relaying, RAN2 has discussed the potential RAN2 impact and made the following agreements:

Agreements:

R2 assume no additional R2 impact for gNB to know the initiated service-type of remote UE, and to select proper relay UE serving the initiated service-type of remote UE.

R2 confirm R18 relay UE sets cause value for emergency service relaying as in Rel-17 for SL-RLC0 traffic.  FFS SL-RLC1 case for path switching.

Discussion

In R18 U2N relay, the dedicated RSC has been introduced for emergency service by SA2, and it will be used by the Relay UE and Remote UE during discovery and PC5 link establishment. And according to SA2 conclusion on cause value setting, if PC5 connection was requested using emergency RSC, then the U2N Relay sets the RRC Establishment cause to “emergency” when establishing an RRC connection from RRC_IDLE.

- If PC5 connection was requested using emergency RSC, then the 5G ProSe Layer-3 Relay sets the RRC Establishment cause to “emergency” when establishing an RRC connection from RRC_IDLE.

While in R17 RAN2 specification, the R17 U2N relay UE can only set the cause value as emergency when emergency cause is received from the SL-RLC0 message.

NOTE 2: In case the L2 U2N Relay UE initiates RRC connection establishment triggered by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.3.1a, the L2 U2N Relay UE sets the establishmentCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as establishmentCause if the same cause value is in the message received from the L2 U2N Remote UE via SL-RLC0.

RAN2 discussed the cause value setting issue in RAN2 #121bis, and discussed how to handle the mismatch between R18 SA2 spec and R17 RAN2 spec, i.e., in SA2 spec the relay set cause value to emergency based on RSC while in current RAN2 spec, the relay set cause value based on its implementation.

There is a mismatch between SA2 and RAN2 spec on the establishment cause value setting of the relay UE.

RAN2 made the conclusion that when the RRC connection establishment of the relay UE is triggered by SL-RLC0 message from remote UE, the Rel-17 mechanism (up to UE implementation) is reused in R18. But for the case when the RRC connection establishment of the relay UE is triggered by SL-RLC1 message from remote UE, there is no conclusion and there is a collision between RAN2 R17 spec and SA2 R18 spec,

- In RAN2, the relay UE cannot set the cause value to emergency in such case since there is no SL-RLC0 and “it can only set the emergency as establishmentCause if the same cause value is in the message received from the L2 U2N Remote UE via SL-RLC0” and there is no cause value in the SL-RLC1 message;

- In SA2, the relay UE shall set the cause value to emergency if Remote UE has requested emergency service over PC5 via the emergency-related RSC type.

The following 2 alternatives have been raised but not concluded due to lack of time

- Alt-1: follows Rel-17 spec (i.e., for SL-RLC1, relay UE would not set cause value as emergency);

- Alt-2: firstly ask S2 whether there is a requirement to set cause value as emergency for path-switch case before R2 decision.

For these 2 alternatives, Alt-2 is more reasonable considering Alt-1 is against SA2 specification and does not solve the issue, while RSC-based approach works in that case.

For SL-RLC0, RAN2 sends LS to SA2 to notify the RAN2 conclusions on maintaining R17 RAN2 specification.

For SL-RLC1, RAN2 discuss whether to follow SA2 conclusion, i.e, set cause value to emergency according to RSC. If no, RAN2 sends LS to SA2 to ask whether there is a requirement for R18 relay UE to set cause value as emergency for emergency service for SL-RLC1 traffic.

Approve the LS draft as in Appendix.

Conclusion

We have the following observations:

Observation 1	There is a mismatch between SA2 and RAN2 spec on the establishment cause value setting of the relay UE.

We have the following proposals:

Proposal 1	For SL-RLC0, RAN2 sends LS to SA2 to notify the RAN2 conclusions on maintaining R17 RAN2 specification.

Proposal 2	For SL-RLC1, RAN2 discuss whether to follow SA2 conclusion, i.e, set cause value to emergency according to RSC. If no, RAN2 sends LS to SA2 to ask whether there is a requirement for R18 relay UE to set cause value as emergency for emergency service for SL-RLC1 traffic.

Proposal 3	Approve the LS draft as in Appendix.

Reference

[1] TS 23.700 Proximity based Services (ProSe) in the 5G System

Appendix- Draft LS to SA2

3GPP TSG-RAN WG2 Meeting #122	 R2-230xxxx

Incheon, Korea, May 2023

Title:	[Draft] LS on emergency service for SL relay

Response to:

Release:	Rel-18

Work Item:	NR_SL_relay_enh-Core

Source:	OPPO [To be RAN2]

To:	SA2

Cc:

Contact person:	Bingxue Leng

lengbingxue@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN2 observes it is specified in TS 23.304 for R18 U2N relay that the relay UE shall use "Emergency" RRC Establishment Cause if the 5G ProSe Relay needs to establish RRC Connection when the 5G ProSe Remote UE has requested emergency service over PC5.

In R17 U2N relay, the relay UE sets cause value according to the following NOTE in TS 38.331:

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection establishment triggered by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.3.1a, the L2 U2N Relay UE sets the establishmentCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as establishmentCause if the same cause value is in the message received from the L2 U2N Remote UE via SL-RLC0.

RAN2 discussed the RRC establishment cause value setting of the relay UE in the following 2 cases:

- Case-1: When the RRC connection establishment of the relay UE is triggered by SL-RLC0 message from remote UE.

- Case-2: When the RRC connection establishment of the relay UE is triggered by SL-RLC1 message from remote UE (e.g., RRCReconfigurationComplete message during a path switching procedure),.

For Case-1, RAN2 made the conclusion that  R2 confirm R18 relay UE sets cause value for emergency service relaying as in Rel-17 for SL-RLC0 traffic.

R2 confirm R18 relay UE sets cause value for emergency service relaying as in Rel-17 for SL-RLC0 traffic.

For Case-2, [RAN2 agrees to check with S2 whether there is a requirement for R18 relay UE to set cause value as emergency for emergency service for SL-RLC1 traffic (i.e., path switch case). So the following Question is to check SA2 on this issue:

Question: Whether there is a requirement for R18 relay UE to set cause value as emergency for emergency service for SL-RLC1 traffic (i.e., path switch case)?]

2	Actions

To SA2:

ACTION: 	RAN2 respectfully asks SA2 to respond to the above Question and provide feedback on the other RAN2 agreement if there are any concerns.

3	Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #123	August 21 – August 25, 2023	Toulouse, FR

TSG RAN WG2 Meeting #123bis	October 9– October 13, 2023	Xiamen, CN