3GPP TSG RAN WG2 #122                                		    R2-2304771

Incheon, KR, May 22 – 26, 2023

Source:	CATT

Title:	Discussion on RAT-Dependent integrity

Agenda item:	7.2.3

Document for:	Discussion and Decision

Introduction

In this contribution, we continue to discuss the RAT-dependent positioning integrity with consideration of the following issues:

- Analysis on RAN1 reply LS;

- Open issues involve other groups;

- Signaling and procedures for RAT-Dependent integrity;

Discussion

On RAN1 LS analysis

The following information is provided within the RAN1 LS.

1. Overall Description: RAN1 would like to thank RAN2 for the LS R1-2302282/ R2-2302271 on error source distributions. RAN1 would like to provide the answers to the following questions: Q1: RAN2 would like to confirm with RAN1 on the agreement “RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.” regarding the distribution of error sources Reply to Question 1: From RAN1’s perspective, the RAN2 agreement “the error sources are overbounded by a Gaussian distribution” can be confirmed for the error sources listed in Table 6.1.1-2 in TR 38.859 if the error source follows a Gaussian distribution. Q2: RAN2 respectfully ask RAN1 to provide the parameters (e.g. mean and standard deviation) for the overbound Gaussian distribution. Reply to Question 2: Parameters for the overbound Gaussian distribution can be mean and standard deviation. From RAN1’s perspective, zero is a valid possible option for the mean value for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859. From RAN1 perspective, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality, rtd-Quality-r16) and uncertainty information (e.g., LocationUncertainty-r16) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859.

It is agreed that for error sources with Gaussian bounded distribution, the zero mean value is applied. As for its STD, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality, rtd-Quality-r16) and uncertainty information (e.g., LocationUncertainty-r16) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) .

Besides, RAN2 also agreed that for UE-based integrity, LMF needs to provide the error source distribution for TRP related error sources to UE. Considering the detailed signaling, the following options can be considered:

- Option 1: no need to signal the mean value for the error sources, which can be in default specified, and directly reuse the legacy IE rtd-RefQuality/rtd-Quality/TRP-LocationBounds to indicate the STD for TRP related error sources, in case the error source is bounded with Gaussian distribution.

- Option 2: explicitly provide the overbound Gaussian distribution, including both the mean and the STD value.

In general, both options can work. But from our perspective, option 2 is slightly preferred, otherwise there may be some backward compliable issue, e.g., whether the assistance data is only used for positioning, or also used for integrity.

Proposal 1: For UE-based integrity, RAN2 to agree to use new parameters for providing the TRP related error sources overbounded with Gaussian distribution.

Further based on RAN1 agreements, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) for the overbound Gaussian distribution. And for LMF-based integrity, there is work assumption that the error source is based on LMF implementation.  Since there is already such information in the current LPP provide location message, so we think it is enough for LMF to decide the error sources based on such information. Based on this, we would like to confirm that

Proposal 2: RAN2 to confirm the work assumption that “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.”.

Open issues involve other groups

For the integrity calculation for GNSS, the following equation is used, where there is Residual Risk.

P(Error > Bound for longer than TTA | NOT DNU) <= Residual Risk + IRallocation               (Equation 5.3.x.2a-1)

In GNSS, there is the observation station, which can provide the distribution of the Residual Risk. But for RAT-dependent integrity, as we already agreed, the error sources are depend on the implementation, so it is not clear whether the Residual Risk is still needed or not. Also correlation time is used for GNSS integrity, which indicates the minimum time interval beyond which two sets of GNSS assistance data parameters for a given error can be considered to be independent from one another. It is also not clear whether such correlation time should be reused for RAT-dependent integrity. Further since the error sources is indeed within RAN1 scope, so it is proposed that RAN2 to further confirm with RAN1 on these issues.

Proposal 3: RAN2 to confirm with RAN1 on the following open issues:

- Whether there is need to define the Residual Risk for RAT-dependent integrity, and if needed, for which error sources it applies;

- Whether the correlation time need to be reused for RAT-dependent integrity.

Signalling and procedures for RAT-Dependent integrity

For GNSS, UE calculates the PL based on integrity assistance data provided by LMF. The assistance data for GNSS includes the error sources, and the IR. And in that case, the AL/TTA is not provided, with the assumption that the AL can be any random value. As for RAT-dependent integrity, same principle can be reused. So it is proposed that.

Proposal 4: For UE-based integrity, no need to provide the AL/TTA to UE.

If UE can calculate the PL based on corresponding integrity assistance data, UE shall reply the corresponding PL and may also include the TIR to LMF. However, in case UE cannot calculate the PL based on integrity assistance data, some error causes as in legacy should be provided to LMF, so that LMF can aware the reason why the integrity fails.

Proposal 5: For UE-based integrity, RAN2 to agree to introduce new error cause dedicated for integrity calculation in the case that there is no integrity result provided to LMF.

For LMF-based integrity, it is LMF to calculate the positioning integrity based on TRP related error sources and/or the measurement error sources. However, as discussed in P1/P2, since the measurement error sources and the TRP related error sources are already known to LMF due to implementation, thus from RAN perspective, there is not any additional spec impacts to support the LMF-based integrity except the integrity capability coordination between UE and LMF.

Proposal 6: For UE-based and LMF-based integrity, UE need to provide the integrity capability to LMF per positioning method.

Conclusions

In this contribution, we discussed the solutions for the integrity of RAT dependent positioning techniques. Based on the discussion, we have the following proposals:

On RAN1 LS analysis

Proposal 1: For UE-based integrity, RAN2 to agree to use new parameters for providing the TRP related error sources overbounded with Gaussian distribution.

Proposal 2: RAN2 to confirm the work assumption that “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.”.

Open issues involve other groups

Proposal 3: RAN2 to confirm with RAN1 on the following open issues:

- Whether there is need to define the Residual Risk for RAT-dependent integrity, and if needed, for which error sources it applies;

- Whether the correlation time need to be reused for RAT-dependent integrity.

Signalling and procedures for RAT-Dependent integrity

Proposal 4: For UE-based integrity, no need to provide the AL/TTA to UE.

Proposal 5: For UE-based integrity, RAN2 to agree to introduce new error cause dedicated for integrity calculation in the case that there is no integrity result provided to LMF.

Proposal 6: For UE-based and LMF-based integrity, UE need to provide the integrity capability to LMF per positioning method.

References

- TR 38.859, Study on expanded and improved NR positioning