3GPP TSG-RAN WG2 Meeting #122	R2-2304788

22 May – 26 May, 2023, Incheon

Agenda Item	: 7.15.2 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on SL consistent LBT failure and LCP impact

Document for	: Discussion and Decision

1.	Introduction

This contribution presents our views on consistent LBT failure and LCP for SL-U.

2.	Discussion

2.1 Shared COT & LCP

RAN2 made the following agreements on shared COT and LCP in #121bis-e meeting.

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact. If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

RAN2 decided to introduce enhanced LCP so that UE could benefit from type 2 LBT. In order for the UE to use type 2 LBT, CAPC restriction as well as destination restriction should be allowed in the enhance LCP. In other words, SL enhanced LCP can restrict multiplexing any MAC SDU(s) of LCH(s) with CAPC value higher than the CAPC value indicated in the COT sharing information.

Observation 1. In order for the UE to use type 2 LBT, CAPC restriction as well as destination restriction should be allowed in the enhance LCP.

Proposal 1. If destination restriction allowed by COT is satisfied, responding UE shall not include any MAC SDU(s) of LCH(s) having CAPC value higher than the CAPC value indicated in the COT sharing information.

When it comes to the FFS point (i.e., “FFS on the condition for UE to use enhanced LCP.”), the agreement already contains conditions (i.e., “1) If the resource to be used is within a shared COT, 2) if PDU not generated before COT arrival, 3) if there is data in buffer satisfying COT requirement”) under which the enhanced LCP can be used. We don't think we need any additional conditions other than this.

Proposal 2. RAN2 does not pursue to define additional conditions other than the conditions (i.e., “1) If the resource to be used is within a shared COT, 2) if PDU not generated before COT arrival, 3) if there is data in buffer satisfying COT requirement”) of the RAN2 agreement to use enhanced LCP.

2.2 LBT failure on RB set which is shared between uplink transmission and sidelink transmission

Physical resources of Uplink BWP and SL BWP may overlap each other. In other words, an RB set can be shared for uplink transmission and sidelink transmission. In this case, when LBT failure for uplink transmission occurs in the shared RB set, whether or not the UE should count SL LBT failure is unclear. From the perspective of not using the RB set in which the LBT failure occurred, even if Uu LBT failure is detected in the shared RB set, counting the SL LBT failure detection is a desirable UE behavior from a system performance point of view. Therefore, RAN2 can send LS to RAN1 to check whether the physical layer reports the SL LBT failure indication to the MAC layer when the Uu LBT failure is detected in the shared RB set.

Observation 2. An RB set can be shared for uplink transmission and sidelink transmission.

Observation 3. When LBT failure for uplink transmission happens in the shared RB set, whether or not the UE should count SL LBT failure is unclear.

Proposal 3. RAN2 sends LS to RAN1 to check whether the physical layer reports the SL LBT failure indication to the MAC layer when the Uu LBT failure is detected in the shared RB set.

3.	Conclusion

This contribution presented consistent LBT failure and LCP for SL-U, which can be summarized as follows:

Observation 1. In order for the UE to use type 2 LBT, CAPC restriction as well as destination restriction should be allowed in the enhance LCP.

Proposal 1. If destination restriction allowed by COT is satisfied, responding UE shall not include any MAC SDU(s) of LCH(s) having CAPC value higher than the CAPC value indicated in the COT sharing information.

Proposal 2. RAN2 does not pursue to define additional conditions other than the conditions (i.e., “1) If the resource to be used is within a shared COT, 2) if PDU not generated before COT arrival, 3) if there is data in buffer satisfying COT requirement”) of the RAN2 agreement to use enhanced LCP.

Observation 2. An RB set can be shared for uplink transmission and sidelink transmission.

Observation 3. When LBT failure for uplink transmission happens in the shared RB set, whether or not the UE should count SL LBT failure is unclear.

Proposal 3. RAN2 sends LS to RAN1 to check whether the physical layer reports the SL LBT failure indication to the MAC layer when the Uu LBT failure is detected in the shared RB set.

4.	Reference

[1] #121bis-e report from session on NR SL, OPPO