3GPP TSG-RAN WG2 Meeting #122	R2-2304793

22 May – 26 May, 2023, Incheon

Agenda Item	: 7.15.3 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on SL resource (re)selection and MCSt impact

Document for	: Discussion and Decision

1.	Introduction

This contribution presents our views on SL resource (re)selection and MCSt impact.

2.	Discussion

2.1 Type 1 LBT block issue of intra-UE

In #121 meeting, RAN2 discussed Type 1 LBT blocking issue. Based on the discussion, RAN2 made the following agreement. Besides, RAN2 sent LS to RAN1 to check that RAN2's understanding of Type 1 LBT blocking issues is correct.

RAN2#121 Agreement RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case). RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case). Will send LS to RAN1 to check if there is any concern.

Based on the LS, RAN1 discussed Type 1 LBT blocking issue and made following agreements including options to resolve the Type 1 LBT blocking issue.

RAN1#112bis-e Agreement To resolve the Type 1 LBT blocking issue, where one UE performing a Type 1 LBT procedure for using its own selected/reserved resource(s) is blocked by another UE’s SL transmission at least in a slot preceding to the selected/reserved resource and causing the LBT to fail, further study the following options in a future meeting. - Option 1: - UE avoid selection of N consecutive resource(s) before a reserved resource with high priority when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource. - UE avoid selection of N consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection - FFS: whether / how to achieve this in RA mode 1 - FFS: How to determine value of N - Option 2: - UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource). - UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the COT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource). - FFS whether / how to achieve this in RA mode 1. - Option 3: UE selects extra / more resources than required for transmitting a TB (i.e., overbooking) to accommodate potential Type 1 LBT failures. FFS how to determine/preconfigure the number of extra selected resources. - Option 4: The expected LBT duration is determined firstly, then resource selection takes into account of the expected LBT duration is performed. - Option 5: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection. - Option 6: UE excludes frequency resources (if any) previously reserved via SCI by other SL UEs in the corresponding slot, when estimating the detected power within a sensing slot duration in Type 1 channel access. - Option 7: SL UE deems channel busy only if the UE detects transmission other than SL transmission occupying the channel (e.g., exceeding the energy detection threshold), i.e., the energy detection for EDT checking in LBT procedure does not take into account the energy from SL transmissions. - Option X: No solution is needed. To avoid inter-UE blocking from performing Type 1 LBT can be handled based on UE implementation (e.g., as the start timing to perform LBT sensing is determined by each UE).

RAN2 can further discuss which option is the most valid option to solve Type 1 LBT blocking in the Intra-UE case among the options of the RAN1 agreements above. We prefer option 1 as a solution to solve the Type 1 LBT blocking in the intra-UE case. Also, based on option 1, we prefer a solution where the MAC layer performs resource selection to resolve the Type 1 LBT blocking issue. Because RAN2 has already agreed that the MAC layer performs resource (re-)selection to prevent Type 1 LBT blocking issue in the intra-UE case. Also, when it comes to the FFS point of option 1, the N value can be set in advance from gNB or pre-configured. Whether the N value can be set according to sl-priority or CAPC priority, etc. can be further discussed.

Observation 1. RAN2 can further discuss which option is the most valid option to solve Type 1 LBT blocking in the Intra-UE case among the options of the RAN1#112bis-e’s agreements.

Proposal 1. RAN2 adopts option 1 of the RAN1 agreement (i.e., RAN1#112bis-e meeting’s agreement on Type 1 LBT blocking issue) as a solution to prevent Type 1 LBT blocking in the Intra-UE case.

Proposal 2. N value of option 1 (i.e., option 1 of RAN1#112bis-e meeting’s agreement on Type 1 LBT blocking issue) can be set in advance from gNB or pre-configured. Whether the N value can be set according to sl-priority or CAPC priority, etc. can be further discussed.

2.2 MCSt impact on LBT failure

In #120 meeting, RAN2 discussed resource (re)selection due to an LBT failure. Based on the discussion, RAN2 made the following agreement.

RAN2#120 Agreement RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

RAN2 can further discuss whether UE triggers a resource (re-)selection when PSSCH transmission was not performed due to an LBT failure in MCSt case or not. In conclusion, we prefer that in MCSt, the UE does not trigger resource (re-)selection even if PSSCH transmission is not performed due to an LBT failure. Because, for non-MCSt case, RAN2 agreed on UE behaviour that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure for the purpose of giving the UE more transmission opportunities. However, the main purpose of the MCSt is to reduce the Type 1 LBT overhead. If it allows the UE to trigger resource reselection when the PSSCH transmission was not performed due to the LBT failure, Type 1 LBT overhead can be increased which does not meet the purpose of the MCSt design. Therefore, in MCSt case, we prefer UE behavior that UE does not trigger resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

Observation 2. For non-MCSt case, RAN2 agreed on UE behaviour that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure for the purpose of giving the UE more transmission opportunities.

Observation 3. The main purpose of the MCSt is to reduce the Type 1 LBT overhead. If it allows the UE to trigger resource reselection when the PSSCH transmission was not performed due to the LBT failure, Type 1 LBT overhead can be increased which does not meet the purpose of the MCSt design.

Proposal 3. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

2.3 MCSt resource (re-)selection

In RAN1#112bis-e, RAN1 has discussed MCSt resource (re-)selection and made the following approaches to implement/achieve MCSt for SL-U communication.

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA).

In conclusion, from the MAC layer procedure point of view, we prefer approach 2 based resource (re-)selection to guarantee Multi-Consecutive Slot transmission (MCSt) of UE. In other words, a solution in which the MAC layer delivers “number of slots for MCSt” to the Physical layer to ensure the MCSt of the UE is preferred. In addition, approach 2 is preferred over approach 3 in terms of minimizing the impact of the legacy resource selection procedure of MAC layer.

Observation 4. In terms of minimizing the impact of MAC's legacy resource (re-)selection procedure and guaranteeing the MCSt of UE, approach 2 is preferred among the discussions on RAN1's ​​MCSt resource (re-)selection.

Proposal 4. RAN2 considers approach 2 among RAN1's discussion results to implement/achieve MCSt for SL-U communication from the MAC layer perspective.

2.4 Shared COT resource (re-)selection

When the UE received a shared COT from a peer UE, if the generated sidelink grant is within the shared COT and there is logical channel data that satisfies the COT requirement, the UE can transmit sidelink data by performing Type 2 LBT within the shared COT.

In order to ensure the UE's Type 2 LBT gain, we can consider a new trigger condition for the resource (re-)selection when the generated sidelink grant does not belong to the shared COT. For example, a UE can trigger resource reselection to select a new resource(s) within the shared COT when the generated sidelink grant does not belong to the shared COT.

Observation 5. In order to ensure the UE's Type 2 LBT gain, we can consider a new trigger condition for the resource (re-)selection when the generated sidelink grant does not belong to the shared COT.

Proposal 5. A UE can trigger resource reselection to select a new resource(s) within the shared COT when the generated sidelink grant does not belong to the shared COT.

3.	Conclusion

This contribution presented our views on SL resource (re)selection and MCSt impact, which can be summarized as follows:

Observation 1. RAN2 can further discuss which option is the most valid option to solve Type 1 LBT blocking in the Intra-UE case among the options of the RAN1#112bis-e’s agreements.

Proposal 1. RAN2 adopts option 1 of the RAN1 agreement (i.e., RAN1#112bis-e meeting’s agreement on Type 1 LBT blocking issue) as a solution to prevent Type 1 LBT blocking in the Intra-UE case.

Proposal 2. N value of option 1 (i.e., option 1 of RAN1#112bis-e meeting’s agreement on Type 1 LBT blocking issue) can be set in advance from gNB or pre-configured. Whether the N value can be set according to sl-priority or CAPC priority, etc. can be further discussed.

Observation 2. For non-MCSt case, RAN2 agreed on UE behaviour that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure for the purpose of giving the UE more transmission opportunities.

Observation 3. The main purpose of the MCSt is to reduce the Type 1 LBT overhead. If it allows the UE to trigger resource reselection when the PSSCH transmission was not performed due to the LBT failure, Type 1 LBT overhead can be increased which does not meet the purpose of the MCSt design.

Proposal 3. When it comes to the MCSt, RAN2 does not pursue the UE behavior that UE triggers resource (re-)selection when the PSSCH transmission was not performed due to the LBT failure.

Observation 4. In terms of minimizing the impact of MAC's legacy resource (re-)selection procedure and guaranteeing the MCSt of UE, approach 2 is preferred among the discussions on RAN1's ​​MCSt resource (re-)selection.

Proposal 4. RAN2 considers approach 2 among RAN1's discussion results to implement/achieve MCSt for SL-U communication from the MAC layer perspective.

Observation 5. In order to ensure the UE's Type 2 LBT gain, we can consider a new trigger condition for the resource (re-)selection when the generated sidelink grant does not belong to the shared COT.

Proposal 5. A UE can trigger resource reselection to select a new resource(s) within the shared COT when the generated sidelink grant does not belong to the shared COT.

4.	Reference

[1] #121bis-e report from session on NR SL, 	OPPO