3GPP TSG-RAN WG2 Meeting #122	R2-2304794

22 May – 26 May, 2023, Incheon

Agenda Item	: 7.15.4 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on SL-U others

Document for	: Discussion and Decision

1.	Introduction

This contribution presents our views on RAN2 related remaining issues for SL-U.

2.	Discussion

2.1 sl-drx-HARQ-RTT-Timer

RAN2 made the following agreement in #121 meeting. And, we think that RAN1 already agreed to support multiple PSFCH occasions per PSCCH/PSSCH. Therefore, RAN2 can confirm both the working assumptions.

Working assumption: If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback. If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

Proposal 1. RAN2 confirms the following working assumptions regarding sl-drx-HARQ-RTT-Timer:

- If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.

- If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

2.2 Shared COT & DRX active time

At the RAN1#112 meeting, RAN1 agreed to support additional IDs to identify UE that can use shared COT.

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated

That is, UEs identified with an additional ID included in the shared COT can use the shared COT. It is advantageous to consider shared COT as DRX active time so that the UE identified with the additional ID can perform data transmission at the active time of the COT initiating UE. For example, the additional UE ID may indicate a UE having data to be transmitted to the COT initiating UE. So, it is desirable for the COT initiating UE to consider the shared COT to the active time in order to monitor data to be transmitted by the UE indicated by the additional ID. For another example, since the additional UE ID may be a UE that does not have unicast session with the COT initiating UE, UE indicated by the additional UE may not know the DRX active time of the COT initiating UE. If the shared COT is considered as DRX active time, the UE identified by the additional ID can determine the shared COT as the DRX active time of the COT initiating UE and transmit data at the active time of the COT initiating UE.

As examples of additional identity use cases, there are advantages to considering the shared COT as the DRX active time, so RAN2 should postpone confirming this working assumption (“Not define shared COT as SL DRX active time.”) until RAN1 completes the discussion of the additional IDs.

Also, if there is no inactivity timer and retransmission timer running between the COT responding UE and the COT initiating UE, the two UEs should rely only on the onduration timer for DRX active time.  For example, in this case, the COT Responding UE can extend its own DRX active time by starting an inactivity timer when receiving the shared COT in the onduration. However, if the COT initiating UE does not consider shared COT as DRX active time, it can only use onduation as the DRX active time. That is, even if the COT Responding UE has data to send in the extended DRX active time, the COT Initiating UE is dependent only on the onduation, so it should perform an inefficient behavior of receiving data in the next onduation. If the COT initiating UE considers the shared COT as additional DRX active time, the COT responding UE can transmit data to the COT initiating UE during the extended DRX active time. As such, the UE behavior of considering the shared COT as DRX active time has advantages in many aspects. Therefore, RAN2 needs to reconsider RAN2's decision (“Not define shared COT as SL DRX active time.”).

Observation 1. At the RAN1#112 meeting, RAN1 agreed to support additional IDs to identify UE that can use shared COT. UEs identified with an additional ID included in the shared COT can use the shared COT. It is advantageous to consider shared COT as DRX active time so that the UE identified with the additional ID can perform data transmission at the active time of the COT initiating UE. For example, the additional UE ID may indicate a UE having data to be transmitted to the COT initiating UE. So, it is desirable for the COT initiating UE to consider the shared COT to the active time in order to monitor data to be transmitted by the UE indicated by the additional ID. For another example, since the additional UE ID may be a UE that does not have unicast session with the COT initiating UE, UE indicated by the additional UE may not know the DRX active time of the COT initiating UE. If the shared COT is considered as DRX active time, the UE identified by the additional ID can determine the shared COT as the DRX active time of the COT initiating UE and transmit data at the active time of the COT initiating UE.

Observation 2. If there is no inactivity timer and retransmission timer running between the COT responding UE and the COT initiating UE, the two UEs should rely only on the onduration timer for DRX active time.  For example, in this case, the COT Responding UE can extend its own DRX active time by starting an inactivity timer when receiving the shared COT in the onduration. However, if the COT initiating UE does not consider shared COT as DRX active time, it can only use onduation as the DRX active time. That is, even if the COT Responding UE has data to send in the extended DRX active time, the COT Initiating UE is dependent only on the onduation, so it should perform an inefficient behavior of receiving data in the next onduation. If the COT initiating UE considers the shared COT as additional DRX active time, the COT responding UE can transmit data to the COT initiating UE during the extended DRX active time. As such, the UE behavior of considering the shared COT as DRX active time has advantages in many aspects. Therefore, RAN2 needs to reconsider RAN2's decision (“Not define shared COT as SL DRX active time.”).

Proposal 2. RAN2 should postpone confirming this working assumption (“Not define shared COT as SL DRX active time.”) until RAN1 completes the discussion of the additional IDs.

Proposal 3. UE behavior of considering the shared COT as active time has advantages in many aspects. Therefore, RAN2 needs to reconsider RAN2's decision (i.e., “Not define shared COT as SL DRX active time.”).

2.3 DTX & LBT failure of PSFCH transmission

In SL-U, RX UE may not be able to transmit PSFCH due to LBT failure. However, the TX UE cannot distinguish PSFCH transmission failure due to LBT failure. That is, in SL-U, a problem in which the TX UE frequently increases the DTX count may occur.

For this issue, RAN2 can discuss three options.

Option 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

Option 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur.

Option 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1.

Option 1 can prevent a frequent DTX count increasing behaviour of the TX UE, but it is difficult to correctly distinguish PSFCH transmission failure due to LBT failure of the RX UE. For example, the TX UE needs to know the SL-CAPC value related to the RX UE’s PSFCH transmission to perform LBT, but the TX UE cannot know this information.

In the case of Option 3, the TX UE can frequently increase the DTX count. However, the probability of LBT failure can be reduced by supporting multiple PSFCH occasions under consideration in RAN1. Multiple PSFCH opportunities provide more opportunities for PSFCH transmission, but does not solve the problem of not distinguishing between DTX and LBT failure of the PSFCH.

RAN2 determined the granularity of LBT indication as RB set. That is, when LBT occurs, the UE can continue transmission by using the resources of another RB set. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure dit not occur to help TX UE to distinguish the DTX and LBT failure of PSFCH. From this point of view, we prefer option 2 to option 1 and option 3.

Considering these points, RAN2 can discuss whether or how to support the TX UE procedure to distinguish PSFCH transmission failure due to the LBT failure of the RX UE.

Observation 3. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

Observation 4. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur. RAN2 determined the granularity of LBT indication as RB set. That is, when LBT occurs, the UE can continue transmission by using the resources of another RB set. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure dit not occur to help TX UE to distinguish the DTX and LBT failure of PSFCH.

Observation 5. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1. In the case of this option, the TX UE can frequently increase the DTX count. However, the probability of LBT failure can be reduced by supporting multiple PSFCH occasions under consideration in RAN1. Multiple PSFCH opportunities provide more opportunities for PSFCH transmission, but does not solve the problem of not distinguishing between DTX and LBT failure of the PSFCH.

Proposal 4. RAN2 can discuss whether to support the TX UE procedure to distinguish PSFCH transmission failure due to the LBT failure of the RX UE.

- Option 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

- Option 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur.

- Option 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1.

3.	Conclusion

This contribution presented remaining issues on SL-U, which can be summarized as follows:

Proposal 1. RAN2 confirms the following working assumptions regarding sl-drx-HARQ-RTT-Timer:

- If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.

- If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

Observation 1. At the RAN1#112 meeting, RAN1 agreed to support additional IDs to identify UE that can use shared COT. UEs identified with an additional ID included in the shared COT can use the shared COT. It is advantageous to consider shared COT as DRX active time so that the UE identified with the additional ID can perform data transmission at the active time of the COT initiating UE. For example, the additional UE ID may indicate a UE having data to be transmitted to the COT initiating UE. So, it is desirable for the COT initiating UE to consider the shared COT to the active time in order to monitor data to be transmitted by the UE indicated by the additional ID. For another example, since the additional UE ID may be a UE that does not have unicast session with the COT initiating UE, UE indicated by the additional UE may not know the DRX active time of the COT initiating UE. If the shared COT is considered as DRX active time, the UE identified by the additional ID can determine the shared COT as the DRX active time of the COT initiating UE and transmit data at the active time of the COT initiating UE.

Observation 2. If there is no inactivity timer and retransmission timer running between the COT responding UE and the COT initiating UE, the two UEs should rely only on the onduration timer for DRX active time.  For example, in this case, the COT Responding UE can extend its own DRX active time by starting an inactivity timer when receiving the shared COT in the onduration. However, if the COT initiating UE does not consider shared COT as DRX active time, it can only use onduation as the DRX active time. That is, even if the COT Responding UE has data to send in the extended DRX active time, the COT Initiating UE is dependent only on the onduation, so it should perform an inefficient behavior of receiving data in the next onduation. If the COT initiating UE considers the shared COT as additional DRX active time, the COT responding UE can transmit data to the COT initiating UE during the extended DRX active time. As such, the UE behavior of considering the shared COT as DRX active time has advantages in many aspects. Therefore, RAN2 needs to reconsider RAN2's decision (“Not define shared COT as SL DRX active time.”).

Proposal 2. RAN2 should postpone confirming this working assumption (“Not define shared COT as SL DRX active time.”) until RAN1 completes the discussion of the additional IDs.

Proposal 3. UE behavior of considering the shared COT as active time has advantages in many aspects. Therefore, RAN2 needs to reconsider RAN2's decision (i.e., “Not define shared COT as SL DRX active time.”).

Observation 3. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

Observation 4. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur. RAN2 determined the granularity of LBT indication as RB set. That is, when LBT occurs, the UE can continue transmission by using the resources of another RB set. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure dit not occur to help TX UE to distinguish the DTX and LBT failure of PSFCH.

Observation 5. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1. In the case of this option, the TX UE can frequently increase the DTX count. However, the probability of LBT failure can be reduced by supporting multiple PSFCH occasions under consideration in RAN1. Multiple PSFCH opportunities provide more opportunities for PSFCH transmission, but does not solve the problem of not distinguishing between DTX and LBT failure of the PSFCH.

Proposal 4. RAN2 can discuss whether to support the TX UE procedure to distinguish PSFCH transmission failure due to the LBT failure of the RX UE.

- Option 1. RAN2 consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE.

- Option 2. RAN2 can consider an assistance information of RX UE to distinguish between DTX and LBT failure of PSFCH transmission. That is, the RX UE may inform the TX UE of the number of PSFCH transmission failures due to LBT failure using an RB set in which LBT failure did not occur.

- Option 3. RAN2 does not consider an enhancement that TX UE can perform LBT to distinguish PSFCH transmission failure due to LBT failure of the RX UE. In other words, if the TX UE does not receive the PSFCH, the DTX count is increased by 1.

4.	Reference

[1] #121bis-e report from session on NR SL, 	OPPO