3GPP TSG-RAN WG2 Meeting #122	R2-2304798

22 May –  26 May, 2023, Incheon

Agenda Item	: 7.15.6 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on remaining issues of SL-CA enhancements

Document for	: Discussion and Decision

1.	Introduction

In this contribution, we introduce our views on unicast issues for SL-CA enhancements.

2.	Discussion

2.1 Carrier mapping for unicast SL CA

In #121bis-e meeting, RAN2 discussed carrier mapping for supporting a unicast communication in SL CA. Based on the discussion, RAN2 made the following agreement. Besides, RAN2 sent LS to SA2 to clarify RAN2's understanding of carrier mapping to unicast.

Proposal 1: Based on observation that section 6.1.2.12 of TS 24.587-v18.0.0 has captured V2X layer can be provisioned with service to frequency mapping for unicast. RAN2 assume it is applicable to PC5 unicast SL CA after link has been established. RAN2 notify SA2 this assumption and ask their input on identified questions. Proposal 3: RAN2 ask SA2 input on Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, how can the UE's AS layer determine the mapping between L2 ID and frequencies? Proposal 4: RAN2 ask SA2 input on Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies in the V2X layer?

In this contribution, we introduce our views on carrier mapping for unicast SL CA.

In LTE SL, only broadcast and groupcast are supported. To support LTE SL CA, the UE is configured by V2X layer with the carrier(s) which are allowed for a specific service based on the destination layer-2 ID (i.e. service to frequency mapping). To support NR unicast SL CA, if the UE is configured by V2X layer with the carrier(s) which are allowed for a specific unicast link based on a pair of source layer-2 ID and destination layer-2 ID (i.e. unicast link to frequency mapping), following problem can happen. New QoS model using PC5 QoS flows was introduced in NR V2X. In addition, in unicast, multiple QoS flows can be mapped to the same destination layer-2 ID. In other words, if destination layer-2 ID-based carrier mapping of LTE CA is applied to NR unicast SL CA, there may be happening a restriction that only one frequency should be mapped to services for multiple QoS flows on the same unicast link. This breaks the existing concept of the carrier mapping rule (i.e. service to frequency mapping) used for carrier configuration in the V2X layer. Therefore, in NR unicast SL CA, it is necessary to design a new carrier mapping rule suitable for QoS models based on PC5 QoS flow.

Observation 1. If destination layer-2 ID-based carrier mapping of LTE CA is applied to NR unicast SL CA, there may be happening a restriction that only one frequency should be mapped to services for multiple QoS flows on the same unicast link. This breaks the existing concept of the carrier mapping rule (i.e. service to frequency mapping) used for carrier configuration in the V2X layer.

Observation 2. In NR unicast SL CA, it is necessary to design a new carrier mapping rule suitable for QoS models based on PC5 QoS flow.

For example, if the V2X layer passes down radio frequency information mapped to unicast service or QoS flow (i.e., PFI) in the unicast link to the AS layer, the AS layer can select a carrier to be used in the unicast link based on the layer-2 ID (i.e., source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) among the radio frequency configuration to use for NR unicast transmission and reception. In other words, the AS layer can determine a radio frequency to be used in SL unicast by referring to QoS flow (e.g., PFI) information together, rather than determining a mapping radio frequency based on the destination layer-2 ID.

Proposal 1. RAN2 sends LS to SA2 asking whether selecting a radio frequency based on Layer 2 ID (i.e., pair of source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) is valid in terms of AS layer carrier selection.

Proposal 2. Assuming the V2X layer passes down radio frequency information mapped to the layer-2 ID (i.e., source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) in the unicast link to the AS layer, the AS layer selects carrier to use for NR unicast transmission and reception based on layer-2 ID (i.e., source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) of established PC5 unicast link (or PC5 RRC Connection).

One issue is how to map carriers for monitoring signals (e.g., PC5-S signalling, PC5 RRC signalling, MAC CE) without QoS flow. When it comes to the PC5-S message transmission, the simple solution is that AS layer uses radio frequencies received from V2X layer when the V2X layer provides PC5-S message to the AS layer.

Observation 3. Carrier mapping for messages (e.g., PC5-S message, PC5 RRC message, MAC CE) not related to service should be supported.

Proposal 3. Assuming the V2X layer provides one or more radio frequencies mapped to the V2X service type(s) for the Layer-2 link establishment when providing PC5-S message to the AS layer, the AS layer use the radio frequencies received from the V2X layer to transmit a PC5-S message (i.e., DCR, DCA) when the V2X layer provides PC5-S message to the AS layer.

When it comes to the transmission of PC5 RRC message and MAC CE, the UE can select a radio frequency configured through PC5 RRC Reconfiguration to transmit the PC5 RRC message and MAC CE.

When it comes to the reception of the message (e.g., PC5-S message, PC5 RRC message, MAC CE) without QoS flow, the UE can select the radio frequencies mapped with its own interested unicast service (e.g., PFI) as a radio frequency for the message (i.e., PC5-S message, PC5 RRC message, MAC CE) without QoS flow.

As another option, a default radio frequency to be used for transmission and reception of messages (e.g., PC5-S message, PC5 RRC message, MAC CE) not related to QoS flow can be configured gNB or pre-configuration.

Proposal 4. UE can select a radio frequency configured through PC5 RRC Reconfiguration to transmit the PC5 RRC message and MAC CE.

Proposal 5. For the reception of the messages (e.g., PC5-S message, PC5 RRC message, MAC CE) which is not related to QoS flow, the UE can select the radio frequencies mapped with its own interested unicast service (e.g., PFI) as a radio frequency for the message (i.e., PC5-S message, PC5 RRC message, MAC CE) without QoS flow.

Proposal 6. RAN2 can discuss whether it is necessary to define a default radio frequency to be used for the transmission and reception of messages (e.g., PC5-S message, PC5 RRC message, MAC CE) not related to QoS flow.

2.2 DTX based RLF enhancement in SL CA enhancements

Considering SL CA enhancements, it can be prioritized to reuse the LTE CA design as much as possible, but the NR V2X CA should not be designed to cause performance degradation to the NR V2X feature.

DTX-based RLF was introduced in NR V2X. RAN2 needs to check whether the existing DTX-based SL RLF works well in NR V2X CA environment. TX UE transmits the PSCCH/PSSCH to RX UE and the TX UE monitors the reception of PSFCH, and simultaneously can transmit PSFCH to other TX UEs that have established other unicast connections. That is, the TX UE may not be able to receive PSFCH due to prioritization between PSFCH transmission and PSFCH reception. If this problem occurs in NR V2X CA, since the TX UE cannot receive PSFCH from multiple carriers, multiple DTX is counted. This can cause the UE to quickly declare SL RLF. Therefore, RAN2 needs to check whether there are features that can cause the performance degradation of NR V2X communication when the NR V2X features are applied in NR V2X CA, as in the example.

Observation 4. Considering SL CA enhancements, it can be prioritized to reuse the LTE CA design as much as possible, but the NR V2X CA should not be designed to cause performance degradation to the NR V2X feature.

Observation 5. TX UE may not be able to receive PSFCH due to prioritization between PSFCH transmission and PSFCH reception. If this problem occurs in NR V2X CA, since the TX UE cannot receive PSFCH from multiple carriers, multiple DTX is counted. This can cause an UE to quickly declare SL RLF.

Proposal 7. RAN2 considers DTX based RLF enhancement to avoid frequent RLF declaration of a UE in SL CA enhancements.

3.	Conclusion

This contribution introduces our views on unicast issues for SL-CA enhancements, which can be summarized as follows:

Observation 1. If destination layer-2 ID-based carrier mapping of LTE CA is applied to NR unicast SL CA, there may be happening a restriction that only one frequency should be mapped to services for multiple QoS flows on the same unicast link. This breaks the existing concept of the carrier mapping rule (i.e. service to frequency mapping) used for carrier configuration in the V2X layer.

Observation 2. In NR unicast SL CA, it is necessary to design a new carrier mapping rule suitable for QoS models based on PC5 QoS flow.

Proposal 1. RAN2 sends LS to SA2 asking whether selecting a radio frequency based on Layer 2 ID (i.e., pair of source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) is valid in terms of AS layer carrier selection.

Proposal 2. Assuming the V2X layer passes down radio frequency information mapped to the layer-2 ID (i.e., source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) in the unicast link to the AS layer, the AS layer selects carrier to use for NR unicast transmission and reception based on layer-2 ID (i.e., source layer-2 ID and destination layer-2 ID) and QoS flow (i.e., PFI) of established PC5 unicast link (or PC5 RRC Connection).

Observation 3. Carrier mapping for messages (e.g., PC5-S message, PC5 RRC message, MAC CE) not related to service should be supported.

Proposal 3. Assuming the V2X layer provides one or more radio frequencies mapped to the V2X service type(s) for the Layer-2 link establishment when providing PC5-S message to the AS layer, the AS layer use the radio frequencies received from the V2X layer to transmit a PC5-S message (i.e., DCR, DCA) when the V2X layer provides PC5-S message to the AS layer.

Proposal 4. UE can select a radio frequency configured through PC5 RRC Reconfiguration to transmit the PC5 RRC message and MAC CE.

Proposal 5. For the reception of the messages (e.g., PC5-S message, PC5 RRC message, MAC CE) which is not related to QoS flow, the UE can select the radio frequencies mapped with its own interested unicast service (e.g., PFI) as a radio frequency for the message (i.e., PC5-S message, PC5 RRC message, MAC CE) without QoS flow.

Proposal 6. RAN2 can discuss whether it is necessary to define a default radio frequency to be used for the transmission and reception of messages (e.g., PC5-S message, PC5 RRC message, MAC CE) not related to QoS flow.

Observation 4. Considering SL CA enhancements, it can be prioritized to reuse the LTE CA design as much as possible, but the NR V2X CA should not be designed to cause performance degradation to the NR V2X feature.

Observation 5. TX UE may not be able to receive PSFCH due to prioritization between PSFCH transmission and PSFCH reception. If this problem occurs in NR V2X CA, since the TX UE cannot receive PSFCH from multiple carriers, multiple DTX is counted. This can cause an UE to quickly declare SL RLF.

Proposal 7. RAN2 considers DTX based RLF enhancement to avoid frequent RLF declaration of a UE in SL CA enhancements.

4.	Reference

- #121bis-e’s session chair note	OPPO