3GPP TSG-RAN WG2 Meeting #122 	R2-2304799

Incheon, Korea, 22th – 26th May, 2023

Title:		Discussion on LPHAP

Source:	Huawei, HiSilicon

Agenda item:	8.2.4

Document for:	Discussion and Decision

- Introduction

Within the WID RP-223549, the following objectives have been specified for LPHAP:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

This paper provides our thinking on the further enhancements on LPHAP.

- Enhanced SRS configuration for RRC_INACTIVE

2.1 TA Validation

In Rel-17, SRS configurations for positioning in RRC-INACTIVE is delivered to the target UE in RRCRelease message along with the RSRP change threshold and the TA timer. According to TS 38.321, the SRS transmission is only considered to be valid when the increase/decrease of the RSRP is within the threshold and the TA timer is running:

### 5.26.2 TA validation for SRS transmission in RRC_INACTIVE RRC configures the following parameters for validation for SRS transmission in RRC_INACTIVE: - inactivePosSRS-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall: 1> if the UE receives configuration for SRS transmission in RRC_INACTIVE: 2> store the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference as in TS 38.331 [5]. 1> else if the UE is configured with SRS transmission in RRC_INACTIVE: 2> if Timing Advance Command MAC CE is received as in clause 5.2, or; 2> if Timing Advance Command or Absolute Timing Advance Command is received for Random Access procedure that is successfully completed: 3> update the stored the RSRP of the downlink pathloss reference with the current RSRP value of the downlink pathloss reference. The MAC entity shall consider the TA to be valid when the following conditions are fulfilled: 1> compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference has not increased/decreased by more than inactivePosSRS-RSRP-ChangeThreshold, if configured; and 1> inactivePosSRS-TimeAlignmentTimer is running.

According to MAC spec, for Rel-17, the MAC entity starts or restarts the inactivePosSRS-TimeAlignmentTimer when the indication is received from the upper layer for starting the timer if the SRS is configured, or when there is ongoing Positioning SRS Transmission in RRC_INACTIVE and:

- A Timing Advance Command MAC CE is received, and if a NTA has been maintained with the indicated TAG; or

- A Time Advance Command is received in a RAR message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell and the Contention Resolution is considered successful for RA procedure; or

- An Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE;

The TA timer stops when the indication is received from the upper layer, which happens when the RRCSetup or RRCResume is received, or upon cell reselection. In addition, if the TA timer expires, the MAC entity would notify the RRC the release the positioning SRS configurations:

1> when the inactivePosSRS-TimeAlignmentTimer expires: 2> notify RRC to release Positioning SRS for RRC_INACTIVE configuration(s).

The following agreements made at RAN1 #112 [1]:

Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold

Thus, based on the RAN1 agreement, it is feasible to configure area-specific TA timer, as well as area-specific RSRP change threshold. RAN2 should discuss how to define the TA validation for SRS configuration with positioning validity area. Since for RSRP change threshold, RAN1 has not agreed on the reference RS, we can focus on the TA timer for this meeting on the three aspects on its relationship: (a) with SRS transmission, (b) with cell selection/reselection (c) and with SRS configuration.

First, on the area-specific timer and SRS transmission, from our understanding, area-specific TA means a single TA timer is associated with the SRS transmission in the uplink. The timer is started or restarted when the SRS is configured via RRC Release message. Besides, the timer should be started/restarted when TA command is received by RACH procedure or the TAC MAC CE is received via SDT. When the timer expires, the UE should not perform uplink transmission.

Proposal1: On the maintenance of the area-specific TA timer for a validity area:

- The area-specific TA timer starts/restarts when TA command is received

- When the are-specific TA timer expires, the TA is considered invalid and UE should stop performing SRS transmission

Next, on the area-specific timer and cell selection/reselection. According to TS 38.331, upon cell reselection, the TA timer stops and the SRS configuration is released. For LPHAP, if the UE moves within the area, the TA would not be stopped when cell reselection happens, and the positioning SRS configuration is considered to be uplink time aligned.

1> else if cell reselection occurs when srs-PosRRC-Inactive is configured: 2> indicate to the lower layer to stop inactivePosSRS-TimeAlignmentTimer; 2> release the srs-PosRRC-Inactive.

Proposal2: The area-specific TA timer is stopped if the UE moves out of the positioning validity area when the cell reselection happens.

Finally, on the relationship between area-specific TA timer and SRS configuration, as has been discussed above, the main usage of the timer is for controlling the SRS transmission in RRC_INACTIVE UL positioning when configured with a validity area. Hence, the SRS configuration does not need to be released as in legacy when the legacy TAT expires.

Proposal3: When the area-specific TAT expires, the UE does not release the SRS configuration associated with a validity area.

2.2	SRS Configuration Update

During the last RAN2 meeting, the following has been agreed on the SRS configuration request [2].

Agreement:

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

A remaining problem of last meeting is whether the SRS configuration request is in the RRC message or an accompanying MAC CE. The only necessity to perform such RACH procedure for SRS configuration request are:

(a) when the UE is within the validity area it is configured, TA update;

(b) when the UE is out of the validity area it is configured, TA update and SRS configuration update.

Then, for achieving the above purpose, it needs to be discussed what needs to be included in the RRC message or MAC CE. For the UE’s operation in RRC_INACTIVE, in order for the network to be able to identify the UE and authenticate the UE, clearly the I-RNTI and the resumeMAC-I need to be carried. Hence, we can reuse the legacy RRCResumeRequest message for the SRS configuration request, and a new resume cause value is needed. However, it is not clear what needs to be carried by the accompanying MAC CE. The network knows the cell where the UE performs SRS configuration update and the SRS configuration is configured by the network. Hence, there is no need to carry cell id/SRS configuration in the MAC CE. In addition, an accompany MAC CE will cause cell coverage issue for the msg3/msgA.

Based on the discussion above, we propose the following:

Proposal4: SRS configuration request is indicated by the legacy RRCResumeRequest message with a new cause value. An accompanying MAC CE is not needed.

2.3	Provision of SRS Configuration with positioning area

In the last RAN2 meeting, we have agreed that the SRS configuration within RRC_INACTIVE can be delivered to the UE by the legacy RRCRelease message [2].

Agreements:

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

To enable this mechanism of positioning SRS validity area, an issue to discuss is how the gNBs obtain the SRS configurations in the first place? Since the configurations are configured for multiple cells, the coordination between gNBs and/or between the gNB and the LMF may be required. A potential solution is that the LMF firstly collects the candidate SRS configurations for LPHAP from the gNBs. Then the LMF can recommend the SRS resources and the suggested positioning areas to the gNBs based on the collected information. Then the serving gNB can determine the SRS configuration and the validity area configuration, and respectively send to the UE and the LMF.

Figure 1, Generation of SRS configuration by TRP information exchange

Proposal5: Reuse the legacy non-UE associated NRPPa message TRP INFORMATION EXCHANGE for coordination of SRS configurations between gNBs and LMF.

In the R17 spec for UL positioning in RRC_INACTIVE, the SRS configuration is delivered to the UE in dedicated signaling by RRCRelease message. While for the requirement for LPHAP with its setup in the warehouse and assembly where the amount of UEs might be massive, the amount of overhead with dedicated signaling will also be huge if configuration is delivered to the UE individually. Also, the service of LPHAP is deterministic in nature and can be pre-defined. Hence, for LPHAP, we should think about efficient delivery of SRS configuration to the UE, e.g. via system information. For example, multiple SRS configurations can be broadcasted in the posSIB, or delivered to the UE with RRC message and each SRS configuration can be associated with an identity and the positioning areas. When an event is detected, a UE in RRC_INACTIVE state can send RRC messages to the gNB to request for the desired SRS configuration(s) among the multiple SRS configurations it has received.

Figure 2, Broadcast of SRS configuration with validity area by posSIB

Proposal6: Support the provision of SRS configuration with positioning area by system information.

2.4 Overall Stage-2 procedure

In summary, the following gives a detailed procedure for the SRS configuration with positioning validity area. It is proposed to adopt the procedure as the baseline for the discussion.

Proposal7: Adopt the following procedure as the baseline for the discussion on SRS configuration with validity area

Figure 3, SRS configuration request for UL positioning with validity area 1. LMF collects the candidate SRS configurations of the TRPs via TRP INFORMATION REQUEST message. 2. The gNBs sends the response with available SRS configurations via TRP INFORMATION RESPONSE message. 3a-4a. The LMF sends POSITIONING INFORMATION REQUEST message to request the serving gNB to configure the UE with SRS configurations and validity area. The LMF can suggest SRS configurations and the associated validity area to the serving gNB, then the serving gNB determines the configurations and sends to the LMF via POSITIONING INFORMATION RESPONSE message. 3b-4b. The LMF sends ASSISTANCE INFORMATION CONTROL to the gNB to send posSIB for the delivery of multiple SRS configurations with validity area. 5. The gNB provides SRS configuration to the UE via RRCRelease message or posSIB and it sends the UE to RRC_INACTIVE. 6. Event is triggered within the UE that the UE needs to initiate the positioning procedure with the network; or when cell reselection happens, the UE selects one cell out of the current positioning validity area. The UE needs to request the SRS configuration to the network; 7. The UE sends RRC Resume Request message to the newly selected gNB; 8. The network determines new SRS configuration and the associated validity area for the UE, the detailed procedures pending to RAN3. 9. The serving gNB sends RRCRelease message to the UE, configuring the UE with new SRS configuration; 10. LMF sends NRPPa message MEASUREMENT REQUEST to the gNBs where SRS receptions are expected; 11. UE performs SRS transmission according to the SRS configuration that the UE receives and the gNBs perform SRS reception; 12. The gNBs measure the SRS and sends measurement report to the LMF.

- Alignment between DRX and PRS

To summarize, there are three approaches for the DRX configuration in RRC_IDLE/INACTIVE to be delivered to the UE:

- Option 1, PCCH-Config in SIB1 for both RRC_IDLE and RRC_INACTIVE

- Option 2, PCCH-Config in RRCRelease message for RRC_INACTIVE when the UE enters RRC_INACTIVE

- Option 3, NAS message for both RRC_IDLE and RRC_INACTIVE received when the UE is in RRC_CONNECTED

When multiple DRX cycles are configured with the options above, the UE should choose the shortest cycle to monitor paging such that the UE does not miss any paging occasion, either from the gNB or from the AMF. Based on the discussions during WI, there are two solutions for the alignment of DRX and PRS configurations:

- Option 1. PRS alignment with fixed DRX

- Option 2. DRX alignment with fixed PRS

In this section, we discuss the solutions for these two approaches in detail.

3.1	PRS alignment with fixed DRX

For the case of aligning PRS with fixed DRX, UE-based on-demand PRS procedure introduced in Rel-17 can be reused to change the PRS configuration, e.g., periodicity and offset. The PRS alignment with fixed DRX can be discussed in the following two cases for UE in RRC_INACTIVE and RRC_IDLE.

3.1.1	Alignment of PRS and DRX in RRC_IDLE

For DRX configuration in RRC_IDLE, since the UE’s configuration is by NAS and SIB1, the UE knows the DRX configuration when it is in RRC_CONNECTED. Since for positioning in RRC_IDLE, the UE should have already obtained the PRS configuration in RRC_CONNECTED, it can know whether the PRS needs any adjustment. While, to realize the adjustment, the legacy UE-based on-demand PRS request can be performed. For this, no stage3 change is required.

Proposal8: For the PRS alignment with fixed DRX for UE in RRC_IDLE, the legacy UE-based on-demand PRS procedure can be sent in RRC_CONNECTED with no stage-3 impacts.

3.1.2	Alignment of PRS and DRX in RRC_INACTIVE

If the UE obtains the DRX configuration in RRC_CONNECTED, the PRS alignment with fixed DRX can be achieved by the UE sending LPP message for on-demand PRS request to the LMF. If the UE obtains the DRX configuration only when the UE is in RRC_INACTIVE, the UE can also send LPP message to the LMF for UE-based on-demand PRS request with SDT and SRB2 configured for SDT when the deferred MT-LR procedure is on-going.

With the discussion above, it can be seen that if we perform on-demand PRS request to change the PRS configuration with DRX fixed, the legacy mechanism for on-demand PRS request can be reused with no stage3 impacts.

Proposal9: For the PRS alignment with fixed DRX for UE in RRC_INACTIVE, the legacy UE-based on-demand PRS procedure can be sent in RRC_CONNECTED or RRC_INACTIVE by SDT with no stage-3 impacts.

3.2	DRX alignment with fixed PRS

For option 2, the DRX configuration is changed with the PRS configuration fixed. For the case of DRX configuration delivered by NAS message, currently, the UE can negotiate with the AMF for the NAS DRX configuration by the REGISTRATION REQEUST/RESPONSE, which needs to be enhanced to support the alignment with PRS. Another aspect is the alignment of RAN-generated DRX configurations with the PRS. Either the UE or the LMF can send the request and assistance information, e.g. PRS configurations of the neighbor cells, to the serving gNB for the alignment. Considering the large spec. impacts, it is proposed to deprioritize option2.

Proposal10. Deprioritize DRX alignment with fixed PRS for Rel-18.

4.	DL Positioning in RRC_IDLE

Based on the WID, the solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurement in RRC_CONENCTED state should be specified. There should be no technical issues if the measurements are reported to the LMF after security activation, which is already supported by the existing deferred MT-LR procedures, as shown in figure below from TS 23.273.

Figure 4, Deferred 5GC-MT-LR for periodic, triggered and UE available location events [23.273, figure6.3.1-1]

Observation1: The current spec can already support the measurement report in RRC_CONNECTED after security activation.

Proposal11: UE can perform the DL PRS measurements in RRC_IDLE state and report them in RRC_CONNECTED state to the LMF with the current SA2 stage2 procedure in Clause 6.3.1 in TS 23.273. Send a LS to SA2 to confirm the understanding.

5. Summary

Based on the discussion above, we make the following proposals:

SRS Enhancement:TA validation

Proposal1: On the maintenance of the area-specific TA timer for a validity area:

- The area-specific TA timer starts/restarts when TA command is received

- When the are-specific TA timer expires, the TA is considered invalid and UE should stop performing SRS transmission

Proposal2: The area-specific TA timer is stopped if the UE moves out of the positioning validity area when the cell reselection happens.

Proposal3: When the area-specific TAT expires, the UE does not release the SRS configuration associated with a validity area.

SRS Enhancement: SRS configuration Update

Proposal4 SRS configuration request is indicated by the legacy RRCResumeRequest message with a new cause value. An accompanying MAC CE is not needed.

SRS Enhancement: SRS configuration provisioning

Proposal5: Reuse the legacy non-UE associated NRPPa message TRP INFORMATION EXCHANGE for coordination of SRS configurations between gNBs and LMF.

Proposal6: Support the provision of SRS configuration with positioning area by system information.

SRS Enhancement: Overall stage2 procedure

Proposal7: Adopt the following procedure as the baseline for the discussion on SRS configuration with validity area

Alignment between PRS and DRX

Proposal8: For the PRS alignment with fixed DRX for UE in RRC_IDLE, the legacy UE-based on-demand PRS procedure can be sent in RRC_CONNECTED with no stage-3 impacts.

Proposal9: For the PRS alignment with fixed DRX for UE in RRC_INACTIVE, the legacy UE-based on-demand PRS procedure can be sent in RRC_CONNECTED or RRC_INACTIVE by SDT with no stage-3 impacts.

Proposal10: Deprioritize DRX alignment with fixed PRS for Rel-18.

DL Positioning in RRC_IDLE

Observation1: The current spec can already support the measurement report in RRC_CONNECTED after security activation.

Proposal11: UE can perform the DL PRS measurements in RRC_IDLE state and report them in RRC_CONNECTED state to the LMF with the current SA2 stage2 procedure in Clause 6.3.1 in TS 23.273. Send a LS to SA2 to confirm the understanding.

6.	References

- RAN1-112 Chair’s notes

- RAN2-121bis Chair’s notes

Annex A: General procedure for SRS configuration with validity area

Figure 3, SRS configuration request for UL positioning with validity area 1. LMF collects the candidate SRS configurations of the TRPs via TRP INFORMATION REQUEST message. 2. The gNBs sends the response with available SRS configurations via TRP INFORMATION RESPONSE message. 3a-4a. The LMF sends POSITIONING INFORMATION REQUEST message to request the serving gNB to configure the UE with SRS configurations and validity area. The LMF can suggest SRS configurations and the associated validity area to the serving gNB, then the serving gNB determines the configurations and sends to the LMF via POSITIONING INFORMATION RESPONSE message. 3b-4b. The LMF sends ASSISTANCE INFORMATION CONTROL to the gNB to send posSIB for the delivery of multiple SRS configurations with validity area. 5. The gNB provides SRS configuration to the UE via RRCRelease message or posSIB and it sends the UE to RRC_INACTIVE. 6. Event is triggered within the UE that the UE needs to initiate the positioning procedure with the network; or when cell reselection happens, the UE selects one cell out of the current positioning validity area. The UE needs to request the SRS configuration to the network; 7. The UE sends RRC Resume Request message to the newly selected gNB; 8. The network determines new SRS configuration and the associated validity area for the UE, the detailed procedures pending to RAN3. 9. The serving gNB sends RRCRelease message to the UE, configuring the UE with new SRS configuration; 10. LMF sends NRPPa message MEASUREMENT REQUEST to the gNBs where SRS receptions are expected; 11. UE performs SRS transmission according to the SRS configuration that the UE receives and the gNBs perform SRS reception; 12. The gNBs measure the SRS and sends measurement report to the LMF.