3GPP TSG-RAN WG2 Meeting #122	R2-2304820

Incheon, Korea, 22th – 26th May, 2023

Agenda item:	7.11.2.1

Source:	Huawei, Hisilicon

Title:	Multicast reception for RRC_INACTIVE UE

Document for:	Discussion

- Introduction

The following agreements were made during the discussion in RAN2#121bis meeting for service continuity and notifications for multicast reception in RRC_INACTIVE.

- Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection. - When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving. - UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality. - Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS. - No need to define a mechanism other than the frequency prioritization, i.e., per cell based prioritization in cell re-selection, to help UE choose the right cell to camp on. - The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast. - A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation - Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS. - For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data. - FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data. - No additional enhancement (with regard to enhancements made for ‘deactivation/temp no data’) is needed specifically for enabling UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release. - Legacy group paging (i.e., Rel-17 group paging) can be used to resume UE to RRC_CONNECTED state. - Upon events like session activation/data transmission resumed, if PTM configuration is not available to UE, UE initiates RRC connection resumption. - UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior). - When both enhanced group paging and unicast paging are received by the UE (and targeted for this UE), the UE follows unicast Paging and goes to RRC CONNECTED.

In this contribution, we will further analyze the remaining issues based on the progress on multicast reception in RRC_INACTIVE and also some issues related to the Stage-3 RRC CR.

- Discussion

2.1 PTM configuration and mobility aspects

UE resume due to bad reception quality

It has been agreed that UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold. Based on proposals from companies, the reception quality might be BLER or signal quality (RSRP/RSRQ/SINR). Considering that the measurement definition for signal quality is quite mature in NR specification and the relation between SINR and BLER is expectable, the threshold based on signal quality can be used to trigger such resumption to avoid the need of further introducing the measurement definition for BLER.

In addition, different gNBs might have different policy to trigger resume for bad quality UEs, i.e., the thresholds may be different in different cells. Therefore it is better to configure the threshold in MCCH rather than in RRC release message. Otherwise, after mobility, the UE will need to resume to receive the threshold from the new cell. Besides, it should be further discussed whether this threshold can be per service or service group to allow different policy for different services.

Proposal 1a: the threshold used to trigger RRC resumption is based on signal quality, i.e., configuration among RSRP, RSRQ, or SINR.

Proposal 1b: the threshold used to trigger RRC resumption is configured in MCCH.

Proposal 1c: further discuss whether the threshold is per service or per service group or per cell.

NCL for multicast

It has been agreed that NCL is supported for multicast reception in RRC_INACTIVE similar as MBS broadcast NCL. However, the NCL can only indicate whether there is ongoing RRC_INACTIVE multicast service in neighbour cells. The UE still has to read SIB and multicast MCCH after mobility. For intra-gNB case, it is very likely that the gNB will use exactly the same PTM configuration to provide the same multicast service in different cells. If this information is provided to the UE beforehand and the target cell is using the same PTM configuration as the source cell, the UE can reduce the interruption time to acquire SIB and multicast MCCH in the target cell during mobility. Note that this doesn’t mean to provide full PTM configuration of neighbor but just indicate whether the configuration is the same or not.

Proposal 2: NW should be able to provide the UE with the information on whether the same PTM configuration is being used in neighbour cells for multicast in RRC_INACTIVE.

Frequency prioritization

The frequency prioritization mechanism is beneficial for multicast reception in RRC_INACTVIE at least for the scenario where a multicast service is provided on the same frequency. But different from broadcast, there is no pre-planned area for multicast in RRC_INACTIVE, thus it is not possible to use the FSAI based solution as for MBS broadcast. Since the dedicated frequency priority can be provided via RRC dedicated signaling, we think this legacy mechanism can be reused for frequency prioritization for multicast in RRC_INACTIVE without introducing a new mechanism.

Proposal 3: legacy mechanism of providing the dedicated frequency priority in RRC dedicated signaling can be reused for multicast frequency prioritization in RRC_INACTIVE.

Resume cause and access category

Based on the email discussion in the last meeting, the UE needs to trigger RRC Resume in several scenarios when configured to receive multicast in RRC_INACTIVE including:

Case 1: UE receives group paging (without indication to stay in RRC_INACTIVE)

Case 2: the UE can’t acquire PTM configuration for an active MBS session after mobility

Case 3: the reception quality of MBS multicast session is bad

When triggering the RRC Resume for these cases, the UE needs to know which resume cause to use. For case 1, it is straightforward to use MT-Access and this would mean the access control is not performed in such case. However, for case 2 and case 3, no existing cause values can be mapped to such scenarios. There are two possible options for the resume cause setting in these scenarios:

Option 1: reuse MO-data resume cause and access category

Option 2: introduce new resume causes and access categories

We think option 2 should be used based on the reasons below:

Reason 1: the MO-data is triggered by NAS while MBS resume is triggered by AS. As NAS is not aware of the above two cases, it cannot trigger it. Besides, the UE behavior after having been barred are different for AS triggered resume and NAS triggered resume. Therefore, separate cause and access category should be used to make RRC procedure work and to allow different treatment for AS and NAS triggered resume.

Reason 2: for case 2, if NW can know from the corresponding resume cause that this is only triggered for MBS multicast reception and not due to other services that require the UE to move to RRC_CONNECTED, the NW can directly send RRC release in response to the RRC resume request to keep the UE in RRC_INACTIVE for multicast reception. For case 3, NW can know from the corresponding resume cause that the UE cannot receive multicast in RRC_INACTIVE any longer and let it resume.

Proposal 4a: introduce new MBS resume causes for cases where the UE needs to resume to continue multicast reception.

Then, if the resume procedure to continue multicast reception is barred or rejected, the UE’s AS layer should re-initiate the procedure when barring is alleviated similarly as RNA update procedure.

Proposal 4b: if the resume procedure to continue multicast reception is barred or rejected, the UE’s AS layer should re-initiate the procedure when barring condition is alleviated, similar as RNA update procedure

It has been discussed on how to enable a special (identified by MBS assistance information from 5GC) UE to resume to RRC_CONNECTED upon session activation. In our understanding, this would be supported naturally by stage 3 considering the UE needs to be anyway configured with something to enable the R18 feature of multicast reception in RRC_INACTIVE as shown below (e.g., multicastConfigInactive-r18 in current running CR). For the special UE, the network will not configure the new R18 configuration to the UE(i.e. is not allowed to receive MBS multicast in RRC_INACTIVE) and the UE will trigger resume after reception of group paging same as a legacy UE.

multicastConfigInactive-r18         SetupRelease { MulticastConfigInactive-r18 }                        OPTIONAL   -- Need M

MulticastConfigInactive-r18::=               SEQUENCE {

mbs-ServiceList-r18                 MBS-ServiceList-r18,

OPTIONAL -- Need N

servCellId-r17                      ServCellIndex,

OPTIONAL -- Need N

servingCellPTM-Config               OCTET STRING (CONTAINING MBSMulticastConfiguration),

OPTIONAL -- Need N

mcch-Config                         OCTET STRING (CONTAINING SystemInformation)

OPTIONAL-- Need N

}

Proposal 5: the UE capable of MBS multicast reception in RRC_INACTIVE should behave the same as the legacy R17 MBS UE if no R18 configuration for MBS multicast reception in RRC INACTIVE is configured in RRCRelease, i.e., trigger resume upon group paging for session activation.

PDCP COUNT Continuity

During the email discussion in the last meeting, the majority think the a UE in RRC_INACTIVE doesn't have to re-initialize the PDCP window each time when it moves to another cell, which will lead to unnecessary data loss. There is one issue left on how to indicate whether PDCP COUNT is synchronized in a cell.

In Rel-17, the PDCP COUNT synchronization among gNBs can be achieved by setting the PDCP COUNT based on the same MBS QoS Flow SN. In addition, the gNBs should also have the same QoS flow to MRB mapping, which is guaranteed based on NW implementation. Following the similar logic, the same QoS flow to MRB mapping can also be guaranteed by implementation. In this case, one cell can just indicate the UE whether it is synchronized or not. And if the UE receive the “synchronized” indication from two cells, it means the two cells are synchronized. Otherwise, the two cells are not synchronized. Here by "synchronized" it means the cell follows a common flow-to-MRB mapping rule by NW implementation and at the same time sets the PDCP COUNT according to the MBS QoS Flow SN.

Proposal 6a: One cell can indicate "synchronized", if by implementation, it follows a common QoS flow to MRB mapping rule and at the same time sets the PDCP COUNT according to the MBS QoS Flow SN.

Proposal 6b: The UE doesn’t need to re-initialize the PDCP window, if both the source cell and the target cell indicate “synchronized”.

Proposal 6c: The UE re-initializes the PDCP window similar as broadcast, if the source and the target cells are not synchronized.

2.2 Notification and state transition

Group paging for service activation

It has been agreed that the UE can stay in RRC_INACTIVE and start monitoring the corresponding G-RNTI upon an enhanced group paging. The straightforward way is to add one indicator per multicast service in group paging to enable the UE to stay in RRC_INACTIVE for multicast reception and the absence of this indicator means the UE is required to enter RRC_CONNECTED for multicast reception.

Proposal 7: add one indicator per multicast service in group paging upon session activation to enable the UE to stay in RRC_INACTIVE for multicast reception. The absence of this indicator means the UE is required to enter RRC_CONNECTED for multicast reception.

Notification for session deactivation

For the notification of session deactivation, there are two options to provide it, as below:

Option 1: indicate the session deactivation in paging message

For this option, the UE shall be notified in paging message with an additional TMGI list to avoid mistakenly invoke legacy UEs. This would increase the load for paging and would also introduce additional power consumption for legacy UE due to more frequent false alarms.

In addition, the UE may miss this notification in paging message in some cases such as cell reselection. Then the UE will not know the service is de-activated in such case and would still monitoring the G-RNTI causing more power consumption. Thus, it is not desirable to indicate the session deactivation in paging message.

Option 2: indicate the session deactivation in MCCH message

In R17, when MBS broadcast service stops, the UE is notified by MCCH change notification and the UE acquires MCCH to check whether the concerned service is stopped or not. Similar mechanism can be reused for multicast session deactivation, i.e. the UE is notified by MCCH-RNTI about session deactivation (the same bit as for session change notification can be reused), and UE can learn which service is deactivated by reading MCCH. In this option, given that MCCH is transmitted periodically, the UE can sync to target cell upon reception of MCCH of target cell even if the UE missed the change notification during cell reselection.

Proposal 8: Upon the deactivation of multicast session, MCCH message is used to notify the RRC_INACTIVE UE which is receiving this multicast session.

MCCH monitoring during session deactivation period

Since it has been agreed to use group paging for session activation, it seems better to allow the UE not to monitor MCCH during session deactivation period to save the UE power consumption in RRC_INACTIVE. The UE can start to monitor MCCH after receiving group paging to acquire the latest PTM configuration for the concerned service.

Proposal 9: The UE is not required to monitor MCCH-RNTI during session deactivation period and should start to monitor MCCH-RNTI after receiving group paging to acquire the latest PTM configuration for the concerned service.

2.3 Stage 3 RRC CR issues

RRC modelling for multicast reception in RRC_INACTIVE

It has been agreed that the PTM configuration to receive multicast in RRC_INACTIVE is configured (by dedicated signaling /MCCH) separately from the parameters used in the connected mode. Usually, the network needs to change some MRB configurations when the UE in RRC_INACTIVE is allowed to receive the same service, due to the absence of uplink for such UE and also for compliance to the minimal UE capabilities. For example, the network may need to change the configuration of header compression, e.g., disable EthernetHeaderCompression.

Regarding how to establish the multicast MRB in RRC_INACTIVE, there are two possible options:

Option 1: the UE continues to use the MRB configured in RRC_CONNECTED and updates the configuration based on MCCH/RRC release, i.e. the UE does not suspend the multicast MRB that relates to the TMGI to be received in RRC_INACTIVE. When the UE resumes to RRC_CONNECTED, the network updates the parameters by RRC Resume message.

Option 2: the UE suspends all the configurations as legacy and establishes a new multicast MRB based on MCCH/RRC release, and the UE releases the multicast MRB used in RRC_INACTIVE when it enters RRC_CONNECTED.

We assume the MRB structure in MCCH would be very similar as that used for broadcast, so the IE structure is somehow different from the MRB configuration in RRC_CONNETED. This means it is difficult to implement the delta configuration between the MRB used in RRC_CONNETED and the MRB configured in MCCH. In addition, the UE may anyway need to do full-config of MRB during mobility considering the MRB ID might not be aligned among cells.

Comparing with option 1, option 2 is cleaner and the UE just establishes a new multicast MRB as for MBS broadcast operation and the configuration used in RRC_CONNETED can be suspended and resumed as legacy. The only drawback would be that there might be some interruption during switch from the MRB used in RRC_CONNETED to MRB used in RRC_INACTIVE. However, this can be somehow optimized by UE itself, for example by continuing the reception from the MRB used in RRC_CONNETED before switch.

Proposal 10a: When the UE is released to RRC_INACTIVE to continue multicast reception, the UE suspends the multicast MRB(s) used in RRC_CONNECTED and establishes new multicast MRB(s) based on the configuration in MCCH/RRCRelease.

Proposal 10b: The UE releases the multicast MRB(s) used in RRC_INACTIVE when it enters RRC_CONNECTED.

From stage 3 point of view, multicast MRB used in RRC_CONNECTED and the multicast MRB used in RRC_INACTIVE is much different and multicast MRB used in RRC_INACTIVE mode is more like broadcast MRB. So, to facilitate the stage 3 work, it would be better to define a new terminology such as “multicast-inactive MRB” to differentiate it from the multicast MRB used in RRC_CONNECTED mode. Otherwise we will have to use a term like “multicast MRB used in RRC_INACTIVE” and change the current terminology “multicast MRB” to “multicast MRB used in RRC_CONNECTED” throughout the specifications.

Proposal 11: Define a new terminology as “multicast-inactive MRB” for the multicast MRB used in RRC_INACTIVE, to differentiate from the multicast MRB used in RRC_CONNECTED.

- Conclusion

Based on the above discussion we have the following proposals:

### 3.1 Proposals for PTM configuration and mobility aspects

UE resume due to bad reception quality

Proposal 1a: the threshold used to trigger RRC resumption is based on signal quality, i.e., configuration among RSRP, RSRQ, or SINR.

Proposal 1b: the threshold used to trigger RRC resumption is configured in MCCH.

Proposal 1c: further discuss whether the threshold is per service or per service group or per cell.

NCL for multicast

Proposal 2: NW should be able to provide the UE with the information on whether the same PTM configuration is being used in neighbour cells for multicast in RRC_INACTIVE.

Frequency prioritization

Proposal 3: legacy mechanism of providing the dedicated frequency priority in RRC dedicated signaling can be reused for multicast frequency prioritization in RRC_INACTIVE.

Resume cause and access category

Proposal 4a: introduce new MBS resume causes for cases where the UE needs to resume to continue multicast reception.

Proposal 4b: if the resume procedure to continue multicast reception is barred or rejected, the UE’s AS layer should re-initiate the procedure when barring condition is alleviated, similar as RNA update procedure

Proposal 5: the UE capable of MBS multicast reception in RRC_INACTIVE should behave the same as the legacy R17 MBS UE if no R18 configuration for MBS multicast reception in RRC INACTIVE is configured in RRCRelease, i.e., trigger resume upon group paging for session activation.

PDCP COUNT Continuity

Proposal 6a: One cell can indicate "synchronized", if by implementation, it follows a common QoS flow to MRB mapping rule and at the same time sets the PDCP COUNT according to the MBS QoS Flow SN.

Proposal 6b: The UE doesn’t need to re-initialize the PDCP window, if both the source cell and the target cell indicate “synchronized”.

Proposal 6c: The UE re-initializes the PDCP window similar as broadcast, if the source and the target cells are not synchronized.

### 3.2 Proposals for Notification and state transition

Group paging for service activation

Proposal 7: add one indicator per multicast service in group paging upon session activation to enable the UE to stay in RRC_INACTIVE for multicast reception. The absence of this indicator means the UE is required to enter RRC_CONNECTED for multicast reception.

Notification for session deactivation

Proposal 8: Upon the deactivation of multicast session, MCCH message is used to notify the RRC_INACTIVE UE which is receiving this multicast session.

MCCH monitoring during session deactivation period

Proposal 9: The UE is not required to monitor MCCH-RNTI during session deactivation period and should start to monitor MCCH-RNTI after receiving group paging to acquire the latest PTM configuration for the concerned service.

### 3.3 Proposals for Stage 3 RRC CR issues

RRC modelling for multicast reception in RRC_INACTIVE

Proposal 10a: When the UE is released to RRC_INACTIVE to continue multicast reception, the UE suspends the multicast MRB(s) used in RRC_CONNECTED and establishes new multicast MRB(s) based on the configuration in MCCH/RRCRelease.

Proposal 10b: The UE releases the multicast MRB(s) used in RRC_INACTIVE when it enters RRC_CONNECTED.

Proposal 11: Define a new terminology as “multicast-inactive MRB” for the multicast MRB used in RRC_INACTIVE, to differentiate from the multicast MRB used in RRC_CONNECTED.