3GPP TSG-RAN WG2 Meeting #122	R2-2304821

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on shared processing for MBS broadcast and unicast reception

Document for:   Discussion and decision

Introduction

In the RAN2#121 meeting, we made the agreements for shared processing as below [1]:

- Indicate the capability of receiving MBS broadcast from a non-serving cell. FFS whether the granularity is at FeatureSetDownlink or FeatureSetDownlinkPerCC level. - FFS Whether to include additional information in MII can be controlled by the network. Should consider whether this would be two-step procedure or one-step procedure (e.g. having more info in SIB1).

In this contribution, we further discuss the granularity of capability signaling for non-serving cell broadcast reception, the procedure for non-serving cell MII reporting and the content of the MII.

Discussion

Granularity of UE capability signalling

In the last meeting, it was agreed to introduce capability signalling for MBS broadcast reception from a non-serving cell. One reason behind this is that the non-serving cell for broadcast reception may not be synchronised with the serving cells and the supported feature sets for the band combination for mixed unicast and broadcast capability may not be identical to the feature sets for unicast only. With such capability signalling, the network can determine which feature set for a band/CC in the band combination may be used by the UE for non-serving cell reception. One FFS is on whether the capability signalling is defined per FSPC level or per FS level.

In our view, to simplify the capability signalling, a per FS level capability combined with the MII reported by the UE is enough for the serving gNB to determine the occupied FSPC capability for non-serving cell broadcast reception. It should be also noted that according to the current capability reporting principle, the FSPCs within one FeatureSet can only be configured as contiguous carriers for unicast from serving cell and for broadcast from non-serving cell. Whether the other FSPCs within the FeatureSet can be used for unicast/multicast should be left to NW decision based on carrier deployment. If the contiguous carrier configuration cannot be guaranteed by the serving gNB, the whole FeatureSet capability should be considered occupied for non-serving cell reception, and the serving gNB may need to stop the unicast/multicast on the corresponding band.

TS 38.331 6.3.3 UE capability information blocks #### – FeatureSetCombination The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Ids and FeatureSetDownlinkPerCC-Ids. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of carriers supported by the UE is also restricted by the bandwidth class indicated in the associated BandCombination, if present.

As shown in Figure-1, let’s assume the nonServingCellMBS capability is indicated for a FeatureSet of band C. When the network determines that band C will be used by the UE for non-serving cell reception after receiving the MII, the other FeatureSets in the band combination (i.e., FS#1 for band A and FS#3 band B) in the same line could be used for unicast/multicast transmission.

Proposal 1: Indicate the capability of receiving MBS broadcast from a non-serving cell in FeatureSetDownlink.

Figure-1 BC and FSC for unicast/multicast in serving cell(s) and broadcast in non-serving cell(s)

MII reporting procedure

In previous meetings, it was agreed to introduce MII reporting for non-serving cell broadcast reception, in which the broadcast frequency, subcarrier spacing and bandwidth can be included.

However, in some scenarios, the UE may not be able to acquire the additional information (e.g. SCS, bandwidth) from the non-serving cell when configured with certain CA/DC combination by the serving gNB, due to capability restrictions. For example, when the CA/DC combination configured for unicast/multicast transmission has occupied full hardware resource of the UE, no additional resource can be used to acquire the SIB from a non-serving cell. To avoid interruption of the on-going services, the additional information of the non-serving cell cannot be reported by the UE without proper configuration from the NW.

Currently in legacy NR, to report the CGI for inter-frequency or inter-RAT, the UE can be configured by the network to use autonomous gap. Similar mechanism (a configured gap) can be used by the UE to acquire the additional information from the non-serving cell in case this cannot be done while performing the communication with the serving cell. To be specific, the UE may report all the information in the MII if available. But if the additional information is not available, the UE can only report the interested broadcast frequency in the MII. In this case, the NW should configure a gap to the UE in order to let the UE acquire the additional information from the non-serving cell. After that, the additional information can be reported to the NW.

Proposal 2：In case the additional information (e.g. SCS, bandwidth) is not available (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE only reports frequency information in the MII. The NW may configure a gap to let the UE acquire the additional information from non-serving cell and report it once available.

MII content

In our understanding, the MBS broadcast configuration in the non-serving cell should be aligned with the MBS broadcast configuration used for serving cell in Rel-17. Therefore, if SCS is included in the MII for shared processing, it should be the SCS of the initial BWP (i.e. CORESET#0) of the non-serving cell. Besides, for the bandwidth included in the MII, it was proposed to be either the cell bandwidth or the CFR bandwidth of the non-serving cell. However, it should be noted that when the UE camps on a cell, the UE only needs to support a bandwidth which is larger than initial BWP and smaller than or equal to the cell bandwidth, according to the current RRC spec:

2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which - is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for RedCap UE, of the RedCap-specific initial downlink BWP if configured), and which - is wider than or equal to the bandwidth of the initial downlink BWP or, for RedCap UE, of the RedCap-specific initial downlink BWP if configured, and …… 4> apply a supported downlink channel bandwidth with a maximum transmission bandwidth which - is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for RedCap UEs, RedCap-specific initial downlink BWP, if configured, and which - is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a RedCap UE, of the RedCap-specific initial downlink BWP if configured; 4> select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present; 4> forward the cellIdentity to upper layers; ……

Based on the 38.331 text, it is clear that the channel bandwidth the UE actually uses may NOT be the bandwidth of initial BWP or CFR. In MBS scenario, the serving gNB should decide the available unicast capability based on the bandwidth the UE actually uses for broadcast reception in non-serving cell. For example, the CFR configured by the non-serving cell for broadcast reception is 30MHz, but the UE may actually use 40MHz channel bandwidth to receive broadcast if 30MHz is not supported by the UE. We understand the channel bandwidth reported in the MII should be aligned with the signalled bandwidth capability in UE capability information. This will give NW more precise information about how much processing capability can be left to unicast/multicast scheduling.

Observation: When the UE camps on a cell, the actual channel bandwidth used by the UE may NOT be the bandwidth of initial BWP or CFR.

Therefore, when the UE performs broadcast reception from the non-serving cell, the indicated bandwidth in the MII should be the channel bandwidth actually used by the UE, which covers the broadcast CFR bandwidth but can be smaller than the carrier bandwidth of the non-serving cell.

Proposal 3: The SCS indicated in the MII is the SCS of the initial BWP of the non-serving cell where the UE intends to receive MBS broadcast.

Proposal 4: The bandwidth indicated in the MII is the channel bandwidth used by the UE for broadcast reception in the non-serving cell, which covers the CFR bandwidth.

To assist the gNB to select a correct feature set for broadcast reception, the gNB needs to know all the information that is used to uniquely define a feature set which includes the required bandwidth/SCS/MIMO layer/MCS for broadcast as shown in the figure below:

Figure-2 UE capability parameters within FeatureSetDownlinkPerCC

Since the MIMO layer for broadcast is fixed to be 1 layer, it is not needed to be indicated for FSPC selection. In addition to the bandwidth and the SCS, modulation order is another parameter needed for the serving gNB to uniquely identify the FSPC used for broadcast reception. It should be noted that 64QAM is only the mandatory capability for single CC case, however, for CA scenario between unicast and broadcast, the modulation order capability reported in a specific FSPC should be left to UE decision. The non-serving cell may use a lower modulation order than 64QAM, which is necessary to be reported in the MII.

Proposal 5: Include modulation order in the MII for shared processing.

Conclusion

In this contribution, we provide the following proposals:

Granularity of UE capability signalling

Proposal 1: Indicate the capability of receiving MBS broadcast from a non-serving cell in FeatureSetDownlink.

MII reporting procedure

Proposal 2：In case the additional information (e.g. SCS, bandwidth) is not available (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE only reports frequency information in the MII. The NW may configure a gap to let the UE acquire the additional information from non-serving cell and report it once available.

MII content

Observation: When the UE camps on a cell, the actual channel bandwidth used by the UE may NOT be the bandwidth of initial BWP or CFR.

Proposal 3: The SCS indicated in the MII is the SCS of the initial BWP of the non-serving cell where the UE intends to receive MBS broadcast.

Proposal 4: The bandwidth indicated in the MII is the channel bandwidth used by the UE for broadcast reception in the non-serving cell, which covers the CFR bandwidth.

Proposal 5: Include modulation order in the MII for shared processing.

References

[1] RAN2#121bis-e chairman’s note.

[2] R2-2303970, Discussion on shared processing for MBS broadcast and unicast reception, Huawei, HiSilicon.