3GPP TSG-RAN WG2 #122 	R2-2304824

Incheon, Korea, 22nd – 26th May, 2023

Title:		Discussion on the remaining CP issues for NCR

Source:	Huawei, HiSilicon

Agenda item:	7.1.3

Document for:	Discussion

- Introduction

In the last meeting, the proposals below were discussed [1], but not converged, which is expected to be further discussed in this meeting:

Issue of the wake-up timer: - Proposal: In Rel-18, do not define “wake-up timer” IE in RRCRelease message, if needed, it can be done via OAM (no specification impact) Issue of beam management in RRC_INACTIVE: - Proposal: If needed, beam monitoring for backhaul link when NCR-MT in RRC_INACTIVE state can be done by implementation.

In this contribution, we will discuss the remaining issues above.

Before the discussion, we would like to highlight that this work item was supposed to be closed in the meeting before last meeting, and has been extended for one quarter already. We should only discuss key issues that may block the closure of this work item, rather than optimizations or solutions that even with no clear motivations.

Proposal 1: RAN2 should only discuss key issues that may block the closure of NCR WI.

- Discussion

  - Wake-up timer

The gNB-configured wake-up timer has been discussed for a couple of meetings. Basically the proponents were discussing how the timer should work (e.g. AS or NAS impacts), but the motivation of the timer itself is still unclear.

Before we discuss the timer, we may need to understand first why the gNB sends the NCR-MT to RRC_IDLE.

To us the following case is valid. OAM may decide to disable the NCR-Fwd, e.g., for the reason of maintenance and diagnosis or when the network doesn’t want to extend the coverage (e.g. during nights when there is no much traffic). The NCR-Fwd would be switched off, and in this case it is beneficial if the NCR-MT can be release to RRC_IDLE. The procedure can be like Figure 1:

Figure 1. Procedure of releasing NCR-MT to RRC_IDLE

- OAM sends the control information to the NCR device to switch off NCR-Fwd. The OAM belongs to the operator, and connects to the NCR device either by the NCR-MT’s DRB or by other means (e.g. wired connection).

- The NCR-Fwd switches off after receiving the command.

- To be energy efficient, NCR-MT can request gNB to release it to RRC_IDLE. This can be done by the existing signaling, e.g., preferredRRC-State reported in UEAssistanceInformation message.

- The gNB releases the NCR-MT to RRC_IDLE.

- For the DRB based remote OAM connection, when OAM wants to switch on NCR-Fwd again, it will send another control information to the NCR device, and this control packet will trigger paging message to bring NCR-MT back to RRC_CONNECTED, and then the control information will be transmitted to the NCR-MT. For other kinds of connection between NCR device and OAM, the OAM will inform the NCR device to switch on, and based on impelementation, the NCR-MT will initiate the RRC establishment to the network,.

- The NCR device will switch on the NCR-Fwd based on the side control information from gNB.

We don’t see why the wake-up timer is needed. Some companies during the email discussion clarify how the wake-up timer is working (i.e. NCR-MT to initiate RRC connection after the timer expires), but it is unclear why the UE needs a timer to wake up itself to initiate RRC connection. It was also mentioned by the proponent that that the gNB may want the NCR device not working for a period of time, but this “motivation” itself is not motivated. It is unclear why the gNB wants the NCR to rest for a period of time, when there is a requirement to use the NCR device for coverage extension. If it is in the case when there is no need for coverage extension (e.g. during nights when no much traffic), it is unclear why it has to be the timer (no matter OAM or gNB based timer) to serve the purpose. The operator should be able to control the switch on/off of any its network devices via OAM based on the need (including but not limited to timer, pattern during a day cycle, etc.)

It seems like that the original trigger of the discussion of wake-up timer is that the gNB initialized RRC release and NCR-MT itself doesn’t know when to reinitiate the RRC connection, but it is never clear what the case is and why the gNB has to send to NCR-MT to RRC_IDLE.

Observation 1: The OAM via whatever connection should be able to control the switch on/off of any its network devices (including NCR devices) based on the need. How the OAM triggers the switch on/off would be up to implementation which includes but is not limited to timers, time patterns during a day cycle, event trigger etc.

Observation 2: When OAM switches off NCR-Fwd, the gNB can/may release NCR-MT to RRC_IDLE based on the request from NCR. OAM will trigger NCR-MT to re-initiate RRC connection when there is a need to switch on NCR-Fwd.

Observation 3: It is unclear why the wake-up timer is useful for the NCR-MT.

Again, we think we anyway need to discuss any solution based on the clear issue. Here RAN2 even didn’t identify any particular issue that could motivate this timer.

Proposal 2: The wake-up timer is not pursued (no real issue identified to motivate this solution).

  - Backhaul beam management in RRC_INACTIVE

In the last meeting, another discussion was on the backhaul link beam management in RRC_INACTIVE. In RRC_CONNECTED, the backhaul beam of the NCR is indicated by the gNB semi-persistently via MAC CE. However, when NCR-MT is released to RRC_INACTIVE, there will be no way for this semi-persistent indication, but the best beam for the backhaul link may change even if the location of the NCR device may not change (e.g. due to blockage in high frequency).

Observation 4: After NCR-MT is released to RRC_INACTIVE, the gNB cannot indicate the backhaul beam via the semi-persistent indication, but the best beam for the backhaul link may change (e.g. due to blockage in high frequency) even if the location of the NCR device may be fixed.

In the last meeting, an agreement was achieved for the NCR-Fwd access link beam configuration when the NCR-MT is released to RRC_INACTIVE:

- When NCR-MT is released to RRC_INACTIVE state (NCR-Fwd is forwarding), the periodic beam indication configuration (if configured and not removed) shall be applied.

But for the backhaul beam, there was no conclusion. The agreements of backhaul beam selection in RAN1 #112 and RAN1 #111 were given as follows [2] [3]:

Agreement In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-17 beam indication framework is used for C-link, - If no unified TCI is applied for C-link, the beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. (i.e. same as the default beam defined for Rel-15/16 beam indication framework) - If there is unified TCI applied for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Agreement The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link.

According to RAN1 agreements, when there are simultaneously C-link and backhaul link transmission/reception, the backhaul link will follow C-link in terms of beam management.

Observation 5: According to RAN1 agreements, in some cases, e.g. when there are simultaneously C-link and backhaul link transmission/reception, the beam for backhaul link will follow C-link beam.

When the NCR-MT is in RRC_INACTIVE, there should be no simultaneous downlink/uplink transmission in C-link and backhaul link. We have agreed in the last meeting that only the periodic beam indication for the access link shall be applied, which indicates the semi-persistent access beam configuration is discarded. Similarly, the semi-static backhaul beam indication by the MAC CE (i.e., the “dedicated signal” in RAN1 agreement) should also be discarded. In this sense, it is unclear what beam is used for backhaul link.

Observation 6: According to RAN2 agreements, the semi-static beam indication is discarded when the NCR-MT is released to RRC_INACTIVE, and it is unclear what beam would be used for backhaul link.

Based on RAN1 agreement, the backhaul beam can follow the C-link beam configuration for the NCR-MT in RRC_CONNECTED. Similarly, we think it is reasonable to extend such idea to RRC_INACTIVE. Considering the dynamic beam features of the high frequency, the appropriate backhaul beam should be selected based on the current beam status of the NCR-MT C-link. Specifically, The RRC_INACTIVE NCR-MT performs C-link beam monitoring and knows the current best beam in the camped cell. The backhaul beam should be the same as the best beam of the camped cell. In other words, the backhaul beam needs to be changed if the best beam changes.

When the backhaul beam selected by the NCR is changed, the gNB should know the new beam selected by the NCR and be adapted to the new beam for transmission/reception. There are different ways for the gNB to be aware of this change. In the last meeting, a proposal was that the NCR-MT in RRC_INACTIVE resumes connection to receive updated side control configuration if it reselects a new beam of the same camped cell. This solution means that the NCR-MT should go to RRC_CONNECTED state to inform the gNB about this change. The solution should be feasible, but would cause frequent RRC resume as the best beam may change frequently.

One way to resolve this problem could be to introduce Msg1 based uplink indication. The NCR-MT can be configured with a dedicated PRACH resource and initiate Msg 1 transmission when there is a backhaul beam change. By detecting the Msg1 transmission, the gNB would know the backhaul beam selected by the NCR-MT. This solution is similar to the Msg-1 based BFR in RRC_CONNECTED, and can avoid the RRC resume procedure, which helps to make the change quickly.

The solutions proposed above all require additional standardization efforts, and the WI is closing. From another perspective, the motivation for leaving the NCR-Fwd working when the NCR-MT is in RRC_INACTIVE is not so strong, as from power saving perspective it should be NCR-Fwd part consuming clear majority of power. If the NCR-Fwd is running, there is no clear benefit to release the NCR-MT to RRC_INACTIVE. Another way forward is just to turn off the NCR-Fwd when NCR-MT is in RRC_INACTIVE (same as RRC_IDLE), i.e. to revert the former conclusion. Further enhancement can be done in future releases.

Proposal 3: With regard to the backhaul link beam issue, two WFs can be considered:

- Option 1: RAN2 to agree on a solution for selecting a backhaul link beam when NCR-MT is in RRC_INACTIVE.

- Option 2: RAN2 to agree that NCR-Fwd is switched off when NCR-MT is in RRC_INACTIVE, same as in RRC_IDLE (i.e. the previous agreement is reverted).

Proposal 4: In case RAN2 agrees to address the backhaul link beam selection issue (option 1 in Proposal 3 is agreed), the following solution can be considered:

- Backhaul link always follows the best C-link beam and when the best beam changes, NCR-MT transmits Msg1 on the preconfigured PRACH resource to indicate the new beam.

3.	Conclusion

This paper mainly discusses the remaining issues of NCR. The following observations and proposals are provided:

General

Proposal 1: RAN2 should only discuss key issues that may block the closure of NCR WI.

Wake-up timer

Observation 1: The OAM via whatever connection should be able to control the switch on/off of any its network devices (including NCR devices) based on the need. How the OAM triggers the switch on/off would be up to implementation which includes but is not limited to timers, time patterns during a day cycle, event trigger etc.

Observation 2: When OAM switches off NCR-Fwd, the gNB can/may release NCR-MT to RRC_IDLE based on the request from NCR. OAM will trigger NCR-MT to re-initiate RRC connection when there is a need to switch on NCR-Fwd.

Observation 3: It is unclear why the wake-up timer is useful for the NCR-MT.

Proposal 2: The wake-up timer is not pursued (no real issue identified to motivate this solution).

Backhaul beam management in RRC_INACTIVE

Observation 4: After NCR-MT is released to RRC_INACTIVE, the gNB cannot indicate the backhaul beam via the semi-persistent indication, but the best beam for the backhaul link may change (e.g. due to blockage in high frequency) even if the location of the NCR device may be fixed.

Observation 5: According to RAN1 agreements, in some cases, e.g. when there are simultaneously C-link and backhaul link transmission/reception, the beam for backhaul link will follow C-link beam.

Observation 6: According to RAN2 agreements, the semi-static beam indication is discarded when the NCR-MT is released to RRC_INACTIVE, and it is unclear what beam would be used for backhaul link.

Proposal 3: With regard to the backhaul link beam issue, two WFs can be considered:

- Option 1: RAN2 to agree on a solution for selecting a backhaul link beam when NCR-MT is in RRC_INACTIVE.

- Option 2: RAN2 to agree that NCR-Fwd is switched off when NCR-MT is in RRC_INACTIVE, same as in RRC_IDLE (i.e. the previous agreement is reverted).

Proposal 4: In case RAN2 agrees to address the backhaul link beam selection issue (option 1 in Proposal 3 is agreed), the following solution can be considered:

- Backhaul link always follows the best C-link beam and when the best beam changes, NCR-MT transmits Msg1 on the preconfigured PRACH resource to indicate the new beam.

4.	References

- Chairs Notes, RAN2-121bis-NCR-(Sasha)-2023-04-26-1345, Online.

- Chairs Notes, RAN1 #112, Athens, Greece.

- Chairs Notes, RAN1 #111, Toulouse, France.